| Question: 3/16 INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION STANDARDISATION SECTOR STUDY PERIOD 1997 - 2000 | COM 16- March 1997 Original: English 30 July, 2026 |

Geneva March 1997

STUDY GROUP 16 – CONTRIBUTION

| SOURCE: | Rapporteur |
| --- | --- |

| CONTACT: | Trevor Peers, John Boucher BT Labs, Martlesham Heath, IPSWICH, Suffolk, UK IP5 7RE. | Tel: +44 1473 647697, 644354 Fax: +44 1473 643791 E-Mail: peers_t_w@bt-web.bt.co.uk boucher_j@bt-web.bt.co.uk |  |
| --- | --- | --- | --- |
| TITLE: | Draft recommendation T.132 - Audio-Visual Control: Services & Protocol |  |  |

___________________

AUDIO-VISUAL CONTROL: SERVICES & PROTOCOL

Draft Recommendation T.132

Summary

This recommendation defines a toolkit of services that allow enhanced control of audio and video streams within a multipoint multimedia conference.  The services provide support for user controlled distribution of real time streams and automated stream processing and distribution services such as video switching, audio mixing, and continuous presence.  Other services include stream identification, floor control and ‘On-Air’ indications.  This recommendation also defines procedures to manage the configuration of network connections and audio-visual processing elements in support of the stream distribution services.  It provides mechanisms to manipulate communication channels to deliver such streams across heterogeneous networks at a specified Quality of Service.   The service is provided in a manner that is independent of the underlying networks.

Draft Recommendation T.132

AUDIO-VISUAL CONTROL: SERVICES AND PROTOCOL

CONTENTS

1. Scope

2. Normative References

3. Definitions

4. Abbreviations

5. Conventions

6. Overview

7. AVC Infrastructure Management Service Definition

7.1 Summary of AVC Abstract Services

7.2 Creation of an AVC Controlled Conference

7.2.1 AVC Conference Framework

7.2.2 Description of Abstract Services

7.2.2.1 AVC-Activate-Management

7.2.2.2 AVC-Initiate-Management

7.2.2.3 AVC-Announce-Zone-Manager

7.3 AVC Sessions

7.3.1 Description of Abstract Services

7.3.1.1 AVC-Session-Join

7.3.1.2 AVC-Private-Session-Admit

7.3.1.3 AVC-Private-Session-Expel

7.3.1.4 AVC-Session-Associate

7.4 Capability Reporting Overview

7.4.1 Support for Multiple Conferences on a Node

7.4.2 Description of Abstract Services

7.4.2.1 AVC-Announce-Link-Capabilities

7.4.2.2 AVC-Announce-Node-Capabilities

7.5 Inheriting Existing Audio and Video Communications

7.5.1 Description of Abstract Services

7.5.1.1 AVC-Announce-Link-Configuration

7.5.1.2 AVC-Announce-Node-Configuration

7.6 The AVC Link and Node Rosters

7.6.1 Description of Abstract Services

7.6.1.1 AVC-Roster-Report

7.6.1.2 AVC-Roster-Enquire

7.6.1.3 AVC-Roster-Monitor

7.7 Resource Configuration

7.7.1 Description of Abstract Services

7.7.1.1 AVC-Set-Link-Multiplex

7.7.1.2 AVC-Set-Link-Transmission-Mode

7.7.1.3 AVC-Set-Node-Configuration

7.8 Logical Channel Management

7.8.1 Description of Abstract Services

7.8.1.1 AVC-Open-Logical-Channel

7.8.1.2 AVC-Close-Logical-Channel

7.9 Resource Arbitration

7.9.1 Description of Abstract Services

7.9.1.1 AVC-Reserve-Resource

7.9.1.2 AVC-Cancel-Resource-Reservation

7.9.1.3 AVC-Confirm-Resource-Reservation

7.9.1.4 AVC-Relinquish-Resource

8. AVC Service Management Service Definition

8.1 Introduction to AVC Conference Services

8.1.1 Class 1 - Core Services

8.1.2 Class 2 - Supplementary Services

8.1.3 Class 3 - Custom Services

8.1.4 Class 4 - User Defined Services

8.1.5 AVC Peer-to-Peer Functions

8.2 Advertising and Accessing AVC Services

8.3 Summary of Conference Services

8.4 AVC Service Access

8.4.1 Description of Abstract Services

8.4.1.1 AVC-Service-Offer

8.4.1.2 AVC-Service-Subscribe

8.4.1.3 AVC-Service-Subscribe-Report

8.4.1.4 AVC-Service-Activate

8.4.1.5 AVC-Service-Launch

8.4.1.6 AVC-Service-Launch-Report

8.5 AVC Core Services

8.5.1 AVC Session Roles

8.5.1.1 Conference Presenter(s)

8.5.1.2 Presenter Assignment

8.5.1.3 Releasing the Presenter Role

8.5.1.4 Passing the Presenter Role

8.5.1.5 Asking about the Presenter Role

8.5.1.6 The Chair Role

8.5.1.7 Chair Assignment

8.5.1.8 Releasing the Chair Role

8.5.1.9 Asking for the Chair Role

8.5.1.10 Giving the Chair Role

8.5.1.11 Asking about the Chairperson

8.5.2 Description of Abstract Services

8.5.2.1 AVC-Session-Role-Assign

8.5.2.2 AVC-Session-Role-Release

8.5.2.3 AVC-Session-Role-Please

8.5.2.4 AVC-Session-Role-Give

8.5.2.5 AVC-Session-Role-Inquire

8.5.3 Floor Control

8.5.3.1 AVC-Request-Floor

8.5.3.2 AVC-Release-Floor

8.5.3.3 AVC-Floor-Grant

8.5.4 Multipoint Channel Management

8.5.4.1 AVC-Create-Multipoint-Channel

8.5.4.2 AVC-Modify-Multipoint-Channel

8.5.4.3 AVC-Disband-Multipoint-Channel

8.5.4.4 AVC-Multipoint-Channel-Status

8.5.5 Source Identification

8.5.6 On-Air Indication Service

8.6 AVC Supplementary Services

8.6.1 Audio Mixing Service (AMS).

8.6.1.1 Audio Mixing Service -Capabilities

8.6.1.2 Chair Controlled Sessions

8.6.1.3 Manual Control Mode

8.6.1.4 Description of Abstract Services

8.6.2 Video Switching Service (VSS).

8.6.2.1 Video Switching - Operation

8.6.2.2 Rôle Holder Input Options

8.6.2.3 Rôle Holder's Output Options

8.6.2.4 Video Switching Matrix

8.6.2.5 Standard Operating Modes

8.6.2.6 Mode Overrides

8.6.2.7 Convenor Options

8.6.2.8 Video Switching Service -Default Operation

8.6.2.9 Description of Abstract Services

8.6.3 "On Air" Indication Service

8.6.4 Enhanced Video Service

8.6.4.1 Continuous Presence Multiplexer

8.6.4.2 Continuous Presence Transcoder

8.6.4.3 Continuous Presence Array Processor

8.7 AVC Peer-to-Peer Functions

8.7.1 Privacy

8.8 Stream Servers { place-holder}

8.9 Source Selection

8.9.1 AVC-Source-Select

8.9.2 Remote Device Control

8.9.2.1 AVC-Device-Attributes

8.9.3 Device Control

8.9.3.1 AVC-Device-Lock

8.9.3.2 AVC-Device-Lock-Inquire

8.9.3.3 AVC-Device-Lock-Terminated

8.9.3.4 AVC-Device-Control

8.9.3.5 AVC-Device-Status

8.9.3.6 AVC-Configure-Device-Events

8.9.3.7 AVC-Notify-Device-Event

8.10 Conducted Mode Behaviour

9. AVC Infrastructure Management Protocol Definition

9.1 AVC Communication Channels

9.1.1 AVC Broadcast Channel

9.1.2 AVC Network Element Control Channel

9.1.3 AVC Network Element Communication Channel

9.1.4 AVC Top Provider Channel

9.2 Nodes Entering an AVC Controlled Conference

9.2.1 Enrolling in the AVC Terminal Standard Base Session

9.2.2 AVC Network Element Enrolment Procedure

9.3 Updating Late Joining Network Elements

9.4 Zone Manager Identification

9.5 Session Management

9.5.1 Joining an AVC Session

9.5.2 Admitting Nodes to a Private AVC Session

9.5.3 Expelling Nodes from a Private AVC Session

9.5.4 Associating an AVC Session with Data Sessions

9.5.5 Impact on AVC Services

9.6 Activation and Termination of AVC Operation

9.6.1 Activating AVC Control

9.7 Capability Reporting

9.7.1 Overview

9.7.2 Advertising Link Capabilities

9.7.3 Advertising Node Capabilities

9.8 Inheriting Existing Audio and Video Communications

9.8.1 Inheriting an Existing Link Configuration

9.8.2 Inheriting an Existing Node Configuration

9.9 The Link and Node Rosters

9.9.1 Retrieving an Entry

9.9.2 Monitoring an Entry

9.10 Resource Configuration

9.10.1 Configuring a Link Multiplex

9.10.2 Configuring the Media Streams Transported by a Link

9.10.3 Configuring a Node

9.10.4 Opening a Logical Channel

9.10.5 Closing a Logical Channel

9.11 Resource Arbitration

9.11.1 Reserving Resources

9.11.2 Cancelling a Reservation

9.11.3 Confirming a Reservation

9.11.4 Relinquishing a Reservation

10. AVC Service Management Protocol Definition

10.1 AVC Service Access

10.1.1 Offering AVC Services to Terminals

10.1.2 Subscribing to AVC Services

10.1.3 Activating AVC Services

10.1.4 Launching AVC Services

10.1.5 Reporting the Launch of AVC Services

10.2 Multipoint Channel Management

10.2.1 Creating a Multipoint Channel

10.2.2 Modifying a Multipoint Channel

10.2.3 Disbanding a Multipoint Channel

10.2.4 Multipoint Channel Status

10.3 Source Selection

10.4 Remote Device Control

10.4.1 Requesting Device Attributes

10.4.2 Locking Devices

10.4.3 Device Lock Inquire

10.4.4 Device Lock Terminated Indication

10.4.5 Device Control Request

10.4.6 Device Status Inquire

10.4.7 Configuring Device Events

10.4.8 Device Event Notify

11. AVC PDU Definitions

Annex A

Annex B

Annex C

C.1. Standard Device Classes

C.1.1. Device Number

C.1.2. Camera Device Class

C.1.3. Slide Presenter Device Class.

C.1.4. Microphone Device Class

C.1.5. Light Source Device Class.

C.1.6. Stream Player Recorder Device Class

C.1.7. Source Combiner Device Class

C.1.8. Document Scanner Device Class

C.1.9.  Device Control Attributes

C.1.9.1 General Control Attributes

C.1.9.2  Camera Control Attributes

C.1.9.3  Camera Control Attributes Used For Start Stop Movement

C.1.9.4  Camera Control Attributes Using Fixed Range Positioning

C.1.9.5  Camera Control Attributes Using Spatial Positioning

C.1.9.6 Slide Presenter Control Attributes

C.1.9.7 Stream Player Recorder Control Attributes for Stream Positioning

C.1.9.8 Stream Player Recorder Attributes for Configuration

C.1.9.9 Stream Player Recorder Attributes for Playback

C.1.9.10 Stream Player Recorder Control Attributes for Recording

C.1.9.11 Source Combiner Control Attributes

C.1.10. Device Status Attributes

C.1.10.1 General Status Attributes

C.1.10.2 Camera Status Attributes

C.1.10.3 Camera Status Attributes Using Fixed Range Positioning

C.1.10.4 Camera Status Attributes Using Absolute Positions

C.1.10.5 Slide Presenter Status Attributes

C.1.10.6 Stream Player Recorder Status Attributes

C.1.10.7 Source Combiner Status Attributes

C.1.11. Device Event Attributes

C.1.11.1 Camera Event Attributes

C.1.11.2 Slide Presenter Event Attributes

C.1.11.3 Stream Player Recorder Event Attributes

Appendix I

AUDIO-VISUAL CONTROL

# Scope

This recommendation specifies mechanisms for the provision of services to control audio and video streams within a multipoint multimedia conference as well as procedures to allow management and control of the infrastructure used to provide such services.  It defines procedures for arbitrating use of real time audio-visual services, advertising services to participating nodes and methods for service access and control.  T.132 specifies mechanisms for multimedia capability reporting at the start of and during communications and defines procedures for arbitrating access to and configuring the audio-visual infrastructure in support of the audio-visual control services.  Whilst the focus of this recommendation is on audio and video control, the provisions of this recommendation may be applied to control of other real time data media types.

T.132 supports conferences in which data communication is established first as well as conferences in which audio-visual communications are established first.  In the latter case, provision is made for seamless transfer of control of existing audio-visual streams.

The provisions of this recommendation are independent of the underlying network(s) and the content of real time streams that traverse those networks. Furthermore, this recommendation provides the possibility for different types of multimedia system (e.g. H.320, H.323, H.324) to co-exist within the same conference and for those systems to be controlled in a uniform manner.

This recommendation is complementary to existing multimedia control protocols such as H.245 and H.242/3.  It enables systems based on the point-to-point H.245 protocol to participate in multipoint conferences, whilst for H.242/3 based systems, it provides enhanced functionality and more flexible control.  Provision is also made for less sophisticated systems, which may have separate audio and data connections.

T.132 is a Client-Server like protocol, with a minimal implementation burden on terminals and most of the complexity concentrated at network elements.  It supports centralised or distributed management of the real time infrastructure, enabling it to scale readily to support large conference environments.

T.132 requires the services of MCS, GCC and an underlying multimedia control protocol such as H.245.

FIGURE 1/T.132

Audio-visual control architecture

# Normative References

The following Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid.  All Recommendations and other references are subject to revision: All users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below.  A list of the currently valid ITU-T Recommendations is regularly published.

–	ITU-T Recommendation T.120 (1996), Transmission Protocols for Multipoint Data Conferencing

–	ITU-T Recommendation T.121 (1996), Generic Application Template

–	ITU-T Recommendation T.122 (1993), Multipoint Communication Service for audiographics and audiovisual conferencing service definition.

–	ITU-T Recommendation T.123 (1996), Protocol Stack for Audiographics and Audiovisual Teleconference Applications.

–	ITU-T Recommendation T.124 (1995), Generic Conference Control.

–	ITU-T Recommendation T.125 (1994), Multipoint Communication Service protocol specification.

–	ITU-T Recommendation T.130 (199x), Audio-Visual Control for Multimedia Conferencing: Architecture and Overview -under development

–	ITU-T Recommendation T.131 (199x), Audio-Visual Control: Network Specific Mappings - under development

–	ITU-T Recommendation H.245 (1997), Control Protocol for Multimedia Communication (Version 2)

_	ITU-T Recommendation X.680 (1994),  Information Technology - Abstract Syntax Notation One (ASN.1) - Specification of Basic Notation.

_	ITU-T Recommendation X.691 (1995), Information Technology - ASN.1 Encoding Rules - Specification of Packed Encoding Rules (PER).

# Definitions

For the purposes of this Recommendation, the following definitions apply:

Adjacent Node: An adjacent node is a node that forms one of a pair of nodes that are connected and thus associated by a real time link.

Application Protocol: Any standard or non-standard protocol specification that makes use of T.120 services.

Application Protocol Entity: The instantiation of an Application Protocol in a terminal or MCU.

Application Protocol Session: A group of peer Application Protocol Entities communicating with each other.

AVC Conference - A conference in which the control of audio and video is performed using AVC.

AVC Network Element: A node that provides AVC Services, either on its own or in conjunction with other AVC Network Elements.

AVC Provider: Entity within a node responsible for providing AVC Services.

AVC Session - A subset of AVC conference participants formed into a separate and autonomous group of nodes for the purposes of communicating real-time streams within that group.

AVC Terminal: A node that makes use of AVC Services; typically a terminal.

Controllable Device: A device that has the capability to be controlled remotely in a conference environment.

Communication Port: A physical interface at a node that possesses AVC media capabilities and is able to provided a connection to a real time link.

Device: An entity which provides a specific function, or set of functions, to the conference. Some devices such as cameras and microphones are capable of generating a source to the conference. Other devices provide control and processing functions for streams.

Inter-MCU-Link: A real time link that interconnects two adjacent MCU nodes.

Logical channel: A communications path within a real time link that carries a single stream between adjacent nodes.

MCU (Multipoint Control Unit): A device used to interconnect multiple terminals and/or further MCUs to allow multiple participants to be connected to single conference.  Also known as a bridge.

Multipoint Service: One or more functions that are provided and managed by MCUs.

Network Element: An AVC capable node with multiple real time links that provides management of multipoint conferencing or bridging/processing of real time streams.  MCUs, MCs and Gateways are examples of Network Elements.

Network Specific Control Entity: The entity at a node responsible for directly controlling the operation of a real time link.  Typically an NSCE provides capability exchange and link configuration functionality.

Node: Any entity that supports the T.120/T.132 protocols.  This includes both terminal equipment and network elements such as routers and MCUs.

Real Time Link: A connection between two adjacent nodes used for the transfer of real time information.  A real time link may provide one or more communication paths (termed logical channels), each of which conveys a single stream between the two nodes.

Source: The origin of a real time media stream.

Stream: A flow of real time information of a specific media type (e.g. audio) and format (e.g. G.722) from a single source to one or more destinations.

Video Processor: A device that modifies the content of video streams, e.g. a transcoder, continuous presence processor.

Virtual MCU: Term used to describe the collection of network elements within a real time conference which present a single logical interface to terminals.

Zone: A collection of AVC resources (real time links and nodes) under the control of a single AVC Network Element.

Zone Manager: The AVC Network Element responsible for managing a portion of the AVC infrastructure (Zone) and handling all requests for AVC services from AVC Terminals within that Zone.

# Abbreviations

APE			Application Protocol Entity

AVC		Audio-Video Control

ATM		Asynchronous Transfer Mode

GCC		Generic Conference Control

ISDN		Integrated Services Digital Network

LAN		Local Area Network

MC			Multipoint Controller (as defined in H.323)

MCS		Multipoint Communication Service

MCU		Multipoint Control Unit

MP			Multipoint Processor (as defined in H.323)

NSCE		Network Specific Control Entity

PDU		Protocol Data Unit

PSTN		Public Switched Telephone Network

MCS	Multipoint Communication Service

MCU	Multipoint Control Unit

PDU	Protocol Data Unit

RDC	Remote Device Control

SAP	Service Access Point

# Conventions

The primitive parameters of the abstract services defined in this Recommendation use the following key:

M:		parameter is mandatory

C:		parameter is conditional

O:		parameter is optional

Blank:	parameter is absent

(=):		value of the parameter is identical to the value of the corresponding parameter of the preceding primitive, where preceding is defined relative to the order:  request, indication, response, confirm.

(=RQ):	value of the parameter is identical to the value of the corresponding parameter in a preceding primitive, where RQ = request, IN = indication, RS = response, and CF = confirm.

Primitives are categorised in up to four types: Request, Indication, Response, and Confirm.  Some primitives support all of these types, while others do not.  These four types are defined as follows:

Request primitive:	Those that are sourced from a Node Controller, Application Protocol Entity or User Application to initiate a certain action.

Indication primitive:	Those that are sourced from an AVC Provider either as a result of a Request primitive, or as a result of an AVC initiated action.

Response primitive:	Those that are sourced from a Node Controller in response to an Indication primitive which is defined to require a response.

Confirm primitive:	Those that are sourced from an AVC Provider as a result of a Response primitive.

# Overview

This recommendation defines mechanisms to establish a generic platform for control of the real time aspects (audio and video) of multimedia communication applications.  It provides a toolkit of high level services to control the manipulation and distribution of real time streams within a heterogeneous multipoint communication environment and defines low level procedures to allow nodes that are managing a conference to arbitrate use of and deliver those services.

The services range from basic tools such as speaker identification and ‘on-air’ indication functions, which can be used to enhance audio-conferencing services, through to audio mixing, video switching and continuous presence services which can be used to deliver sophisticated multimedia conferencing services.  These features are designed to be used in a wide range of group communication scenarios, ranging from the traditional tightly coupled conference (as typically used for business meetings), through to loosely coupled conferences (e.g. social gatherings in which participants may join and leave discussion groups at will) as well as hybrid conferences (e.g. a convention with a panel of participants who interact, watched by a large audience).  Provision is made for autonomous communication sessions within a conference (e.g. to allow private side conversations or breakout groups), without requiring a separate conference to be established.

Whilst a number of the AVC services are by their nature complex, the goal of AVC is to shield this complexity from the end user.  By defining a number of conference models to support commonly used meeting scenarios, AVC services can be configured to run automatically with minimal user interaction.  Conference participants thus gain the benefits of a range of sophisticated services without the burden of having to understand how to configure or control them.

AVC also supports a number of peer to peer functions such as remote device control, privacy and source selection.  These differ from AVC services in that they are not dependent on the presence of AVC capable nodes in the network to be used.

AVC builds on the strengths of T.120 and is thus inherently independent of network and platform.  It also overcomes some of the scalability limitations of T.120 by adopting a distributed management philosophy and by supporting the concept of anonymous nodes which have minimal interaction with and influence on the conference.  Typically, these nodes would be members of an audience.

AVC services apply within an established conference environment, they are not used to add or remove participants to the conference.

AVC allows control of audio and video in point-to-point communications, although with reduced functionality due to the absence of network based services.

Editor’s Note: introductory text (up to & including former section 8) has been transfered to T.130

# AVC Infrastructure Management Service Definition

## Summary of AVC Abstract Services

Table 1 is a list of all AVC infrastructure management primitives and their associated PDUs.  The table also shows whether or not each primitive is mandatory (M), conditionally required (C), or optional (O) for an AVC Terminal or AVC Network Element.  The table also shows, for the corresponding PDUs, whether these are mandatory (M) or conditionally required (C) for both the transmit (T) and receive (R) directions.  A conditionally required PDU is one which is required only if the corresponding primitive is to be supported at that node.  In the case that a primitive is not mandatory, but its corresponding PDU is mandatory, this implies that there is some portion of the protocol, not related to the primitive, that relies on the use of that PDU which is required to be supported.

Observe that terminals which act in a network element capacity under certain circumstances (e.g. H.323 endpoints which act as MCs) must conform to the requirements for AVC Network Elements.

TABLE 1/T.132.

AVC infrastructure management primitives and PDUs

| Functional Unit | Primitives | AVCTmn | AVCN.E. | Associated PDUs | Dir. | AVCTmn | AVCN.E. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AVC | AVC-Activate-Management request | O | O | ActivateManagementRequest | T | C | C |
| Management | AVC-Activate-Management indication | - | M | ActivateManagementRequest | R | M | M |
| Activation | AVC-Activate-Management response | - | M | ActivateManagementResponse | R | M | M |
|  | AVC-Activate-Management confirm | - | M | ActivateManagementResponse | R | C | C |
|  | AVC-Initiate-Management indication | O | O | InitiateManagementIndication | T,R | - | M |
| AVC | AVC-Session-Join request | O | O | - |  |  |  |
| Session | AVC-Session-Join confirm | O | O | - |  |  |  |
| Management | AVC-Private-Session-Admit request | O | O | PrivateSessionAdmitIndication | T | C | C |
|  | AVC-Private-Session-Admit indication | M | M | PrivateSessionAdmitIndication | R | M | M |
|  | AVC-Private-Session-Admit confirm | O | O | - |  |  |  |
|  | AVC-Private-Session-Expel request | O | O | PrivateSessionExpelIndication | T | C | C |
|  | AVC-Private-Session-Expel indication | M | M | PrivateSessionExpelIndication | R | M | M |
|  | AVC-Private-Session-Expel confirm | O | O | - |  |  |  |
|  | AVC-Session-Associate request | O | O | - |  |  |  |
|  | AVC-Session-Associate confirm | O | O | - |  |  |  |
| Zone Manager Identification | AVC-Announce-Zone-Manager indication | M | M | AnnounceZoneManagerIndication | T R | - M | M M |
| Capability | AVC-Announce-Link-Capabilities request | - | M | RosterUpdateIndication | T, R | - | M |
| Reporting | AVC-Announce-Link-Capabilities confirm | - | M | - | - | - | - |
|  | AVC-Announce-Node-Capabilities request | O | M | RosterUpdateIndication | T R | C - | M M |
| Link | AVC-Roster-Report indication | - | M | RosterUpdateIndication | T, R | - | M |
| and Node | AVC-Roster-Enquire request | O | O | RosterEnquireRequest | T R | C - | C M |
| Rosters | AVC-Roster-Enquire confirm | O | O | RosterEnquireResponse | T R | - C | M C |
|  | AVC-Roster-Monitor request | O | O | RosterMonitorRequest | T R | C - | C M |
|  | AVC-Roster-Monitor confirm | O | O | RosterMonitorResponse | T R | - C | M C |
| Inheriting | AVC-Announce-Link-Configuration request | - | M | RosterUpdateIndication | T, R | C | M |
| Existing | AVC-Announce-Link-Configuration confirm | - | M | - | - | - | - |
| Configuration | AVC-Announce-Node-Configuration request | C | M | RosterUpdateIndication | T R | C - | M M |
| Resource Configuration | AVC-Set-Link-Multiplex request | - | M | SetLinkMultiplexRequest | T R | - M | M M |
|  | AVC-Set-Link-Multiplex indication | M | M | SetLinkMultiplexRequest | T R | - M | M M |
|  | AVC-Set-Link-Multiplex response | M | M | SetLinkMultiplexResponse | T R | M - | M M |
|  | AVC-Set-Link-Multiplex confirm | - | M | SetLinkMultiplexResponse | T R | M - | M M |
|  | AVC-Set-Link-Transmission-Mode request | - | M | SetLinkTransmissionModeRequest | T R | - M | M M |
|  | AVC-Set-Link-Transmission-Mode indication | M | M | SetLinkTransmissionModeRequest | T R | - M | M M |
|  | AVC-Set-Link-Transmission-Mode response | M | M | SetLinkTransmissionModeResponse | T R | M - | M M |
|  | AVC-Set-Link-Transmission-Mode confirm | - | M | SetLinkTransmissionModeResponse | T R | M - | M M |
|  | AVC-Set-Node-Configuration request | - | M | SetNodeConfigurationRequest | T R | - M | M M |
|  | AVC-Set-Node-Configuration indication | M | M | SetNodeConfigurationRequest | T R | - M | M M |
|  | AVC-Set-Node-Configuration response | M | M | SetNodeConfigurationResponse | T R | M - | M M |
|  | AVC-Set-Node-Configuration confirm | - | M | SetNodeConfigurationResponse | T R | M - | M M |
|  | AVC-Open-Logical-Channel request | - | M | OpenLogicalChannelRequest | T R | - M | M M |
|  | AVC-Open-Logical-Channel indication | M | M | OpenLogicalChannelRequest | T R | - M | M M |
|  | AVC-Open-Logical-Channel response | M | M | OpenLogicalChannelResponse | T R | M - | M M |
|  | AVC-Open-Logical-Channel confirm | - | M | OpenLogicalChannelResponse | T R | M - | M M |
|  | AVC-Close-Logical-Channel request | - | M | CloseLogicalChannelRequest | T R | - M | M M |
|  | AVC-Close-Logical-Channel indication | M | M | CloseLogicalChannelRequest | T R | - M | M M |
|  | AVC-Close-Logical-Channel response | M | M | CloseLogicalChannelResponse | T R | M - | M M |
|  | AVC-Close-Logical-Channel confirm | - | M | CloseLogicalChannelResponse | T R | M - | M M |
| Resource | AVC-Reserve-Resource request | - | M | ReserveResourceRequest | T, R | - | M |
| Arbitration | AVC-Reserve-Resource indication | - | M | ReserveResourceRequest | T, R | - | M |
|  | AVC-Reserve-Resource response | - | M | ReserveResourceResponse | T, R | - | M |
|  | AVC-Reserve-Resource confirm | - | M | ReserveResourceResponse | T, R | - | M |
|  | AVC-Cancel-Resource-Reservation request | - | M | CancelResourceReservationRequest | T, R | - | M |
|  | AVC-Cancel-Resource-Reservation indication | - | M | CancelResourceReservationRequest | T, R | - | M |
|  | AVC-Cancel-Resource-Reservation confirm | - | M | - |  |  |  |
|  | AVC-Confirm-Resource-Reservation request | - | M | ConfirmResourceReservationRequest | T, R | - | M |
|  | AVC-Confirm-Resource-Reservation indication | - | M | ConfirmResourceReservationRequest | T, R | - | M |
|  | AVC-Confirm-Resource-Reservation response | - | M | ConfirmResourceReservationResponse | T, R | - | M |
|  | AVC-Confirm-Resource-Reservation confirm | - | M | ConfirmResourceReservationResponse | T, R | - | M |
|  | AVC-Relinquish-Resource request | - | M | RelinquishResourceRequest | T, R | - | M |
|  | AVC-Relinquish-Resource indication | - | M | RelinquishResourceRequest | T, R | - | M |
|  | AVC-Relinquish-Resource confirm | - | M | - |  |  |  |

## Creation of an AVC Controlled Conference

When a GCC conference is created, the only AVC functionality available to terminals are the peer-to-peer functions such as remote device control.  To make use of AVC services offered by AVC Network Elements, the conference convenor must explicitly activate AVC management.  This operation may be performed at the start of the GCC conference or at any time subsequently.  When activating AVC management, the convenor specifies a framework for operation of AVC services.  This information may be derived from a conference reservation and allows the convenor to specify whether participants have to be able to send and/or receive video and/or audio, the range of services to be made available to participants either globally or by rôle type (convenor, conductor, presenter, viewer,...) and the default behaviour of nodes joining the conference.  The convenor may also specify how the conference is managed.  The framework is described in more detail in §, with framework  parameters detailed  in Table 2.

Use of AVC services requires AVC to be present at the Top Provider node and all Network Elements within the conference.  Terminals are not required to support AVC, although its absence is likely to give reduced functionality.  Network Elements may choose to proxy AVC functionality for non-AVC capable terminals; how this is achieved is outside the scope of this recommendation.

### AVC Conference Framework

The framework itself consists of range parameters which allows the convenor  to independently define as many different aspects of a conference as is required.  These include the management regime, the start-up mode, the maximum limits on participation for each node type, the number and types of sessions, the permitted rôles  within a given session etc. The framework allows the convenor the capability when appropriate to very precisely model a real world conferencing requirement as a teleconference event.  Access controls governing the participation in an AVC conference session in a particular rôle can be defined by the convenor, each rôle may have a rôle identifier and rôle access key specified allowing the intended rôle holder on entry to the conference to be identified and assigned the intended rôle.  Groups of nodes can be associated for the purpose of control and the group given a common access key. An ordered group may be defined to provide, for example, a sequence of presenters.  Each member of the group is given a position in the sequence and unique key.  The framework supports the following rôles:

Management Rôles:-	Network Element, Zone Manager, Trusted Node

Participant Rôles:-		Conference Participant, Conductor, Convenor Privileged

Session{n} Rôles:-		Chair, Presenter, Group Member, Audience Peer.

Each rôle can be assigned a rôle identification key and access key which is then used to validate the participating nodes  when they join.  The Top AVC Provider is by default responsible for policing access and assignment to conference rôles and when a joining node provides an AVC rôle key and access key they are passed to the Top AVC Provider for validation.  The means of communicating these keys is out of band and beyond the scope of this recommendation.

A conference framework is passed to the Top AVC Provider when an AVC conference is activated by the convenor.

By using the conference framework detailed models and regimes can be put in place allowing the conferencing  protocols to be configured for a diverse range of applications without burdening the protocol with significant additional complexity.  An AVC Conference framework may consist of only a single parameter or at the other end of the scale it may contain a complete model and management regime for the AVC conference.  The framework is intended to be predefined by the convenor, it may exceptionally be redefined and applied by the convenor or convenor privileged node after an AVC conference has been activated to overcome a problem or  significant change in requirements.  This can avoid the need to tear down the AVC conference.  (Editor’s Note: this needs to be reconciled with the AVC-Activate-Management primitive)

TABLE 2/T.132.

AVC service framework parameters

| Parameter | Description |
| --- | --- |
| Framework Update Flag | This flag determines if the framework may be changed/updated after it has been activated. |
| AVC Management Mode | This parameter can have one of the following three values: Centralised: Indicates that AVC is managed by a single node, the Top AVC Provider (Editor’s note: do we make provision for transferring this responsibility to another node?). Distributed-Unrestricted: Indicates that the conference is managed by a group of Network Elements designated as Zone Managers. Network Elements can pass their zone management responsibilities up the T.120 hierarchy. There are no restrictions on which Network Elements act as Zone Managers. Distributed-By Network: Indicates that the conference is managed by a group of Network Elements designated as Zone Managers. The AVC infrastructure is partitioned into zones by network type. For each distinct sub-network in the conference, there is a single Zone Manager, the Network Element at the top of the T.120 hierarchy within that sub-Network. |
| Maximum Number of Conventional Nodes | An optional parameter imposing an upper limit on the number of conventional nodes allowed to join the conference. |
| Maximum Number of Anonymous Nodes | An optional parameter imposing an upper limit on the number of anonymous (audience) nodes allowed to join the conference. |
| Presenter Rôle Flag | This flag indicates whether use of the Presenter rôle is permitted. |
| Chair Rôle Flag | This flag indicates whether use of the Chair rôle is permitted. |
| Group Flag | This flag indicates whether use of AVC groups is permitted. |
| Public Session Flag | This flag indicates whether use of public sessions is permitted. |
| Private Session Flag | This flag indicates whether use of private sessions is permitted. |
| Permitted AVC Services | A list of optional AVC services which may be used within the conference (identified either explicitly or by service type). |
| Prohibited AVC Services | A list of mandatory AVC services which are prohibited (identified either explicitly or by service type). |
| Additional Sessions: AVC Service Restrictions | A list of AVC services which may be used in the AVC Terminal Standard Base Session, but which are prohibited in additional sessions. |
| Additional Sessions: Presenter Rôle Flag | This flag indicates whether use of the Presenter rôle is permitted in additional sessions. |
| Additional Sessions: Chair Rôle Flag | This flag indicates whether use of the Chair rôle is permitted in additional sessions. |
| Session Descriptor List | An optional list characterising private/public sessions and their participation. For each session, the following parameters may be specified: Session Type: Base/Public/Private Optional Session Access Key Optional Conventional Node Access Key Optional Presenter(s) Access Key(s) Optional Chair Access Key |
| Group Descriptor List | An optional list characterising AVC groups. For each group, the following parameters may be specified: Group Identifier Group Purpose Ordered Group flag Group Access Key Group Participant Identifiers Group Participant Access key |
| Conventional Node Audio Transmit Mandatory/ Optional Flag | This flag indicates whether conventional nodes must have the capability to source audio in order to participate in the conference. |
| Conventional Node Audio Receive Mandatory/ Optional Flag | This flag indicates whether conventional nodes must have the capability to sink audio in order to participate in the conference. |
| Conventional Node Video Transmit Mandatory/ Optional Flag | This flag indicates whether conventional nodes must have the capability to source video in order to participate in the conference. |
| Conventional Node Video Receive Mandatory/ Optional Flag | This flag indicates whether conventional nodes must have the capability to sink video in order to participate in the conference. |
| Anonymous Node Audio Receive Mandatory/ Optional Flag | This flag indicates whether anonymous nodes must have the capability to sink audio in order to participate in the conference. |
| Anonymous Node Video Receive Mandatory/ Optional Flag | This flag indicates whether anonymous nodes must have the capability to sink video in order to participate in the conference. |
| Primary Session Flag | This flag indicates whether the AVC Terminal Standard Base Session acts as Primary Session, or has equal status to all other AVC sessions. In the former case, participants may only transfer to and from the AVC Terminal Standard Base Session. In the latter case participants may transfer between any two sessions. |
| Start-Up Mode | This parameter indicates the default behaviour with respect to audio and/or video for nodes entering the conference, prior to receiving an offer of services from their respective Zone Manager. Valid start-up modes will be defined in Recommendation T.130. |
| AVC Conference Model | Model Identifier |
| Chair AVC Privilege List | This is a list of flags indicating which AVC services the convenor has permitted the Chair to control. |
| Conventional Node AVC Privilege List (Conducted Mode) | This is a list of flags indicating which AVC services the convenor has permitted conventional nodes to access in conducted mode. |
| Conventional Node AVC Privilege List (Non-Conducted Mode) | This is a list of flags indicating which AVC services the convenor has permitted conventional nodes to access in non-conducted mode. |
| AVC Core Services |  |
| Floor Control Flag Control Association Start-up configuration | Required flag Media & Service to be floor controlled |
| AVC Supplementary Services |  |
| Audio Mixing Service permitted/required/prohibited Configuration Parameters Operational mode | Parameters determining if the Audio Mixing service is either permitted, required or prohibited within the conference For Network Elements: Number of streams per port Number of ports available Remote management available flag For Terminals: Number of streams per port Number of speakers per port Distributed/centralised/manual/floor control Service Announcements :enabled/disabled |
| Video Switching Service permitted/required/prohibited Configuration Parameters Control Matrix - Settings | Parameter determining if the Video Switching service is either permitted, required or prohibited within the conference Maximum number of sessions supported Maximum number of receiving channels per terminal Required number of receiving channels per terminal Maximum number of transmitting channels per terminal Required number of transmitting channels per terminal Configuration Access Permissions Overrides Permitted flag Multiple presenters permitted Rotation/ Random Switch interval default VSS Control matrix -Rôle ID (List) Input Mode Output Mode Matrix Access Permissions Sends are choice of:- Rotation, Random, Role, Specified Input Receives are choice of:- Broadcast to all, Broadcast to others, Role Id, Specified outputs(s) New Speaker Receives New Speaker Sends Previous Speaker Previous Speaker Designated presenter(s) Designated presenter(s) Group Receives Group Sends Chair Receives Chair Sends Convenor Sends Convenor Receives Designated Stream Receives Designated Stream Sends Overriding Node Sends Overriding Node Receives |

### Description of Abstract Services

#### AVC-Activate-Management

The AVC-Activate-Management-request primitive enables the Node Controller at the conference convenor node to activate provision of AVC services and to specify a framework for operation of those services.  Once defined, the framework remains in force for the duration of the conference.  The request is validated by the Node Controller at the Top AVC Provider which then instructs all AVC Network Elements to initiate AVC management. The convenor may make use of a priori knowledge (e.g. expected number of participants) to determine the most appropriate time to activate AVC management.

Prior to the receipt of an AVC-Initiate-Management indication primitive, AVC Network Elements are in a dormant state and no AVC services are available from the network.  At this stage, only AVC peer-to-peer functionality such as remote device control is available to AVC terminals.  Following receipt of an AVC-Initiate-Management indication primitive, AVC Network Elements complete their enrolment and AVC Zone Managers may offer AVC services to their respective terminals.

FIGURE 2/T.132

AVC-Activate-Management - sequence of primitives

TABLE 3/T.132.

AVC-Activate-Management

| Content | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) | M(=) |
| Requesting Node |  | M | M(=) |  |
| Convenor Password | M | M(=) |  |  |
| Conference Framework | M |  |  |  |
| Result |  |  | M | M(=) |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User Id of node responsible for activating the AVC Session.

Convenor Password:  This parameter is used to validate that the requesting node has the authority to activate AVC management.  The Node Controller at the Top AVC Provider is responsible for comparing this parameter with the convenor password received in the GCC-Conference-Create-indication.

Conference Framework: A set of parameters defining a framework for provision of AVC services.  See table 2.

Result: An indication of whether the request was successful.  It contains one of the following results: successful, not authorised node, AVC management already active.

#### AVC-Initiate-Management

The AVC-Initiate-Management indication primitive is issued to the Node Controller at all AVC Network Elements to instruct them to complete their enrolment in the AVC Network Element Session and to notify AVC Zone Managers that they may offer AVC services to their respective terminals.  The primitive is issued as a result of a successful AVC-Activate-Management request.  Figure 2 shows the sequence of events leading to the use of this primitive.

TABLE 4/T.132.

AVC-Initiate-Management

| Content | Indication |
| --- | --- |
| Conference Framework | M |

Conference Framework: A set of parameters defining the framework for provision of AVC services.  See table 2.

#### AVC-Announce-Zone-Manager

The AVC-Announce-Zone-Manager indication primitive is used to notify a node of the identity of its Zone Manager and to indicate that the Node Controller may submit capability and configuration information for its node and associated real time links.  Following receipt of this primitive, the Node Controller may issue any of the following primitives, as appropriate:

AVC-Announce-Link-Configuration
AVC-Announce-Node-Configuration
AVC-Announce-Link-Capabilities
AVC-Announce-Node-Capabilities

The Node Controller at an AVC Terminal shall not attempt to issue any AVC service related primitives until it has received an AVC-Service-Offer from the Zone Manager identified in the AVC-Announce-Zone-Manager indication.  Use of AVC peer-to-peer functions is, however, permitted.

FIGURE 3/T.132

AVC-Announce-Zone-Manager - sequence of primitives

TABLE 5/T.132.

AVC-Announce-Zone-Manager

| Content | Indication |
| --- | --- |
| Zone Manager | M |

Zone Manager: AVC User Id of the Network Element acting as Zone Manager for the node.

## AVC Sessions

By default, all terminals participating in an AVC managed conference interact with each other within a single communication session.  However, in some scenarios, there may be a requirement for separate autonomous communications within the same conference.  For example, in a business meeting there may be a requirement for private  conversations in a separate room, whilst a social gathering may consist of a number of discussion groups which participants join according to their interest in the topic of conversation.  AVC defines mechanisms to support the creation of separate communication sessions, to allow the membership of those communication sessions to be controlled, and to allow nodes to transfer from one communication session to another without leaving a conference.

AVC employs two types of application protocol session: the AVC Terminal Session in which AVC Terminals enroll and the AVC Network Element Session in which AVC Network Elements enroll.  The purpose of this dual application session approach is to segregate the nodes managing the conference (the Network Elements) from the nodes participating in the conference (the Terminals).  Essentially, the AVC Network Element Session is a new type of session: a management application protocol session, whose existence does not need to be exposed to terminals.  AVC Network Elements would typically not participate in an AVC Terminal Session unless they also support terminal functionality.

AVC supports the following types of Terminal Session:

- AVC Terminal Standard Base Session: The session in which AVC terminals participate by default.

- AVC Terminal Public Session: An additional communication session with unrestricted participation.  An AVC Terminal Public Session may be used, for example, to establish separate discussion groups.  Nodes may join and leave a public session at will using the AVC-Session-Join service.  A public session is terminated automatically when all nodes have left the session.

- AVC Terminal Private Session: An additional communication session with restricted participation.  An AVC Terminal Private Session may be used, for example, to establish a private side conversation (analogous to leaving a room for a private discussion).  Participation in an AVC Private Session is controlled by the node that created the private session who can invite nodes to join the session using the AVC-Private-Session-Admit service or can force nodes to leave the session with the AVC-Private-Session-Expel primitive.  Nodes join and leave the session using the AVC-Session-Join service.  A private session may be terminated by using the AVC-Private-Session-Terminate service; it is also terminated automatically if the creator of the private session leaves the session.

Each conference has at least one AVC Terminal Session (the Standard Base Terminal Session) and zero or one AVC Network Element Sessions.  Use of multiple AVC Terminal Sessions (public and/or private) allows nodes to break out from the main conference (e.g. for a side discussion), without requiring a new conference to be established.  An AVC Network Element Session would not be present in a point-to-point conference or in a multipoint conference where the network elements do not support AVC.  In such cases, only peer-to-peer functionality such as remote device control, would be available.

Private and public sessions have the same range of services available to them as the standard base session from which they are derived.   However, optional services are launched in the context of a session, so the active services present in one session may differ from the active services present in another session (for example, one session may be using video switching and audio mixing, whilst another session may be using audio mixing only).

An AVC Terminal Session can be considered to be analogous to a meeting room.  A user can be only be in one meeting room at any instance, but may move between rooms as the need arises.  In the same way, A node can only participate in a single AVC Terminal session at any instance, but may transfer between AVC Terminal Sessions.  The effect of this is to remove the node from the services provided in one session and add it to the services provided in the other session. So, for example, if audio mixing is in operation, the effect of a node leaving the Standard Base AVC Terminal Session and joining a Public AVC Terminal Session would be to remove that node from the audio mix in the Standard Base AVC Terminal Session and add the node to the audio mix in the Public AVC Terminal Session. When a node changes session, it adopts the peer rôle in the session it is joining, unless it has the audience rôle, in which case it retains this rôle.

The purpose of a session may be specified explicitly or determined by the framework that is in use. The conference convenor is responsible for specifying whether public or private AVC Terminal sessions may be created as part of the conference framework.

### Description of Abstract Services

#### AVC-Session-Join

The AVC-Session-Join request primitive is used by a Node Controller to transfer from one communication session to another.  It may also be used to establish a new private or public AVC Terminal session and optionally indicate its intended purpose.  A successful request  results in the node leaving the AVC Terminal session in which it is currently enrolled and transfering to the selected (or new) AVC Terminal Session.

FIGURE 4/T.132

AVC-Session-Join - sequence of primitives

TABLE 6/T.132.

AVC-Session-Join

| Content | Request | Confirm |
| --- | --- | --- |
| Request Handle | M | M(=) |
| Session Type | C |  |
| Session Id | C | C |
| Session Context | O |  |
| Result |  | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Session Type: Public or Private: used to specify the required AVC session type when creating a new AVC session.  This parameter is omitted when joining an existing session.

Session Id: The Session Id for the session that the node wishes to join.   If omitted in the request primitive, a new AVC session will be created of the specified session type and its id returned in the confirm primitive.

Session Context: An optional octet string, used to indicate the intended purpose of a new AVC session

Result:  Indicates whether the request was successful.  It contains one of the following results: successful, invalid session id, not authorised node.

#### AVC-Private-Session-Admit

The AVC-Private-Session-Admit request primitive enables a the creator of a private session to invite nodes to participate in that session.  Such nodes may then join the designated session using the AVC-Session-Join service.

FIGURE 5/T.132

AVC-Private-Session-Admit - sequence of primitives

TABLE 7/T.132.

AVC-Private-Session-Admit

| Content | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M |  | M(=) |
| Session Id | M | M(=) |  |
| Invited Node List | O |  |  |
| Session Creator Id |  | M |  |
| Result |  |  | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Session Id: The AVC Session to which nodes are being invited.

Invited Node List: List of AVC User Ids to be admitted to the session.

Session Creator Id: AVC User Id of the node which initiated the AVC-Session-Admit request.

Result:  Indicates whether the request was successful.  It contains one of the following results: successful, not session creator, invalid Session Id.

#### AVC-Private-Session-Expel

The AVC-Private-Session-Expel request primitive enables the creator of a private session  to remove nodes from the session.  Nodes expelled from a private session shall be returned automatically to the AVC Terminal Standard Base Session.

FIGURE 6/T.132

AVC-Private-Session-Expel - sequence of primitives

TABLE 8/T.132.

AVC-Private-Session-Expel

| Content | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M |  | M(=) |
| Session Id | M | M(=) |  |
| Expelled Node List | O |  |  |
| Result |  |  | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Session Id: The AVC Session from which nodes are being expelled.

Expelled Node List: List of AVC User Ids or NULL, indicating that all nodes should be expelled the session.

Result:  Indicates whether the request was successful.  It contains one of the following results: successful, not session creator, invalid SessionID.

#### AVC-Session-Associate

The AVC-Session-Associate request primitive is used by a Node Controller to associate an AVC session with one or more data sessions.  A successful request supersedes any previous associations; the primitive may thus also be used to terminate associations.  By default, all data sessions are implicitly associated with the AVC Terminal Standard Base Session.  Nodes can determine which data sessions are associated with a particular AVC session by consulting the GCC Registry.  The Registry Key of the relevant entry is defined in §9.5.4.

FIGURE 7/T.132

AVC-Session-Associate - sequence of primitives

TABLE 9/T.132.

AVC-Session-Associate

| Content | Request | Confirm |
| --- | --- | --- |
| Request Handle | M | M(=) |
| Session Id | C | C |
| Associated Sessions | O |  |
| Result |  | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Session Id: The Session Id for the AVC session to be associated with data application protocol sessions.

Associate Sessions: A list of Session Ids of data application protocol sessions with which the specified AVC session is to be associated.

Result:  Indicates whether the request was successful.  It contains one of the following results: successful, invalid session id, not authorised node, registry full.

## Capability Reporting Overview

An AVC controlled conference is comprised of a number of nodes that are interconnected in a given configuration by real time links for the transfer of real time information.  Nodes may have differing media capabilities depending on their rôle within the conference. The networks and bitrate available to nodes may also differ, thus real time links within a conference which are on different networks will inherently have different capabilities.  In some cases it may be necessary to try to establish a common conference wide capability, but in many cases this may unnecessarily diminish the communication capability of the conference.

Nodes have one or more communication ports for real time communications.  At any instance, some of these communications ports may be connected to other nodes by real time links and are considered to be active.  Other communication ports may have no associated real time links and are considered to be inactive.  AVC defines capability reporting and link configuration mechanisms to support communication between active ports and provides the ability for nodes to advertise the presence of inactive ports.

Real time link capability reporting may be a multi-stage process.  AVC assumes the presence of an underlying Network Specific Control Entity (NSCE) layer (e.g. H.245 for H.324 PSTN endpoints) which performs point-to-point capability exchange.  Systems which do not support a separate NSCE are for further study.

When a physical real time link is established, a point-to-point capability exchange typically occurs at the Network Specific Control Entity (NSCE) layer.  The results of these negotiations are inherited by AVC at the higher node of the real time link and are then distributed to all Network Elements by AVC (see §).

When a node joins a conference, the Node Controller consults the GCC Conference Roster to determine whether it is the higher or lower node for each of the real time links it is connected to as described above.  For each of those real time links where it is higher in the real time link hierarchy, the node reports the results of the underlying capability exchange to AVC.

Real time link capabilities are divided into two types:

- multiplex capabilities which identify the network type and multiplex type available on the real time link.  They may optionally include Quality of Service (QoS) parameters (e.g. bit rate, latency) which quantify the transmission characteristics of a link.  The actual transmission capacity available at any instance may be less than the values specified in this field due to network conditions (e.g. other traffic on a LAN, PSTN modems training down to a lower data rate).

- media specific capabilities which describe the media formats a link endpoint is able to transmit or receive over the link, e.g. the abilty to receive an H.261 CIF video stream. Essentially they define the modes of operation that the node can support for the given link.

FIGURE 8/T.132

Example of real time link capability record

Once the higher node has announced a set of capabilities for the real time link, subsequent operations are dependent on the mode of the conference as defined at conference creation time.  If the conference is configured for distributed management, decisions on real time link configurations may be made by the owning Zone Manager; if the conference is configured for centralised management, a single entity (the Top AVC Provider)  is responsible for specifying how the AVC infrastructure is to be configured.  The Top AVC Provider is at the same node as the Top GCC Provider and Top MCS Provider.

Observe that AVC does not inherently mandate any requirements for real time link symmetry, although restrictions may be imposed by the underlying networks.

A node may change its link capabilities at any time by simply invoking a capability exchange as outlined in this section.

Nodes may indicate the presence of inactive communication ports by advertising a set of node capabilities.  This mechanism may also be used to advertise the presence of separate devices (e.g. analogue PSTN phone) used to provide real time audio and/or video capabilities.  It may also be used by network elements to advertise their bridging and transcoding capabilities.

### Support for Multiple Conferences on a Node

Nodes (particularly network elements such as MCUs) may be able to participate in more than one conference simultaneously.  AVC manages real time communications on a conference by conference basis.  It is the responsibility of the AVC Node Controller to apportion a node’s resources between a number of conferences.  The method of achieving this is outside the scope of this recommendation.

### Description of Abstract Services

#### AVC-Announce-Link-Capabilities

The AVC-Announce-Link-Capabilities request primitive enables the Node Controller at the upper node of a real time link to advertise a set of audio-visual processing capabilities and network support for a specific real time link.   It is the responsibility of the Node Controller to determine a set of capabilities for the link prior to enrolling in the AVC Terminal Standard Base Session (if the node is an AVC Terminal), or the AVC Network Element Session (if the node is an AVC Network Element).  The AVC-Announce-Link-Capabilities request primitive shall be issued by the Node Controller immediately after having enrolled active in the AVC Terminal Standard Base Session or AVC Network Element Session (as appropriate) and following any subsequent renegotiation of link capabilities by the underlying NSCE.  The content of the AVC-Announce-Link-Capabilities request primitive is typically derived from the outcome of a NSCE capability exchange.

Link capabilities are distributed to all AVC Network Elements.

FIGURE 9/T.132

AVC-Announce-Link-Capabilities - sequence of primitives

TABLE 10/T.132.

AVC-Announce-Link-Capabilities

| Content | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M |  | M(=) |
| Link ID | M | M(=) |  |
| Multiplex Capability Table | O | O(=) |  |
| Media Capability Table | O | O(=) |  |
| Result |  |  | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Link ID: The Link ID uniquely identifies a real time link.  It consists of the GCC User Ids of the endpoints of the real time link and an optional handle to distinguish multiple real time links between the same endpoints.

Multiplex Capability Table: The multiplex capability table characterises the transmission capabilities of the real time link as shown in table 11.

Media Capability Table:  Contains capability records for each media format supported by the node for the real time link.  It may optionally include a list of valid operating configurations (i.e. combinations of media formats that can be supported simultaneously

Result: An indication of whether the request was accepted or rejected.  It contains one of the following results: successful, not link upper node.

TABLE 11/T.132.

Content of multiplex capability table records

| Parameter | Request | Indication |
| --- | --- | --- |
| Multiplex Capability (e.g. H.223, H.225) | M | M(=) |
| Network Type (e.g. ISDN, PSTN) | O | O(=) |
| Up Link: Mean Bit rate (bits/s) | O | O(=) |
| Up Link: Peak Bit rate (bits/s) | O | O(=) |
| Up Link: Default Bit rate -(bits/s) | O | O(=) |
| Down Link: Mean Bit rate (bits/s) | O | O(=) |
| Down Link: Peak Bit rate (bits/s) | O | O(=) |
| Down Link: Default Bit rate (bits/s) | O | O(=) |
| Up Link: Mean Latency (ms) | O | O(=) |
| Up Link: Latency variation (ms) | O | O(=) |
| Down Link: Mean Latency (ms) | O | O(=) |
| Down Link: Latency variation (ms) | O | O(=) |

Multiplex Capability: This is parameter identifies the type (e.g. H.223, H.225) and characteristics of the multiplex used over the real time link.  (N.B. we need to make provision for H.221 multiplex and separate analogue audio connections)

Network Type: Identifies the type of physical network employed by the real time link e.g. ISDN, PSTN.

Up Link: Mean Bit rate: The average bit rate available from the lower node to the upper node, where relevant.

Up Link: Peak Bit rate: The maximum bit rate available from the lower node to the upper node.  For switched networks the bit rate available within a real time link may be changed by adding dropping connections (e.g. adding/releasing B channels in an ISDN call).

Up Link: Default Bit rate: The bit rate initially available from the lower node to the upper node on establishing the real time link.  This term applies to switched networks which may increase the bit rate available to the real time link by adding further connections between the two nodes.

Down Link: Mean Bit rate: The average bit rate available from the upper node to the lower node, where relevant.

Down Link: Peak Bit rate: The maximum bit rate available from the upper node to the lower node.  For switched networks the bit rate available within a real time link may be changed by adding dropping connections (e.g. adding/releasing B channels in an ISDN call).

Down Link: Default Bit rate: The bit rate initially available from the upper node to the lower node on establishing the real time link.  This term only applies to switched networks which may increase the bitrate available to the real time link by adding further connections between the two nodes.

Up Link: Mean Latency: Specifies the average delay between transmission of a bit by the lower node of the real time link and its receipt by the upper node.

Up Link: Latency variation: The difference between the maximum and minimum possible delays that a bit will experience when traversing the real time link from lower node to upper node.  This may be used to determine the amount of buffer space required at the receiving side in order to restore the original signal.

Down Link: Mean Latency: Specifies the average delay between transmission of a bit by the upper node of the real time link and its receipt by the lower node.

Down Link: Latency variation: The difference between the maximum and minimum possible delays that a bit will experience when traversing the real time link from upper node to lower node.  This may be used to determine the amount of buffer space required at the receiving side in order to restore the original signal.

Use of the parameters in the link transmission record is specified in the appropriate Network Specific Mapping document.  Some parameters may not be relevant to specific networks and may thus be omitted.

#### AVC-Announce-Node-Capabilities

The AVC-Announce-Node-Capabilities request primitive enables a Node Controller to advertise audio-visual capabilities that are not currently associated with a specific real time link.  These include:

- Bridging capabilities (e.g. transcoders).

- Unused communication ports which are capable of supporting real time audio or video.

- Separate audio/video communication devices (e.g. analogue PSTN phone).

These capabilities are additional to any capabilities associated with a specific real time link.  Furthermore, they relate to a specific conference, so a node participating in multiple conferences has a distinct set of capabilities for each conference.

This information is forwarded to all AVC Network Elements.  The primitive is issued via the Control AVCSAP.  It may be issued at any time to announce a change in node capabilities.  (We need to define what happens when a spare communication port comes into use: does it require node capabilities to be re-advertised??)

FIGURE 10/T.132

 AVC-Announce-Node-Capabilities - sequence of primitives

TABLE 12/T.132.

AVC-Announce-Node-Capabilities

| Content | Request | Indication |
| --- | --- | --- |
| AVC User ID |  | M |
| Node Type | M | M(=) |
| Port Table | O | O(=) |
| Transcoder Table | O | O(=) |
| Associated Devices | O | O(=) |

AVC User ID: The AVC User ID identifies the node to which the capability set refers.

Node Type: Indicates whether the node is a terminal or one of the following network element types MCU, MC, MP, Gateway or  Conference Server.

Port Table: A list of unused ports available for real time communication.  The format of the Port Table is shown in Table 13.

Transcoder  Table: This characterises the ability of a network element to transcode from one media format to another, as shown in table 14.

Associated Communication Devices:  A list of separate audio/video communication devices associated with the node that do not have an inband capability exchange mechanism (e.g. analogue PSTN phone).  For each device, the following fields may be specified: network type & address and media type (audio and/or video).

TABLE 13/T.132.

Content of Port Table Entry

| Parameter | Description |
| --- | --- |
| Network Address | A T.124 Network Address structure identifying the network address of the port. The parameter includes sub-fields which specify network type information followed by the actual network address or addresses. |
| Multimedia Terminal Profile List | A list of one or more multimedia terminal profiles supported by the communication port. For each profile, an optional media capability table may be specified which contains capability records for each media format supported by the port. This table may optionally include a list of valid operating configurations (i.e. combinations of media formats that can be supported by the port simultaneously). |

TABLE 14/T.132.

Content of Transcoder Table Entry

| Parameter | Description |
| --- | --- |
| Input Media Format | The encoding format of the transcoder input stream. |
| Output Media Format | The encoding format of the transcoder output stream. |
| Bi-directional Flag | Set to TRUE if the transcoder is capable of transcoding an input stream in the output format to an output stream in the input format. |

## Inheriting Existing Audio and Video Communications

AVC supports the ability to inherit audio or video streams established prior to AVC becoming active in a conference.  Control of a real time stream may pass from the NSCE to AVC without causing an interruption in the real time stream.  Similarly, on deactivation of AVC, it may be possible for real time stream control to revert transparently to the NSCE, although there may be a reduction in available functionality.

### Description of Abstract Services

#### AVC-Announce-Link-Configuration

The AVC-Announce-Link-Configuration request primitive enables the Node Controller at the upper node of a real time link to notify AVC Network Elements of an existing real time link configuration and any audio or video communication paths that are already established over that link.  This enables AVC to inherit existing connections that are conveying real time audio or video.  Terminal support for the AVC-Announce-Link-Configuration primitive is optional.  The primitive is issued by a node upon entering a conference.  It may also be issued subsequently to report a change in configuration (this may be due, for example, to user action or equipment failure).

FIGURE 11/T.132

AVC-Announce-Link-Configuration - sequence of primitives

TABLE 15/T.132.

AVC-Announce-Link-Configuration

| Content | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M |  | M(=) |
| Link ID | M | M(=) |  |
| Up Multiplex Configuration | O | O(=) |  |
| Down Multiplex Configuration | O | O(=) |  |
| Up Media Configuration Table | O | O(=) |  |
| Down Media Configuration Table | O | O(=) |  |
| Result |  |  | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Link ID: The Link ID uniquely identifies a real time link.  It consists of the GCC User Ids of the endpoints of the real time link and an optional handle to distinguish multiple real time links between the same endpoints.

Up Multiplex Configuration: This parameter specifies the type and current configuration of the multiplex of the real time link from the lower node to the upper node.

Down Multiplex Configuration: This parameter specifies the type and current configuration of the multiplex of the real time link from the upper node to the lower node.

Up Media Configuration Table: The up media configuration table specifies the mode of operation for the real time link for media streams transmitted from the lower node to the upper node.  The contents of this table are specified in table 16.

Down Media Configuration Table: The down media configuration table specifies the mode of operation for the real time link for media streams transmitted from the upper node to the lower node.  The contents of this table are specified in table 16.

Result: An indication of whether the request was accepted or rejected.  It contains one of the following results: successful, not link upper node.

TABLE 16/T.132.

Content of media configuration table entry

| Parameter | Description |
| --- | --- |
| Mode Element | Describes the format of an existing media stream. |
| Logical Channel Number | Locally unique number identifying the logical channel used to transport the media stream. |
| Multiplex Parameters | An optional H.245 multiplexParameters structure, indicating parameters specific to the multiplex used to transport the logical channel. |
| Logical Channel QoS | An optional structure, indicating the transmission characteristics of the channel. |
| Associated Logical Channels | A list of logical channels on the same real time link, with which the indicated channel is associated (e.g. a stereo signal may be transported in two associated audio channels). Where such channels require synchronising, an indication of the maximum relative latency is provided. |

#### AVC-Announce-Node-Configuration

The AVC-Announce-Node-Configuration request primitive enables a Node Controller to notify AVC Network Elements of an existing node configuration.    This enables network elements to advertise the configuration of any bridging elements (e.g. transcoders) used for existing audio or video streams, thereby allowing AVC to inherit control of those streams.  Terminal support for the AVC-Announce-Node-Configuration primitive is optional.  The primitive is issued by a node upon entering a conference.  It may also be issued subsequently to report a change in configuration (this may be due, for example, to user action or equipment failure).

FIGURE 12/T.132

AVC-Announce-Node-Configuration - sequence of primitives

TABLE 17/T.132.

AVC-Announce-Node-Configuration

| Content | Request | Indication |
| --- | --- | --- |
| AVC User ID |  | M |
| Bridge Configuration Table | O | O(=) |

AVC User ID: The AVC User ID identifies the node to which the configuration table refers.

Bridge Configuration Table:  The bridge configuration table identifies any incoming logical channels on real time links that are bridged to outgoing logical channels.  If an outgoing logical channel has a different format from the incoming logical channel, then it is assumed that the node is transcoding between the two media formats.  The logical channels themselves are advertised using the AVC-Announce-Link-Configuration primitive.  The contents of the bridge configuration table are shown in table 18.

TABLE 18/T.132.

Content of bridge configuration table records

| Parameter | Description |
| --- | --- |
| Incoming Real Time Link Id | Identifier of the link on which the incoming media stream is transported |
| Input Channel | Logical Channel (identified by Logical Channel Number or Multipoint Channel Number) in which the incoming media stream is transported |
| Outgoing Real Time Link Id | Identifier of the link to which the incoming media stream is being routed |
| Output Channel | Logical Channel (identified by Logical Channel Number or Multipoint Channel Number) in which the outgoing media stream is transported |

Question: How do we advertise the presence of an active (separate) PSTN analogue audio connection?  What parameters do we need to characterise it?

## The AVC Link and Node Rosters

The AVC Rosters allow nodes managing the real time aspects of a conference to learn about the resources available to the conference for real time communications.  Sufficient information is maintained to allow the nodes to make intelligent decisions about the use of those resources for specific AVC services.

AVC has two rosters:

- The Link Roster which is a database of the capabilities and current configuration of real time links

- The Node Roster which is a database of the capabilities and current configuration of real time link endpoints (nodes).

Both the Link Roster and Node Roster are distributed to AVC Network Elements; AVC Terminals do not automatically receive AVC Rosters but may retrieve roster information on request.  As a minimum, an AVC Network Element shall maintain a copy of that portion of each roster pertaining to the AVC resources for which it is responsible.  By default, AVC roster capability information is only disseminated to AVC Network Elements.  AVC roster configuration information is only distributed to those nodes which have requested notification of configuration changes in either specific portions or any part of the AVC infrastructure.

All roster entries are associated with a specific resource; changes to a roster entry may only be made by the AVC Network Element which owns that resource.

Roster updates are generated following receipt of one of the following:

- AVC-Announce-Link-Capabilities request primitive

- AVC-Announce-Node-Capabilities request primitive

- AVC-Announce-Link-Configuration request primitive

- AVC-Announce-Node-Configuration request primitive

- Roster monitor indications are generated following receipt of one of the following:

- SetLinkMultiplexResponse AVCPDU

- SetLinkTransmissionModeResponse AVCPDU

- SetNodeConfigurationResponse AVCPDU

- OpenLogicalChannelResponse AVCPDU

- CloseLogicalChannelResponse AVCPDU

- ConfirmResourceReservationResponse AVCPDU

- AVC-Relinquish-Resource request primitive

### Description of Abstract Services

#### AVC-Roster-Report

An AVC-Roster-Report indication is generated by an AVC Provider when it detects that a change to the Link Roster or Node Roster has occurred.  AVC Terminals must explicitly request AVC Roster information using the AVC-Roster-Retrieve primitive or AVC-Roster-Monitor primitive, as appropriate.  Changes to an AVC Roster may occur as a result of a node entering or leaving the conference, due to revised capabilities being advertised or new configurations being set.  Roster Reports may be presented as incremental updates to previously held information or as full refreshes.

TABLE 19/T.132.

AVC-Roster-Report-Indication

| Content | Indication |
| --- | --- |
| Updated Link Roster | O(=) |
| Updated Node Roster | O(=) |

Updated Link Roster: A list of revised real time link capability and configuration records as specified in Table 20.

Updated Node Roster: A list of revised node capability and configuration records as specified in Table 21.

TABLE 20/T.132.

Content of Link Roster

| Parameter | Description |
| --- | --- |
| Full Link Capability Refresh Flag | This flag is set if the link capability roster contains a full refresh, replacing all existing link capability entries. |
| Full Link Configuration Refresh Flag | This flag is set if the link configuration roster contains a full refresh, replacing all existing link configuration entries. |
| Real Time Links Removed List (optional) | A list of Link Ids corresponding to real time links that have been removed from the conference. |
| Real Time Link Capabilities Added List (optional) | A list of one or more link capability records corresponding to real time links that have been added to the conference. |
| Real Time Link Capabilities Modified List (optional) | A list of one or more link capability records corresponding to existing real time links whose capabilities have been updated. |
| Real Time Link Configurations Added List (optional) | A list of one or more link configuration records corresponding to real time links that have been added to the conference. |
| Real Time Link Configurations Modified List (optional) | A list of one or more link configuration records corresponding to existing real time links whose configurations have been updated. |

TABLE 21/T.132.

Content of Node Roster

| Parameter | Description |
| --- | --- |
| Full Node Capability Refresh Flag | This flag is set if the node capability roster contains a full refresh, replacing all existing node capability entries. |
| Full Node Configuration Refresh Flag | This flag is set if the node configuration roster contains a full refresh, replacing all existing node configuration entries. |
| Nodes Removed List (optional) | A list of AVC User Ids corresponding to nodes that have left the conference. |
| Node Capabilities Added List (optional) | A list of one or more node capability records corresponding to nodes that have joined the conference. |
| Node Capabilities Modified List (optional) | A list of one or more node capability records corresponding to existing nodes whose capabilities have been updated. |
| Node Configurations Added List (optional) | A list of one or more node configuration records corresponding to nodes that have joined the conference. |
| Node Configurations Modified List (optional) | A list of one or more node configuration records corresponding to existing nodes whose configurations have been updated. |

FIGURE 13/T.132

AVC-Roster-Report - sequence of primitives

#### AVC-Roster-Enquire

The AVC-Roster-Enquire request primitive is used to retrieve a portion of an AVC Roster corresponding to a specific link or node.  It may also be used to retrieve information for a group of links or nodes, or to retrieve a complete AVC Roster.  This allows entities delivering AVC services to selectively access the information they require, without having to maintain superfluous information.  This primitive may be issued by any node (typically, it would be used by AVC Network Elements that are unable to maintain a complete copy of the specified AVC Roster).

It may be directed to a specific Zone Manager, hence to retrieve a complete AVC Roster, a node may have to issue a number of AVC-Roster-Enquire requests to different Zone Managers.  Furthermore, if a specific roster entry is required, the requesting node should ensure that the request is directed towards the appropriate (i.e. owning) Zone Manager.

Alternatively, the AVC-Roster-Enquire request primitive may be broadcast to all AVC Network Elements.  This method is appropriate if a node does not know the identity of the Zone Manager responsible for maintaining the link or node information required.  In this case, each AVC Network Element that is also a Zone Manager evaluates the request and responds only if the request relates to a portion of an AVC Roster for which it is responsible.  To avoid duplication, a Zone Manager shall not provide information relating to resources outside its own Zone.  AVC Network Elements that are not Zone Managers may ignore the request. (Editor’s Note: Observe that a request relating to multiple links or nodes may elicit more than one response, and that a request with an invalid roster key may produce no response at all.  Is this acceptable?)

Permission to issue the AVC-Roster-Enquire request primitive may be restricted by the conference convenor and/or conductor.

FIGURE 14/T.132

AVC-Roster-Enquire - sequence of primitives

TABLE 22/T.132

AVC-Roster-Enquire Primitive

| Parameter | Request | Confirm |
| --- | --- | --- |
| Request Handle | M | M(=) |
| Zone Manager | (O) |  |
| Roster Key | O | O(=) |
| Capability Flag | M |  |
| Configuration Flag | M |  |
| List of Roster Entries |  | C |
| Result |  | M |

Request Handle: Locally unique number to allow the requesting node to match confirms.

Zone Manager: AVC User Id of the Zone Manager to which the request is to be directed.  If omitted, the request is directed to all Zone Managers.

Roster Key: Specifies the record(s) to retrieve.  A Roster Key consists of two components, one (Entity Type) which identifies the type of record to be retrieved (node, link) and the other which uniquely identifies the specific entity to be retrieved.  If the second component is omitted, the Zone Manager shall return all roster entries of the specified type.

Capability Flag: Set to TRUE if retrieval of capability information is required, otherwise FALSE.

Configuration Flag: Set to TRUE if retrieval of configuration information is required, otherwise FALSE.

List of Roster Entries: One or more roster entries corresponding to the Roster Key supplied in the request.

Result: Successful, insufficient privilege, entry not found, not entry owner.

#### AVC-Roster-Monitor

The AVC-Roster-Monitor request primitive is used to monitor changes to link or node attributes.  The AVC-Roster-Monitor primitive may be used to enable/disable monitoring of a specific link or node or a group of links or nodes.    Changes may arise due to a revised capability set being advertised, reconfiguration, or nodes leaving the conference.  By default, all AVC Network Elements maintain a copy of the Link and Node Rosters.  Whilst enabled, the requesting entity is notified of all changes to the specified roster entries via the AVC-Roster-Report indication primitive.

The AVC-Roster-Monitor request primitive is directed to a specific Zone Manager, hence to monitor multiple roster entries, a node may have to issue a number of AVC-Roster-Monitor requests to different Zone Managers.  Furthermore, if monitoring of a specific roster entry is required, the requesting node should ensure that the request is directed towards the appropriate (i.e. owning) Zone Manager.  A Zone Manager receiving a monitor request relating to resources outside its own zone may choose to fulfil that request if it maintains an accurate local copy of the information.

Permission to issue the AVC-Roster-Monitor request primitive may be restricted by the conference convenor and/or conductor.

FIGURE 15/T.132

AVC-Roster-Monitor - sequence of primitives

TABLE 23/T.132

AVC-Roster-Monitor Primitive

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M |  | M(=RQ) |
| Zone Manager | M |  |  |
| Roster Key | O | M | O(=RQ) |
| Capability Flag | M |  | M(=RQ) |
| Configuration Flag | M |  | M(=RQ) |
| Enable/Disable | M |  | M(=RQ) |
| Media type | O | O | O(=RQ) |
| Roster Entry |  | M |  |
| Result |  |  | M |

Request Handle: Locally unique number to allow the requesting node to match confirms.

Zone Manager: AVC User Id of the Zone Manager to which the request is to be directed.

Roster Key: Specifies the record(s) to be monitored.  A Roster Key consists of two components, one (Entity Type) which identifies the type of record to be monitored (node, link) and the other which uniquely identifies the specific entity to be monitored.  If the latter is omitted, monitoring will be enabled/disabled for all entities of the specified type.   The media type parameter may be used to provide further qualification of the entries to be monitored.

Capability Flag: Set to TRUE if monitoring of capability information is required, otherwise FALSE.

Configuration Flag: Set to TRUE if monitoring of configuration information is required, otherwise FALSE.

Enable/Disable: TRUE to deliver indications of changes, FALSE to suppress indications.

Media Type: Audio, Video or Real Time Data.  Only applicable when enabling monitoring.  If omitted, monitoring is enabled for all media types.

Roster Entry: Describes the entity whose attributes have changed

Result: Successful, insufficient privilege, entry not found, not entry owner, function not supported.

## Resource Configuration

AVC provides a number of configuration primitives to enable real time links and nodes to be configured in order to deliver AVC services.  To avoid potential conflict, links and nodes may only be configured by their respective owning AVC Zone Managers.  In the case of a link that crosses a zone boundary, ownership is shared between the Zone Managers on either side of the boundary.  Each Zone Manager is responsible for configuring its respective outgoing portion of the link.

The primitives defined in this section allow an AVC Zone Manager to configure resources that it owns, but which it cannot configure directly using a NSCE.  The following section describes how an AVC Zone Manager can request the configuration of resources managed by another AVC Zone Manager.

Resource configuration transactions may only be initiated by an AVC Zone Manager.

### Description of Abstract Services

#### AVC-Set-Link-Multiplex

The AVC-Set-Link-Multiplex request primitive is used by the Node Controller at an AVC Zone Manager to instruct a node to configure its outgoing (i.e. transmit) multiplex on a particular real time link.  This primitive may be only be issued by an AVC Zone Manager to configure a real time link under its jurisdiction.  It is the responsibility of the node receiving the request to ensure that other link endpoints have configured their incoming (i.e. receive) multiplexes before responding to the link owner.  The method of achieving this is outside the scope of this recommendation.

The Node Controller at an AVC Zone Manager may configure its its outgoing multiplex on local (i.e. directly connected) links by interaction with the appropriate local NSCE(s) and does not need to use the AVC-Set-Link-Multiplex primitive.

Receipt of an AVC-Set-Link-Multiplex confirm primitive with a successful result indicates that the multiplex has been reconfigured as requested.  An unsuccessful result indicates that the multiplex remains unchanged.

FIGURE 16/T.132

AVC-Set-Link-Multiplex - sequence of primitives

TABLE 24/T.132.

AVC-Set-Link-Multiplex

| Content | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) | M(=) |
| Requesting Node |  | M | M(=) |  |
| Real Time Link Id | M | M(=) |  |  |
| Direction | M | M(=) |  |  |
| Multiplex Configuration | O | O |  |  |
| Result |  |  | M | M(=) |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User ID of the requesting node.

Real Time Link Id: Identifier of the link to which the primitive refers.

Direction: Up to configure the outgoing multiplex of the lower node, Down to configure outgoing multiplex of the higher node.

Multiplex Configuration: This parameter indicates the required multiplex configuration for the real time link.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why.  It contains one of the following results: successful, not link owner, unspecified failure.

#### AVC-Set-Link-Transmission-Mode

The AVC-Set-Link-Transmission-Mode request primitive is used by the Node Controller at an AVC Zone Manager to instruct a node to adopt a new operating mode for its outgoing media streams on a particular real time link.  The media configuration specified in the primitive replaces the existing configuration.  This primitive may be only be issued by an AVC Zone Manager to configure a real time link under its jurisdiction.

The Node Controller at an AVC Zone Manager may configure its outgoing media streams on local (i.e. directly connected) links by interaction with the appropriate local NSCE(s) and does not need to use the AVC-Set-Link-Transmission-Mode primitive.

It is a local matter for a Node Controller receiving an AVC-Set-Link-Transmission-Mode indication to determine whether it is able to re-use any open logical channels on the link to support the new configuration, or whether it must close existing channels and open a new set of logical channels.  In either case, the logical channel used to transport T.120 data shall not be closed.

Receipt of an AVC-Set-Link-Transmission-Mode confirm primitive with a successful result indicates that the new configuration has been implemented.  An unsuccessful result indicates that the configuration remains unchanged.

FIGURE 17/T.132

AVC-Set-Link-Transmission-Mode - sequence of primitives

TABLE 25/T.132.

AVC-Set-Link-Transmission-Mode

| Content | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) | M(=) |
| Requesting Node |  | M | M(=) |  |
| Real Time Link Id | M | M(=) |  |  |
| Direction | M | M(=) |  |  |
| Media Configuration Table | O | O |  |  |
| Result |  |  | M | M(=) |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User ID of the requesting node.

Real Time Link Id: Identifier of the link to which the primitive refers.

Direction: Up to configure media streams sent from the lower node to the higher node, Down to configure media streams from the higher node to the lower node.

Media Configuration Table:  The media configuration table characterises the media streams that the link endpoint is required to transmit.  The requesting node may optionally specify multipoint channel identifiers to associate the resultant point-to-point logical channels with particular multipoint channels.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why.  It contains one of the following results: successful, not link owner, mode unavailable, multipoint constraint, request denied, unspecified failure.

#### AVC-Set-Node-Configuration

The AVC-Set-Node-Configuration request primitive is issued by the Node Controller at an AVC Zone Manager to configure one of its AVC nodes in a particular operating mode.  It can be used bridge media streams from one link to another, or to cease bridging a media streams.  Transcoding is performed implicitly if the input and output logical channels specified are configured for different formats.

Receipt of an AVC-Set-Node-Configuration confirm primitive with a successful result indicates that the node has been reconfigured as requested.  An unsuccessful result indicates that the node configuration remains unchanged.

FIGURE 18/T.132

AVC-Set-Node-Configuration - sequence of primitives

TABLE 26/T.132.

AVC-Set-Node-Configuration

| Content | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) | M(=) |
| Requesting Node |  | M | M(=) |  |
| Destination Node | M |  |  |  |
| Bridge Configuration Table | O | O |  |  |
| Result |  |  | M | M(=) |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User ID of the requesting node.

Destination Node:  AVC User Id of the node to be reconfigured.

Bridge Configuration Table: The bridge configuration table identifies any incoming logical channels on real time links that are to be bridged to outgoing logical channels.  If an outgoing logical channel has a different format from the incoming logical channel, then it is assumed that the node is transcoding between the two media formats.  The table may also identify pairings of logical channels between which bridging is to cease.  The contents of  bridge configuration table entries are shown in table 18.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why.  It contains one of the following results: successful, not node owner, invalid bridge entry, unspecified failure.

## Logical Channel Management

The AVC logical channel management primitives are used to effect incremental changes to the configuration of real time links by allowing individual logical channels to be opened and closed for specific media flows.  Global changes to the configuration of the media streams transported on a real time link are effected using the AVC-Set-Link-Transmission-Mode primitive.

### Description of Abstract Services

#### AVC-Open-Logical-Channel

The AVC-Open-Logical-Channel request primitive is issued by the Node Controller at an AVC Zone Manager to open a logical channel within one of the real time links under its jurisdiction. Outgoing logical channels on local (directly connected) real time links would typically be opened using the appropriate NSCE, hence this primitive is only required to open logical channels on links which are not directly connected to their owning AVC Zone Manager and incoming logical channels on directly connected real time links.

FIGURE 19/T.132

AVC-Open-Logical-Channel - sequence of primitives

TABLE 27/T.132.

AVC-Open-Logical-Channel

| Content | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) | M(=) |
| Requesting Node |  | M | M(=) |  |
| Direction | M | M(=) |  |  |
| Real Time Link Id | M | M(=) |  |  |
| Logical Channel Number |  |  | M | M(=) |
| Multipoint Channel Id | O | O(=) |  |  |
| Media Type | M | M(=) |  |  |
| Multiplex Parameters | O | O(=) |  |  |
| Logical Channel QoS | O | O(=) | O | O(=) |
| Associated Logical Channels | O | O(=) |  |  |
| QoS Level |  |  | O | O(=) |
| Result |  |  | M | M(=) |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User ID of the requesting node.

Real Time Link Id:  The real time link in which the logical channel is to be opened.

Direction: Up for a logical channel from the lower node to the higher node, Down for a logical channel from the higher node to the lower node.

Logical Channel Number:  Locally unique number identifying the logical channel within the real time link.

Multipoint Channel Id: A globally unique number identifying the multipoint channel to which the logical channel belongs.

Media Type: Indicates the type of media to be conveyed within the logical channel (i.e. audio, video, or real time data) and the format of the media.

Multiplex Parameters: An optional H.245 multiplexParameters structure, indicating parameters specific to the multiplex used to transport the logical channel.

Logical Channel QoS:  Desired Quality of Service is included in the request.  The actual QoS is provided in the confirm.

Associated Logical Channels: A list of logical channels (on the same real time link) with which the new logical channel is to be associated.  The list identifies any logical channel on the real time link to which the new logical channel is to be synchronised, and specifies the maximum relative latency between the logical channels.  (This parameter should specify a descriptor to apply meaning to the association e.g. two audio logical channels used to distribute a stereo signal, a video logical channel & its associated audio logical channels(s).)  Issue: association should probably be between paths and not logical channels.  Keep here pending a decision.

QoS Level:  An optional parameter indicating the level of service available on the channel.  It contains one of the following values - guaranteed service, predictive service, best effort service.

Result:  Indicates whether the logical channel was opened.  It contains one of the following results: successful, unspecified failure, not link owner, unsuitable reverse parameters, data type not supported, data type not available, unknown data type, data type AL combination not supported, multicast channel not allowed, invalid RTP session id

#### AVC-Close-Logical-Channel

The AVC-Close-Logical-Channel request primitive is issued by the Node Controller at an AVC Zone Manager to close a logical channel within one of the real time links under its jurisdiction. Outgoing logical channels on local (directly connected) real time links would typically be closed using the appropriate NSCE, hence this primitive is only required to close logical channels on links which are not directly connected to their owning AVC Zone Manager and incoming logical channels on directly connected links.

FIGURE 20/T.132

AVC-Close-Logical-Channel - sequence of primitives

TABLE 28/T.132.

AVC-Close-Logical-Channel

| Content | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) | M(=) |
| Requesting Node |  | M | M(=) |  |
| Real Time Link Id | M | M(=) |  |  |
| Direction | M | M(=) |  |  |
| Logical Channel Number | M | M(=) |  |  |
| Result |  |  | M | M(=) |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User ID of the requesting node.

Real Time Link Id:  The real time link in which the logical channel is to be closed.

Direction: Up for a logical channel from the lower node to the higher node, Down for a logical channel from the higher node to the lower node.

Logical Channel Number:  Locally unique number identifying the logical channel within the real time link.

Result:  Indicates whether the logical channel was closed.  It contains one of the following results: successful,  unspecified failure, not link owner, invalid channel id.

## Resource Arbitration

AVC provides a group of  primitives which enable AVC Zone Managers to communicate with each other to reserve and configure resources required by an AVC Service.  This set of primitives allows an AVC Zone Manager to request use of resources owned by another AVC Zone Manager.  They allow an AVC Zone Manager processing an AVC service request to confirm the availability of all necessary resources owned by different AVC Zone Managers, before configuring the resources.  This avoids the need to configure and subsequently release resources owned by other AVC Zone Managers if it is discovered that some of the required resources are unavailable.

Resource requests are implicit i.e. the requesting AVC Zone Manager defines the service requirement rather than explicitly specifying the resources required.  The owning AVC Zone Manager thus has the freedom to assign the most appropriate resources to fulfil the request.  For example, an AVC Zone Manager may request a communication path to convey a video stream to a group of endpoints.  Included within this request is a specification of the expected video format at any endpoints which are on zone boundaries (to ensure that Zone Managers of adjacent zones do not select different formats for streams which cross zone boundaries).  Essentially, the requesting Zone Manager is specifying how streams pass between zones, but not how they are distributed within a zone.  The Node Controller at the owning AVC Zone Manager would select the most appropriate method to distribute the stream (e.g. multicast channel or a set of unicast channels) within its own zone, including the location of any stream processing elements (e.g. transcoders), consistent with the boundary constraints and QoS requirements for the stream.  Resource management algorithms to support such decisions are outside the scope of this recommendation.  The AVC Zone concept enables service providers to specify and evolve their own resource management strategies without affecting interoperability or requiring the protocol to be revised.

AVC Terminals do not support the resource management primitives.

### Description of Abstract Services

#### AVC-Reserve-Resource

This primitive allows the Node Controller at an AVC Zone Manager to reserve a set of resources for use by an AVC service.  Requests may only be made to reserve resources owned by the Zone Manager to which the request is directed.  Requests to reserve resources that are in use or are already reserved shall be rejected by the owning Zone Manager.  If a request fails due to insufficient resources, the requesting node may modify the request (e.g. reduce QoS requirements) and make a further attempt.

Reservations made by AVC Network Elements that subsequently leave the conference without confirming or cancelling the reservation shall lapse.  The action to be taken for confirmed reservations is tbd.

FIGURE 21/T.132

AVC-Reserve-Resource - sequence of primitives

TABLE 29/T.132.

AVC-Reserve-Resource

| Content | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) | M(=) |
| Requesting Node |  | M | M(=) |  |
| Destination Node | M |  |  |  |
| Resource Descriptor | M | M(=) |  |  |
| Reservation Handle |  |  | M | M(=) |
| QoS Level |  |  | O | O(=) |
| Result |  |  | M | M(=) |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User ID of the requesting node.

Destination Node: The AVC User ID of the Network Element that owns the requested resource.

Resource Descriptor: Identifies the resource required.

Reservation Handle: Locally unique number to allow the responding node to match subsequent attempts to confirm or cancel a reservation.

QoS Level:  An optional parameter indicating the level of service available.  It contains one of the following values - guaranteed service, predictive service, best effort service.  (Editor’s note: is this parameter only applicable to a multipoint channel request?)

Result:  Indicates whether the requested resources are available.  It contains one of the following results: successful, not Zone Manager, invalid Destination Node ID, invalid Resource Descriptor, insufficient resources, unspecified failure.

TABLE 30/T.132.

Multipoint Channel Resource Parameters

| Parameter | Description |
| --- | --- |
| Multipoint Channel Id | Globally unique number identifying the multipoint channel. |
| Media Type | Indicates the type of media to be conveyed within the multipoint channel(i.e. audio, video, or real time data). |
| Source Node | The AVC User Id of the node from which the media flow (stream) originates. This may either be an AVC Terminal within the Zone or an AVC Network Element at the zone boundary. If the source node is on the zone boundary, the media format (e.g. H.261 CIF) shall also be specified. |
| Destination Node List | A list of AVC User Ids of AVC Terminals within the Zone and AVC Network Elements at the zone boundary to which the stream is to be distributed. For each node on the zone boundary, the media format (e.g. H.261 CIF) shall also be specified. A Zone Manager may select an appropriate media format for terminals within its own Zone, subject to the appropriate QoS criteria being fulfilled, but must adhere to the specified media format for nodes on the zone boundary. |
| QoS | Identifies any generic QoS requirements on the multipoint channel, additional to those implicit in the media type. (Same QoS for all endpoints?) |
| Synchronisation | A list of multipoint channels to which the new multipoint channel is to be synchronised. For each multipoint channel an indication of the maximum relative latency (+ and -) is specified. |

#### AVC-Cancel-Resource-Reservation

This primitive allows the Node Controller at an AVC Network Element to cancel a previous resource reservation.  This would typically occur if an AVC Network Element was unable to obtain sufficient resources to deliver an AVC service.  On receipt of a valid AVC-Cancel-Resource-Reservation indication primitive, an AVC Network Element shall release any resources associated with the reservation indicated.

FIGURE 22/T.132

AVC-Cancel-Resource-Reservation - sequence of primitives

TABLE 31/T.132.

AVC-Cancel-Resource-Reservation

| Content | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) |
| Requesting Node |  | M |  |
| Destination Node | M | M(=) |  |
| Reservation Handle | M | M(=) |  |
| Result |  |  | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User ID of the requesting node.

Destination Node: The AVC User ID of the Network Element to which the cancellation is directed.

Reservation Handle: Identifier assigned to the reservation to be cancelled in a preceding AVC-Reserve-Resource transaction.

Result:  Indicates whether the request was successful.  It contains one of the following results: successful, invalid  reservation handle.

#### AVC-Confirm-Resource-Reservation

This primitive allows the Node Controller at an AVC Network Element to confirm a previous resource reservation.  On receipt of a valid AVC-Confirm-Resource-Reservation indication primitive, an AVC Network Element shall arrange for the necessary resources to be configured and be made available for use before responding to the requesting node.

FIGURE 23/T.132

AVC-Confirm-Resource-Reservation - sequence of primitives

TABLE 32/T.132.

AVC-Confirm-Resource-Reservation

| Content | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) | M(=) |
| Requesting Node |  | M | M(=) |  |
| Destination Node | M | M(=) |  |  |
| Reservation Handle | M | M(=) |  |  |
| Result |  |  | M | M(=) |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User ID of the requesting node.

Destination Node: The AVC User ID of the Network Element to which the confirmation is directed.

Reservation Handle: Identifier assigned to the reservation to be confirmed in a preceding AVC-Reserve-Resource transaction.

Result:  Indicates whether the request was successful.  It contains one of the following results: successful, no such reservation, reservation lapsed, unspecified resource configuration failure.

#### AVC-Relinquish-Resource

This primitive allows the Node Controller at an AVC Network Element to release resources owned by another AVC Network Element.  On receipt of a valid AVC-Relinquish-Resource indication primitive, an AVC Network Element shall release the associated resources.  It is a local matter for the AVC Network Element to choose whether to reconfigure its portion of the AVC infrastructure (e.g. closing logical channels) or to leave it intact but dormant.

Question:  How do we deal with modification of resources?

FIGURE 24/T.132

AVC-Relinquish-Resource - sequence of primitives

TABLE 33/T.132.

AVC-Relinquish-Resource-Reservation

| Content | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) |
| Requesting Node |  | M |  |
| Destination Node | M | M(=) |  |
| Reservation Handle | M | M(=) |  |
| Result |  |  | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User ID of the requesting node.

Destination Node: The AVC User ID of the Network Element to which the request is directed.

Reservation Handle: Identifier assigned to the reservation to be relinquished in a preceding AVC-Reserve-Resource transaction.

Result:  Indicates whether the request was successful.  It contains one of the following results: successful, no such reservation.

# AVC Service Management Service Definition

## Introduction to AVC Conference Services

This section defines a number of  conference support services which are provided to the conference by Network Elements such as MCUs. These conference services are divided into two categories; the first are known as core services, which are more generic in nature and are a mandatory part of the AVC protocol for both network elements and terminals. The second category are known as supplementary services, which include services which provide specific functionality such as video switching, audio mixing and continuous presence.  The requirement for supplementary services may be pre-defined by the convenor, or may be requested after conference start-up by a convenor privileged node.  In both of the above cases the ability of a conference to support a supplementary service must be determined by negotiation using the GCC capability exchange mechanism. Each of the network elements participating in an AVC conference joins the conference management session and advertises any capabilities it has to support supplementary conference services.

Each service may consist of a mandatory part and optional enhancements. The use of enhancements is negotiated using the GCC capability exchange mechanism.

Each of the AVC services will fall into one of four service classifications, each of which are specified below.

### Class 1 - Core Services

Core services aim to provide generic AVC functionality and tools, they are a mandatory part of the AVC protocol and therefore not negotiated. The Core Services provided by AVC are -

·	Multipoint Channel Management

·	Source Identification

·     On Air Indication

·     Audio and Video Floor Control

### Class 2 - Supplementary Services

Supplementary AVC services define dedicated protocol to provide specific functionality such as Audio Mixing and Video Switching. Each service is assigned a standardised identifier which is used in the GCC capability negotiation to advertise its baseline capability to other nodes. Each of the services allows explicit negotiation of any standardised options. Each of these services selects an appropriate negotiation rule. The supplementary services provided by AVC are :-

·     Audio Mixing

·	Video Switching

·     Continuous Presence

·	Multimedia Database

### Class 3 - Custom Services

The third class of service is effectively a custom service and can be regarded as being unique to a specific network element. AVC has no prior knowledge of the service  and no negotiation takes place. AVC zone managers will become aware of the service capability from the GCC capability exchange and if the conference rules allow they will offer the service to their terminals.

A terminal subscribing to a custom service will be directed to that service provider directly, however the zone manager will still continue to own its terminals resources, which must be requested if required.

A custom service does not have  a pre-defined identifier, it is  identified by a user-defined string and a  unique numeric identifier, which is  derived from a locally unique service number appended to its GCC user-id. It operates its service  using the standard infra-structure and core services  protocol.

### Class 4 - User Defined Services

The third class of service are  user-defined, network elements  that recognise the service key and support corresponding functionality may participate in the service negotiation

### AVC Peer-to-Peer Functions

AVC also provides a number of peer-to-peer functions which are not dependent on the presence of AVC capable network elements to use or be useful.  They may thus be used in point-to-point conferences and multipoint conferences where the network elements are not operating AVC.

·	Privacy

·	Source Selection

·	Remote Device Control

AVC Service Support rules

In order for AVC to negotiate any service capability for a conference it must also understand what level of support various Network Elements are required to provide. To achieve this, each AVC service adopts  one of the following  rules for support-

A - All participating MPs and MCUs must support the service.

B - All Zone managers must support the Service. Note not sure if this needed ?

C- One or more NEs support the service.

Each service defines a management session protocol needed in order to provide the service, and a user to management protocol needed for access to; and delivery of the service.

Where a single node  has multiple participants sharing the same connection it should register as both terminal  and a Network Element participating in any services which it can participate and contribute too.

AVC reserves the  numbers  0-1000 for standardised AVC service identifiers.

## Advertising and Accessing AVC Services

AVC defines mechanisms to allow Zone Managers to identify themselves to their respective AVC Terminals and to make an offer of AVC Services to those nodes.  Some services may be designated as active, in which case AVC Terminals may make use of their functionality immediately.  Other services may be designated as inactive; AVC Terminals must subscribe to these services before attempting to use them.  Procedures are defined for launching AVC services.

## Summary of Conference Services

TABLE 34/T.132.

AVC service management primitives and PDUs

| Functional Unit | Primitives | AVCTmn | AVCN.E. | Associated PDUs | Dir. | AVCTmn | AVCN.E. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AVC | AVC-Service-Offer request |  |  | ServiceOfferRequest |  |  |  |
| Service | AVC-Service-Offer indication |  |  | ServiceOfferRequest |  |  |  |
| Access | AVC-Service-Offer confirm |  |  | ServiceOfferResponse |  |  |  |
|  | AVC-Service-Subscribe request |  |  | ServiceSubscribeRequest |  |  |  |
|  | AVC-Service-Subscribe indication |  |  | ServiceSubscribeRequest |  |  |  |
|  | AVC-Service-Subscribe confirm |  |  | ServiceSubscribeResponse |  |  |  |
|  | AVC-Service-Subscribe-Report indication |  |  | ServiceSubscribeReportIndication |  |  |  |
|  | AVC-Service-Activate request |  |  | ServiceActivateRequest |  |  |  |
|  | AVC-Service-Activate indication |  |  | ServiceActivateRequest |  |  |  |
|  | AVC-Service-Activate response |  |  | ServiceActivateResponse |  |  |  |
|  | AVC-Service-Activate confirm |  |  | ServiceActivateResponse |  |  |  |
|  | AVC-Service-Launch request |  |  | ServiceLaunchRequest |  |  |  |
|  | AVC-Service-Launch indication |  |  | ServiceLaunchRequest |  |  |  |
|  | AVC-Service-Launch response |  |  | ServiceLaunchResponse |  |  |  |
|  | AVC-Service-Launch confirm |  |  | ServiceLaunchResponse |  |  |  |
|  | AVC-Service-Launch-Report request |  |  | ServiceLaunchReportIndication |  |  |  |
|  | AVC-Service-Launch-Report indication |  |  | ServiceLaunchReportIndication |  |  |  |
|  | AVC-Service-Launch-Report confirm |  |  | - |  |  |  |
| Presenter | AVC-Session-Role-Assign request |  |  |  |  |  |  |
| Assignment | AVC-Session-Role-Assign indication |  |  |  |  |  |  |
|  | AVC-Session-Role-Assign confirm |  |  |  |  |  |  |
|  | AVC-Session-Role-Release indication |  |  |  |  |  |  |
|  | AVC-Session-Role-Release confirm |  |  |  |  |  |  |
|  | AVC-Session-Role-Please request |  |  |  |  |  |  |
|  | AVC-Session-Role-Please indication |  |  |  |  |  |  |
|  | AVC-Session-Role-Please confirm |  |  |  |  |  |  |
|  | AVC-Session-Role-Give request |  |  |  |  |  |  |
|  | AVC-Session-Role-Give indication |  |  |  |  |  |  |
|  | AVC-Session-Role-Give response |  |  |  |  |  |  |
|  | AVC-Session-Role-Give confirm |  |  |  |  |  |  |
|  | AVC-Session-Role-Inquire request |  |  |  |  |  |  |
|  | AVC-Session-Role-Inquire indication |  |  |  |  |  |  |
|  | AVC-Session-Role-Inquire response |  |  |  |  |  |  |
|  | AVC-Session-Role-Inquire confirm |  |  |  |  |  |  |
| Group | AVC-Group-Create request |  |  |  |  |  |  |
| Management | AVC-Group-Create indication |  |  |  |  |  |  |
|  | AVC-Group-Create confirm |  |  |  |  |  |  |
|  | AVC-Group-Disband request |  |  |  |  |  |  |
|  | AVC-Group-Disband indication |  |  |  |  |  |  |
|  | AVC-Group-Disband confirm |  |  |  |  |  |  |
|  | AVC-Group-Modify request |  |  |  |  |  |  |
|  | AVC-Group-Modify indication |  |  |  |  |  |  |
|  | AVC-Group-Modify confirm |  |  |  |  |  |  |
| Multipoint | AVC-Create-Multipoint-Channel request |  |  | CreateMultipointChannelRequest |  |  |  |
| Channel | AVC-Create-Multipoint-Channel indication |  |  | CreateMultipointChannelRequest |  |  |  |
| Management | AVC-Create-Multipoint-Channel response |  |  | CreateMultipointChannelResponse |  |  |  |
|  | AVC-Create-Multipoint-Channel confirm |  |  | CreateMultipointChannelResponse |  |  |  |
|  | AVC-Modify-Multipoint-Channel request |  |  | ModifyMultipointChannelRequest |  |  |  |
|  | AVC-Modify-Multipoint-Channel indication |  |  | ModifyMultipointChannelRequest |  |  |  |
|  | AVC-Modify-Multipoint-Channel response |  |  | ModifyMultipointChannelResponse |  |  |  |
|  | AVC-Modify-Multipoint-Channel confirm |  |  | ModifyMultipointChannelResponse |  |  |  |
|  | AVC-Disband-Multipoint-Channel request |  |  | DisbandMultipointChannelRequest |  |  |  |
|  | AVC-Disband-Multipoint-Channel indication |  |  | DisbandMultipointChannelRequest |  |  |  |
|  | AVC-Disband-Multipoint-Channel response |  |  | DisbandMultipointChannelResponse |  |  |  |
|  | AVC-Disband-Multipoint-Channel confirm |  |  | DisbandMultipointChannelResponse |  |  |  |
|  | AVC-Multipoint-Channel-Status indication |  |  | MultipointChannelStatusIndication |  |  |  |
| Source | AVC-Source-Identification request |  |  |  |  |  |  |
| Identification | AVC-Source-Identification indication |  |  |  |  |  |  |
|  | AVC-Source-Identification confirm |  |  |  |  |  |  |
| On-Air | AVC-On-Air-Indication request |  |  |  |  |  |  |
| Indication | AVC-On-Air-Indication indication |  |  |  |  |  |  |
|  | AVC-On-Air-Indication confirm |  |  |  |  |  |  |
| Floor | AVC-Floor-Ask request |  |  |  |  |  |  |
| Control | AVC-Floor-Ask indication |  |  |  |  |  |  |
|  | AVC-Floor-Ask response |  |  |  |  |  |  |
|  | AVC-Floor-Ask confirm |  |  |  |  |  |  |
|  | AVC-Floor-Grant request |  |  |  |  |  |  |
|  | AVC-Floor-Grant indication |  |  |  |  |  |  |
|  | AVC-Floor-Grant response |  |  |  |  |  |  |
|  | AVC-Floor-Grant confirm |  |  |  |  |  |  |
|  | AVC-Floor-Release request |  |  |  |  |  |  |
|  | AVC-Floor-Release indication |  |  |  |  |  |  |
|  | AVC-Floor-Release response |  |  |  |  |  |  |
|  | AVC-Floor-Release confirm |  |  |  |  |  |  |
|  | AVC-Floor-Assign request |  |  |  |  |  |  |
|  | AVC-Floor-Assign indication |  |  |  |  |  |  |
|  | AVC-Floor-Assign response |  |  |  |  |  |  |
|  | AVC-Floor-Assign confirm |  |  |  |  |  |  |
| Audio | AVC-Audio-Mix-Announce- request |  |  |  |  |  |  |
| Mixing | AVC-Audio-Mix-Announce indication |  |  |  |  |  |  |
|  | AVC-Audio-Mix-Set request |  |  |  |  |  |  |
|  | AVC-Audio-Mix-Set indication |  |  |  |  |  |  |
| Video | AVC-VSS-Switch-Configure request |  |  |  |  |  |  |
| Switching | AVC-VSS-Switch-Configure indication |  |  |  |  |  |  |
|  | AVC-VSS-Switch-Set Request |  |  |  |  |  |  |
|  | AVC-VSS-Switch-Set Indication |  |  |  |  |  |  |
| Privacy | AVC-Privacy-Notify request |  |  |  |  |  |  |
|  | AVC-Privacy-Notify indication |  |  |  |  |  |  |
|  | AVC-Privacy-Notify confirm |  |  |  |  |  |  |
| Source | AVC-Source-Select request |  |  | SourceSelectRequest |  |  |  |
| Selection | AVC-Source-Select indication |  |  | SourceSelectRequest |  |  |  |
|  | AVC-Source-Select response |  |  | SourceSelectResponse |  |  |  |
|  | AVC-Source-Select confirm |  |  | SourceSelectResponse |  |  |  |

## AVC Service Access

The AVC service access primitives are used to notify AVC Terminals of the range of available AVC services.  Some services may be designated as active and are available for use, other services are designated as inactive and must be explicitly activated by an authorised node.  In order to use an active service (other than those services designated as mandatory), a node must explicitly request to subscribe to that service.

AVC services are activated in the context of a session, hence the services active in one session may differ from those active in another session. Services may be restricted to the standard base session in some cases.  Nodes participating in a session may use any of the active services within that session, subject to the appropriate authorisation criteria being met.

### Description of Abstract Services

#### AVC-Service-Offer

The AVC-Service-Offer request primitive enables the Node Controller at an AVC Zone Manager to offer AVC Services to a node under its jurisdiction. This primitive may be issued at any time to notify a node of a change in the range of services on offer, or a change in the status of any service (i.e. activated or de-activated).

An AVC Zone Manager should only offer an AVC Terminal service for the media types it supports e.g. an audio-only terminal should not be offered a video switching service.

AVC Terminals may immediately use any service that is indicated as being active; they may not use services indicated as inactive.  Authorised nodes may request that such services be launched using the AVC-Service-Activate primitive.

FIGURE 25/T.132

AVC-Service-Offer - sequence of primitives

TABLE 35/T.132.

AVC-Service-Offer

| Content | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) |
| Requesting Node |  | M |  |
| Zone Manager |  | M |  |
| Destination Node | M |  |  |
| AVC Service List | O | O(=) |  |
| Result |  |  | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User ID of the requesting node.

Zone Manager: AVC User Id of AVC Network Element to which the destination node shall direct all requests for AVC Services.

Destination Node: AVC User Id of a node under the jurisdiction of the AVC Network Element issuing this primitive.

Active AVC Service List: A list of active AVC services.  This parameter identifies the sessions in which each service is operational.  For each AVC service, a flag indicates whether the node has been automatically subscribed to the service, or if it must explicitly subscribe to the service using the AVC-Service-Subscribe primitive.

Inactive AVC Service List: A list of AVC services (including optional features) that are to be offered to the destination node, but which are currently inactive.

Result:  Indicates whether the request was successful.  It contains one of the following results: successful, not Zone Manager, Invalid Destination Node Id, AVC management not active.

#### AVC-Service-Subscribe

The AVC-Service-Subscribe request primitive enables the Node Controller at an AVC Terminal to subscribe to an active AVC service which it has been offered by its Zone Manager in an AVC-Service-Offer indication.  It may also be used to allow an AVC Terminal to unsubscribe from an active service.  The request is forwarded by a node to its Zone Manager

FIGURE 26/T.132

AVC-Service-Subscribe - sequence of primitives

TABLE 36/T.132.

AVC-Service-Subscribe

| Content | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) |
| Requesting Node |  | M |  |
| AVC Service | M | M(=) |  |
| AVC Session ID | O | O(=) |  |
| Subscribe/Unsubscribe Flag | M | M(=) |  |
| Result |  |  | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User Id of  the requesting node.

AVC Service: The AVC Service which the requesting node wishes to subscribe to or unsubscribe from.

AVC Session ID: The AVC session in which the designated service is operating.  This parameter is omitted for the AVC Terminal Standard Base session.

Subscribe/Unsubscribe Flag: Indicates whether the requesting node wishes to subscribe to or unsubscribe from an active AVC service.

Result:  Indicates whether the request was accepted or rejected.  It contains one of the following results: successful, service not available, not authorised node, invalid session id.

#### AVC-Service-Subscribe-Report

The AVC-Service-Subscribe-Report indication primitive is used to inform Node Controllers at AVC Network Elements of a node subscribing to or unsubscribing from an active AVC service.  This primitive only applies to AVC services which utilise the AVC-Service-Subscribe primitive.  It does not apply to AVC services which subscribe nodes automatically upon joining an AVC Terminal Session.  Figure 26 shows the sequence of events leading to the use of this primitive.

TABLE 37/T.132.

AVC-Service-Subscribe-Report

| Content | Indication |
| --- | --- |
| Node ID | M |
| AVC Service | M |
| AVC Session ID | O |
| Subscribe/Unsubscribe Flag | M |

Node ID: AVC User ID of the node that is subscribing to or unsubscribing from an AVC service.

AVC Service: The AVC Service which the node is subscribing to or unsubscribing from.

AVC Session ID: The AVC session in which the designated service is operating.  This parameter is omitted for the AVC Terminal Standard Base session.

Subscribe/Unsubscribe Flag: Indicates whether the node is subscribing or unsubscribing.

#### AVC-Service-Activate

The AVC-Service-Activate request primitive enables the Node Controller at an AVC Terminal to request that an AVC service be activated or de-activated.  The request is forward to the Top AVC Provider, which arbitrates on all requests to activate/de-activate AVC services  This primitive may only be issued by an authorised node.

The AVC-Service-Offer primitive is used to notify the requester (and other nodes) when the service has been activated.

FIGURE 27/T.132

AVC-Service-Activate - sequence of primitives

TABLE 38/T.132.

AVC-Service-Activate

| Content | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) | M(=) |
| Requesting Node |  | M | M(=) |  |
| AVC Service | M | M(=) |  |  |
| AVC Session ID | O | O(=) |  |  |
| Activate/De-activate Flag | M | M(=) |  |  |
| Result |  |  | M | M(=) |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User Id of  the node requesting the launch of an AVC service.

AVC Service: The AVC Service that to which the requesting node wishes to activate or de-activate.

AVC Session ID: The AVC session in which the service is to be activated or de-activated.  This parameter is omitted for the AVC Terminal Standard Base session.

Activate/De-Activate Flag: Indicates whether the requesting node wishes to activate an inactive AVC service or de-activate an active AVC service.

Result:  Indicates whether the request was accepted or rejected.  It contains one of the following results: successful, service not available, not authorised node, invalid session id.

#### AVC-Service-Launch

The AVC-Service-Launch request primitive enables the Node Controller at  the Top AVC Provider to request that a particular  AVC service be activated.  The request may be broadcast to all AVC Network Elements which individually respond to the requesting node, indicating whether they have been able to activate or de-activate the service as indicated.  This approach may be used if all AVC Network Elements are required to participate in the provision of a service.  Alternatively, the request may be directed at a specific set of AVC Network Elements, if the Top AVC Provider has explicit knowledge of which AVC Network Elements are required to deliver a service.

This primitive may only be issued if the conference convenor has specified in the conference framework that services may be launched within the conference.

FIGURE 28/T.132

AVC-Service-Launch - sequence of primitives

TABLE 39/T.132.

AVC-Service-Launch

| Content | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) | M(=) |
| Destination Nodes | O |  |  |  |
| Responding Node |  |  | M | M(=) |
| AVC Service | M | M(=) |  |  |
| AVC Session ID | M | M(=) |  |  |
| Activate/Deactivate | M | M(=) |  |  |
| Result |  |  | M | M(=) |

Request Handle: Locally unique number to allow the requesting node to match responses.

Destination Nodes: List of Network Element AVC User IDs, or NULL, indicating that the request should be directed to all AVC Network Elements.

Responding Node: AVC User Id of the AVC Network Element responding to the service launch request.

AVC Service: The AVC Service that the  Top AVC Provider wishes to activate or de-activate.

AVC Session ID: The AVC session in which the service is to be activated or de-activated.  This parameter is omitted for the AVC Terminal Standard Base session.

Activate/Deactivate: Indicates whether the service should be activated or deactivated.

Result:  Indicates whether the request was successful.  It contains one of the following results: successful, service not available, launch failed, not Top AVC Provider, invalid session id.

#### AVC-Service-Launch-Report

The AVC-Service-Launch-Report request primitive enables the Node Controller at the Top AVC Provider to indicate that one or more  services have been successfully activated or de-activated.  This information may be broadcast to all AVC Network Elements, or directed to a specific late joining AVC Network Element.  AVC Terminals may be made aware of changes to active services through receipt of a revised AVC-Service-Offer indication from their respective AVC Zone Managers.

FIGURE 29/T.132

AVC-Service-Launch-Report - sequence of primitives

TABLE 40/T.132.

AVC-Service-Launch-Report

| Content | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M |  | M(=RQ) |
| Destination Nodes | O |  |  |
| AVC Service Table | M | M(=) |  |
| Full/Incremental Report Flag | M | M(=) |  |
| Result |  |  | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Destination Nodes: List of Network Element AVC User IDs, or NULL, indicating that the request should be directed to all AVC Network Elements.

AVC Service Table: A list of AVC services. For each service, the table indicates the sessions in which the service has been activated/de-activated.

Full/Incremental Report Flag: This parameter indicates whether the primitive contains a full list of active services, or an incremental list.

Result:  Indicates whether the request was successful.  It contains one of the following results: successful, not Top AVC Provider.

TABLE 41/T.132.

Content of AVC Service Table Entry

| Parameter | Description |
| --- | --- |
| AVC Service | Unique identifier of a specific AVC service. |
| AVC Session ID | The AVC session in which the specified service has been activated or de-activated. This parameter is omitted for the AVC Terminal Standard Base session. |
| Status | Activated or deactivated. |

## AVC Core Services

### AVC Session Roles

An AVC conference supports participation of both conventional nodes and anonymous nodes within an AVC conference, however only conventional nodes are permitted to take on conference session roles.

AVC defines a number of additional  participant roles which are used within AVC conference sessions, these roles supplement the conductor and convenor roles  defined in ITU recommendation T.124, (GCC), but unlike the GCC roles which are conference wide, these roles are only  valid within the context of an AVC session. All of the defined roles below make use of the AVC-session-Role primitives, using the role field to specify the specific role being addressed. By means of non standard identifiers, additional non-standard roles may also be defined.

The session role primitives may also be used to define groups of associated nodes, allowing participants to take on a group member  role within the session. The AVC services provide support for these groups, allowing a common behaviour or regime to be applied to a group. AVC defines a standardised group, termed a Panel. This group is created automatically when within a session, two or more presenter assignments are made. Further panels may also be explicitly created, allowing teams of presenters to be defined for a session.  The AVC conference framework provides the convenor with the ability to restrict the number of presenters and panels or even to prohibit the role or group  within the session.

The AVC conference framework is typically used to put bounds upon resource requirements for the  conference, it can also be used to impose a required usage model and management regime. For events where role holders are known in advance, the framework will allow not only the specific role and group to be predefined within a session but also allow access keys to be associated with given roles. These key are passed out of band to the defined role-holder. When a participant  with a pre-defined role and an access key  joins a session and makes a role assignment request, it  is validated by the top provider against the framework and the role is assigned if the key is valid.

Role pre-assignments and access keys  when used in conjunction with the framework can automate the establishment of potentially complex conference scenario’s and minimise the complexity for the participant.

The node controller at each node will receive session role indications. If the “Synchronise with data conference” flag is set then the node controller should also pass these indications on to the application protocols.

#### Conference Presenter(s)

AVC provides a method for allowing a node to take on a  a presenter role in an AVC session. Each AVC session can support multiple presenters, and automatically forms multiple presenters into a standardised group termed a Panel. It is also possible to specify  additional panels of presenters. The Conference framework allows the limits on the number of presenters and panels to be specified by the convenor, it also allow access keys to be specified for specific roles allowing participants with pre-allocated roles to use the access keys as a means to take on specific roles. The framework will even allow ordering to be applied to a group for example allowing a pre-planned program of presentation to be specified.

#### Presenter Assignment

The node controller at a node may acquire or release a role at will unless a conference framework either prohibits or restricts access to that role. When any node successfully acquires the Presenter token it takes on a   presenter role. The Node Controller at all nodes in the conference are made aware of this by means of a AVC-Session-Role-Assign indication.  This indication also indicates which node has become a presenter. AVC services respond to provide the designated node with  access to conference resources that it needs to present to that session.

#### Releasing the Presenter Role

An Active presenter may relinquish a  presenter role when the presenting task is complete. Where there is no defined sequence or the sequence is complete an AVC-Session-Role-Release request will return the conference from presenter mode to previous or default state.

The Node Controller at all nodes in the conference are made aware when a there is a change in presenter status.

#### Passing the Presenter Role

Where no pre-assignments are made, a  node which is currently the an active presenter for a session may pass presentership  directly to another node without changing  conference mode during the transition, by issuing an AVC-Session-Role-Give request specifying the desired recipient node.  If the recipient accepts presentership, the Node Controller at all nodes in the conference are made aware of this transition by means of a AVC-Session-Role-Assign indication which indicates which node is the new presenter.  If the AVC session has a pre-defined sequence of presenters  then the role will be passed by the top provider to the next in the sequence when the current role holder relinquishes it. Asking For a Presenter Role

A node may explicitly request from the presenter that presentership be given to it by issuing a AVC-Session-Role-Please request primitive.  The current presenter may choose to give the requesting node presentership, or may ignore the request.

#### Asking about the Presenter Role

The Node Controller as well as Application Protocol Entities at any node may also inquire as to which nodes, if any, currently holds a  Presenter role by using the AVC-Session-Role-Inquire request primitive.

When an AVC Service is made aware that the session in which it is participating in has an active presenter, it shall immediately begin operating according to its Presenter behaviour as prescribed by the specification of the corresponding AVC Service.

#### The Chair Role

The Chair role  operates within the context of  the  AVC conference session in which that role is assigned.  A chair role may be pre-assigned by a convenor as part of the framework, the convenor  may also assign a chair access key for the session. A particular participant who has been pre-assigned a role such as the chair can  be provided with the access key  (by some out of band means)  corresponding to the access key for that role in the framework. When the pre-defined chairman joins  the role assignment request is passed together with any access key to the AVC Top-Provider. The AVC Top Provider will validate the role request and any  associated access key against the framework, if the role assignment request is valid, the requester  will be assigned the requested role.  If the chair role is not prohibited by the current conference framework, then conventional nodes may request and compete for the role as required and if successful act as chair for the session. If the chair is defined in the framework with an access key only the requester who provides the correct access key is assigned the role. The framework does not prohibit the role being asked for by participants or being given by the chair role holder.

An active chairman automatically becomes the arbiter for the floor control service, and consequently the floor control service requires a chair  in order to operate.

#### Chair Assignment

In a session without a chairperson the Node Controller at any node has the possibility to issue a request to become the Chairperson, by issuing a AVC-Session-Role-Assign request primitive with the identifier set to chair. The receipt of the associated AVC-Session-Role-Assign confirm primitive indicates whether the requester has become the Chair or not, depending on the result parameter within the primitive. A successful AVC-Session-Role-Assign request shall be accompanied by AVC-Session-Role-Assign indications to all Node Controllers  at all nodes in the session, signalling that the session has a Chair, and giving information about the identity of the Chair.  Table 42 shows the parameters and types of this primitive.  Figure 30 shows the sequence of events when using this primitive.

#### Releasing the Chair Role

In order to change its status from Chair to normal participant, the Node Controller at the Chair node issues the AVC-Session-Role-Release request primitive. As soon as the Chair has requested the release of Chair, all Node Controllers in the session are informed of the change in operational mode by means of the AVC-Session-Role-Release indication. The order of AVC-Session-Role-Assign and AVC-Session-Role-Release indications represents the actual order of Chair  transitions.  Any participant issuing a AVC-Session-Role-Release, but not being the current Chair, shall be answered by AVC with a AVC-Session-Role-Release confirmation containing a negative result and reason description. In this situation no indications shall be sent to the other participants.  Apart from this user initiated transition to non-conducted mode, the release of Chair rôle can also be initiated by AVC itself, for instance because the Chair disconnected from the conference Table 43 shows the parameters and types of this primitive.  The sequences of primitives belonging to both situations are depicted in figures 31 and 32 respectively.

#### Asking for the Chair Role

The primitive AVC-Session-Role-Please request may be issued by a Node Controller to ask the current Chair to give Chair to the requesting node.  The AVC-Session-Role-Please indication is forwarded to the current conducting node.  The current Chair may then choose to give Chair to the requester by using the AVC-Session-Role-Give primitive.  The confirm portion of this primitive is only local confirmation that the request was accepted by the local AVC Provider.  No confirmation from the Chair is given directly.  Table 44 shows the parameters and types of this primitive.  Figure 33 shows the sequence of events when using this primitive.

#### Giving the Chair Role

The primitive AVC-Session-Role-Give request may be issued by a Node Controller to transfer Chair to a specific node.  If the Chair is not accepted by the intended recipient, conference Chair continues to be held by the original Chair.  Table 45 shows the parameters and types of this primitive.  Figure 34 shows the sequence of events when using this primitive.

#### Asking about the Chairperson

The primitive AVC-Session-Role-Inquire request may be issued at any time by either a Node Controller or Application Protocol Entity in order to find out whether the conference is conducted or not, and if so, which node is the Chair, and if the requesting node has been granted conducted-mode permission.  Table 46 shows the parameters and types of this primitive.  Figure 35 shows the sequence of events when using this primitive.

### Description of Abstract Services

The following is a list of the primitives defined in this section and a brief summary of the function of each:

- AVC-Session-Role-Assign – Used by a Node Controller to request assignment  of a session role .  When a node takes on a role e.g.  presenter, the indication form of this primitive is used to announce a successful bid to all Node Controllers in the conference at all nodes in the conference.

- AVC-Session-Role-Release – Used by a Node Controller to release the specified role in  a session.  When any node releases a role, the indication form of this primitive is used to announce this to all Node Controllers in the conference at all nodes in the session.

- AVC-Session-Role-Please – Used by a Node Controller to request that a role be given to it from the current session role holder.

- AVC-Session-Role-Give – Used by a Node Controller to pass a role to a specified node.

- AVC-Session-Role-Inquire – Used by a Node Controller  to determine whether the session currently has a specific  active role  and if it does what the role holder’s AVC User ID is, or if no role is specified then a list of roles present in the conference is returned.

#### AVC-Session-Role-Assign

During non-conducted mode the Node Controller at any node has the possibility to issue a request to take on a role, by issuing a AVC-Session-Role-Assign request primitive. The receipt of a AVC-Session-Role-Assign confirm primitive indicates whether the requester has been assigned the role or not, depending on the result parameter within the primitive. A successful AVC-Session-Role-Assign request shall be accompanied by AVC-Session-Role-Assign indications to all Node Controllers in the conference at all nodes in the conference signalling that the conference has a new role assignment, and giving information about the identity of the role holder.  Table 42 shows the parameters and types of this primitive.  Figure 30 shows the sequence of events when using this primitive.

Note – If the Presenter disconnects from the conference for any reason, the conference either moves on to next presenter in sequence, if there is one assigned, or it reverts to default mode for the conference model in use. If a chairman leaves the session then the session reverts to default model for the session.

TABLE 42/T.132.

AVC-Session-Role-Assign - types of primitives and their parameters

| Parameter | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| AVC User ID | M | M= | M= | M= |
| AVC Session Identifier | C | C= | M | M= |
| AVC Session Role Identifier | C | C= | M | M= |
| AVC Role Access Key | O | O= | O= | O= |
| Group Identifier | O | O= | O= | O= |
| Group Purpose Descriptor |  |  |  |  |
| Group number | O | O= | O= | O= |
| Result |  |  | C | C= |

AVC User ID:  Identification number of the requesting terminal.

AVC  Session Identifier: The identifier for the AVC -session it is mandatory to provide the session ID in the request unless an access key is provided in which case the Top-provider can resolve the correct session and the parameter  is optional.

AVC Session Role Identifier: This parameter identifies the specific session role “Chairman, Presenter, Group Member” It is mandatory to provide the session role in the request unless an access key is provided in which case the Top-provider can resolve the corresponding role, and the parameter  is optional.

Group Identifier:  The group identifier is an optional parameter it allows a group to be defined by participants, This parameter  is ignored in cases where it is not applicable e.g.  if the role is chair - as only one chair is permitted in a session. In the case of the presenter role the default presenter group Panel {0} will be automatically assumed if no group is specified.

Group Purpose Descriptor: This is an optional descriptive string that can be used to describe the purpose of the grouping.

Group Number:	This parameter allows multiple groups of the same type to be defined, its standard use would be to allow multiple panels of presenters to be defined, the default group of presenters is panel{0}  groups may be defined {1..7}

AVC Role Access Key:  This parameter provides an access key which the requester may optionally provide in a request, if provided it is used by the AVC Top Provider to validate the request agianst the conference framework access controls.

Result:  An indication of whether the request was accepted or rejected, and if rejected, the reason why.  It contains one of a list of possible results:  successful, invalid conference, non-conductible conference, token already owned.

FIGURE 30/T.132

AVC-Session-Role-Assign - Sequence of primitives

Indication goes to Top (on Top provider channel) only for validation against framework response needed for validation response. Confirmation goes to requester and succesful assignment results in indications going to all nodes in session.

#### AVC-Session-Role-Release

In order to change its status from presenter to normal participant, the Node Controller at the presenter node issues the AVC-Session-Role-Release request primitive. As soon as the presenter has requested the release of presentership, all Node Controllers in the are informed of the change in operational mode by means of the AVC-Session-Role-Release indication. The order of AVC-Session-Role-Assign and AVC-Session-Role-Release indications represents the actual order of presentership transitions.  Any participant issuing a AVC-Session-Role-Release, but not being the current presenter, shall be answered by AVC with a AVC-Session-Role-Release confirmation containing a negative result and reason description. In this situation no indications shall be sent to the other participants.  Apart from this user initiated transition to non-conducted mode, the release of presentership can also be initiated by AVC itself, for instance because the presenter disconnected from the conference.  Table 43 shows the parameters and types of this primitive.  The sequences of primitives belonging to both situations are depicted in Figures 31 and 32 respectively.

TABLE 43/T.132.

AVC-Session-Role-Release - types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| AVC User ID |  | M | M= |
| AVC Session Identifier |  | M | M= |
| AVC Session Role Identifier |  | M | M= |
| Group Identifier |  | O | O= |
| Group Number |  | O | O= |
| Release Reason |  | C |  |
| Result |  |  | M |

AVC User ID:  Identification number of the requesting terminal.

AVC  Session Identifier: The identifier for the AVC sub-session

AVC Session Role Identifier: This parameter identifies the specific session role if left blank the response may contain a list of active session roles “Chairman, Presenter ”

Group Identifier:  The group identifier is an optional parameter it allows a group to be defined by participants, This parameter  is ignored in cases where it is not applicable e.g.  if the role is chair - as only one chair is permitted in a session. In the case of the presenter role the default presenter group Panel {0} will be automatically assumed if no group is specified.

Group Number:	This parameter allows ae groups of the role type to be defined, its standard use would be to select a  panels of presenters where more than one had been defined, the default group of presenters is panel{0}  groups may be defined {0..7}

Release Reason:  The reason parameter can provide additional information within indication messages concerning the release, e.g. presenter initiated, or system initiated.

Accept/Reject:  This flag indicates whether the AVC-Session-Role-Release request was accepted or rejected.

Result:  An indication of whether the request was accepted or rejected, and if rejected, the reason why.  It contains one of a list of possible results:  successful, invalid conference, did not own token.

FIGURE 31/T.132

AVC-Session-Role-Release (User Initiated) - Sequence of primitives

FIGURE 32/T.132

AVC-Session-Role-Release (AVC Initiated) - Sequence of primitives

#### AVC-Session-Role-Please

The primitive AVC-Session-Role-Please request may be issued by a Node Controller to ask the current role holder  to give the role  to the requesting node.  The AVC-Session-Role-Please indication is forwarded to the current conducting node.  The current role holder may then choose to give presenter-ship to the requester by using the AVC-Session-Role-Give primitive.  The confirm portion of this primitive is only local confirmation that the request was accepted by the local AVC Provider.  No confirmation from the role holder  is given directly.  Table 44 shows the parameters and types of this primitive.  Figure 44 shows the sequence of events when using this primitive.

TABLE 44/T.132.

AVC-Session-Role-Please - types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| AVC User ID | M | M= |  |
| AVC Session Identifier | M | M= |  |
| User ID of Requester | M | M= |  |
| AVC Session Role Identifier | M | M= |  |
| AVC Group Identifier | O | O= |  |
| AVC Group Number | O | O= |  |
| Result |  |  | M |

AVC  User ID:  Identification number of the requesting terminal.

AVC Session Identifier:

User ID of Requester:  The GCC User ID of the requesting node.

AVC Session Role Identifier: Chair, presenter etc

Group Identifier:  The group identifier is an optional parameter it allows a group to be defined by participants, This parameter  is ignored in cases where it is not applicable e.g.  if the role is chair - as only one chair is permitted in a session. In the case of the presenter role the default presenter group Panel {0} will be automatically assumed if no group is specified.

Group Number:	This parameter allows a specific group of the given role type  type to be selected, its standard use would be to allow selection of a particular  panel of presenters.  The parameter may have the value  {0..7}

Result:  An indication of whether the request was accepted or rejected, and if rejected, the reason why.  It contains one of a list of possible results:  successful, invalid conference, not in conducted mode.

FIGURE 33/T.132

AVC-Session-Role-Please - Sequence of primitives

#### AVC-Session-Role-Give

The primitive AVC-Session-Role-Give request may be issued by a Node Controller to transfer presentership to a specific node.  If the presentership is not accepted by the intended recipient, conference presentership continues to be held by the original presenter.  Table 45 shows the parameters and types of this primitive.  Figure 34 shows the sequence of events when using this primitive.

Note – it is possible to use this primitive (in conjunction with others) to ensure that a particular node becomes the conference presenter with no opportunity for another node to acquire presentership.  The procedure would be for the conference convenor first to create a conference which is locked and to then acquire presentership using the AVC-Session-Role-Assign primitive.  Once the convenor has become the conference presenter, it may then allow other nodes into the conference either by unlocking it using AVC-Conference-Unlock and/or by adding the other nodes directly.  Once the intended conducting node has joined the conference, presentership may be passed directly to that node by use of the AVC-Session-Role-Give primitive.

TABLE 45/T.132.

AVC-Session-Role-Give - types of primitives and their parameters

| Parameter | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| AVC User ID | M | M |  |  |
| Recipient Node (GCC User ID) | M |  |  | M(=) |
| AVC Session Identifier | M | M | M(=IN) | M(=RQ) |
| AVC Session Role Identifier | M | M(=) | M(=) | M(=) |
| Group Identifier | O | O= | O= | O= |
| Group Number | O | O= | O= | O= |
| Result |  |  | M | M(=) |

AVC User ID:  Identification number of the role holder’s terminal.

Recipient Node:  The GCC User ID of the node to which presentership is being transferred.

Group Identifier:  The group identifier is an optional parameter it allows a group to be defined by participants, This parameter  is ignored in cases where it is not applicable e.g.  if the role is chair - as only one chair is permitted in a session. In the case of the presenter role the default presenter group Panel {0} will be automatically assumed if no group is specified.

Group Number:	This parameter allows a specific group of the given role type  type to be selected, its standard use would be to allow selection of a particular  panel of presenters.  The parameter may have the value  {0..7}

Result:  An indication of whether the request was accepted or rejected, and if rejected, the reason why.  It contains one of a list of possible results:  successful, invalid conference, not presenter, give not accepted.

FIGURE 34/T.132

AVC-Session-Role-Give - Sequence of primitives

#### AVC-Session-Role-Inquire

The primitive AVC-Session-Role-Inquire request may be issued at any time by either a Node Controller or Application Protocol Entity in order to find out if a specific role is active or not and if so, which node is the role holder. Table 46 shows the parameters and types of this primitive.  Figure 35 shows the sequence of events when using this primitive.

TABLE 46/T.132.

AVC-Session-Role-Inquire - types of primitives and their parameters

| Parameter | Request | Confirm |
| --- | --- | --- |
| AVC User ID | M | M= |
| AVC Session Identifier | M | M |
| AVC Session Role Identifier | M | M |
| Session Role Node (GCC User ID) |  | C |
| Permission flag |  | C |
| Group Identifier: | O | O= |
| Group Number: | O | O= |
| Result |  | M |

AVC User ID:  Identification number of the requesting terminal.

AVC Session identifier

Presenter Node (GCC User ID):  The GCC User ID of the node that is currently presenter.  Not present if currently in non-conducted mode.

Permission Flag:  If in conducted mode, this flag indicates whether or not the local node has been granted conducted-mode permission.

Group Identifier:  The group identifier is an optional parameter it allows a group to be defined by participants, This parameter  is ignored in cases where it is not applicable e.g.  if the role is chair - as only one chair is permitted in a session. In the case of the presenter role the default presenter group Panel {0} will be automatically assumed if no group is specified.

Group Number:	This parameter allows a specific group of the given role type  to be selected, its standard use would be to allow selection of a particular  panel of presenters.  The parameter may have the value  {0..7}

Result:  An indication of whether the request was accepted or rejected, and if rejected, the reason why.  It contains one of a list of possible results:  successful, invalid conference.

FIGURE 35/T.132

AVC-Session-Role-Inquire - Sequence of primitives

### Floor Control

The AVC floor control service is provided to allow participants to be able to request use of conference services. The specific meaning of requesting the floor is dependant upon the supplementary services in use within a particular conference. By default requesting floor means requesting to have your audio included in the conference audio mix.  Floor control can only be used only in a chaired session, and the chairman is responsible for granting requests. Therefore floor control can only operate when the session has an active chairperson. When a chair is assigned AVC-Chair-Assign indications are sent to the Node Controller’s  at all nodes in the session, signalling that the session has a Chair, and giving information about the identity of the Chair. When the session chair is assigned the floor control capability becomes available and the chair can launch the service when needed.

Once the service becomes active all audio sources except the chair’s are muted at the mixer by the service using the Set-Audio-Mixer primitives. Session participants can issue floor requests which are directed to the chair who can grant or decline the requests. Participants can release the floor when they have completed their requirements.

This service provides the mechanism that allow conference participants to make requests to speak in a chaired conference and for the chair to enable and disable participants from speaking.

The Audio Mixing service is aware of and utilises 3 conference rôles that nodes can adopt in operating the service:-

- Chair  Rôle:-When a terminal is assigned the chair role it is assigned the chair token. If the floor control has been launched the chair becomes the arbiter for floor requests, if the service has not been launched the chair may launch it, as may a convenor or convenor privileged node.

- Participant Role:-  In respect to the flor control service, participants may be either anonymous or conventional nodes. All participant nodes will initially be muted when floor control is launched within a chaired session. Individuals or group may request and be assigned the floor by the chair. Assignment of the floor will have the effect of having their audio stream included in the audio mix.

- is running the AVC protocol and its local GCC pr indication that the conference has become conducted, its transmitted audio will be muted at its local mixer. The terminal must provide its user with the ability to issue "floor requests" (requests for permission to speak).

- Conductor Rôle:-	By means of the framework or the alignment parameter in the floor control service the conference conductor can take on the role  of AVC session chair.

- NE Node Rôle:-		All NE nodes must have joined the NE communications channel and as a consequence will receive Mixer Control  messages sent on the NE channel which they must act upon.

It is not mandatory that all participating nodes be capable of supporting the service in order for it to be used. The conductor may decide to launch the floor control service regardless, this will still result in the desired control, however;  nodes not supporting the service will not be able to make floor requests in the conventional way.

When the floor control service is invoked by issuing a Activate-Service  primitive, the initiating chair or the AVC Conference framework specifies if the service is to be used for audio only or for both audio and video together.

For audio control, the initial condition is with all audio sources  muted at the mixer except that of the chair, video will continue in the current operational mode. Nodes other than the chair wishing to speak make a Floor-Request to the chair. The chair may decide to grant or refuse the request. Normally requests will  either be granted by the chair or held in a queue in received order until the chair is ready to add a new speaker. The chair may also  effectively remove any speaker by muting their audio source at the mixer.

For Video control the audio can be set to track the video regardless of previous modes in operation.

The conductor initiates the control by communicating with mixer(s) via the MCU communication channel. In voice switched mode the action of the video is linked to the output of the audio mixer and is therefore indirectly controlled as well.

#### AVC-Request-Floor

The AVC-Request-Floor primitive is issued  by the node controller of any node within a conference session which wishes to have its audio/video distributed to the session.

Table  47 shows the parameters and types of this primitive.  Figure 36 shows the sequence of events when using this primitive.

TABLE 47/T.132.

AVC-Request-Floor - Types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| AVC User ID | M | M |  |
| AVC Session Identifier | M | M |  |
| Group Floor Req | O | O |  |
| Associated Service | O | O |  |
| Result |  |  | M |

AVC User ID:  Identification number of the requesting terminal.

AVC  Session Identifier: The identifier for the AVC session

Associated Service: Audio Mixing assumed by default - if a parameter is provided it overrides the default.

Group Floor request: Used if the requester is making a request on behalf of the specified group to which it belongs

Result:  An indication of whether the request was accepted or rejected, and if rejected, the reason why.  It contains one of a list of possible results:  successful, invalid conference, non-conductible conference, token already owned.

FIGURE 36/T.132

AVC-Request-Floor - Sequence of primitives

#### AVC-Release-Floor

This primitive  issued by a node that has requested the floor, either to cancel a pending request or to request being to be removed from current floor activity.

Table 48 the parameters and types of this primitive.  The sequences of primitives belonging to both situations are depicted in Figure 37.

TABLE 48/T.132.

AVC-Release-Floor - Types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Release Reason |  | C |  |
| AVC Session Identifier |  | M |  |
| Group Floor Req | O | O |  |
| Associated Service | O | O |  |
| Result |  |  | M |

Conference ID:  Identifier of the conference to which the primitive refers.

Release Reason:  The reason parameter can provide additional information within indication messages concerning the release, e.g. Chair initiated, or system initiated.

Accept/Reject:  This flag indicates whether the AVC-Chair-Release request was accepted or rejected.

Associated Service: Audio Mixing assumed by default - if a parameter is provided it overides the default.

Group Floor reqest: Used if the requester is making a request on behalf of the specified group to which it belongs

Result:  An indication of whether the request was accepted or rejected, and if rejected, the reason why.  It contains one of a list of possible results:  successful, invalid conference, did not own token.

FIGURE 37/T.132

AVC-Release-Floor - Sequence of primitives

#### AVC-Floor-Grant

The AVC-Floor-Grant primitive is issued  by the node controller a session chairing node in response to a previous floor request. The chairing node may queue floor requests and the chair’s node controller should maintain an ordered list of requests.

Table 49 shows the parameters and types of this primitive.  Figure 38 shows the sequence of events when using this primitive.

TABLE 49/T.132.

AVC-Floor-Grant - Types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| AVC User ID | M | M= | M= |
| AVC Session Identifier | M | M= | M= |
| Group Floor Req | O | O= | O= |
| Associated Service | O | O= | O= |
| Access Capability | O | O= | O= |

AVC User ID:  Identification number of the requesting terminal.

AVC  Session Identifier: The identifier for the AVC session

Group Floor request: Used if the requester is making a request on behalf of the specified group to which it belongs

AVC Access Capability : list of functionality to which  access has been granted.

FIGURE 38/T.132

AVC-Floor-Grant - Sequence of primitives

### Multipoint Channel Management

The Multipoint Channel Management service enables users to establish, modify and tear down logical communication paths for the distribution of real time media within a conference.  The service allows for multipoint (single source, multiple sinks) paths to be established and has provision for QoS support.  Mismatches between encoding formats supported by different endpoints may be resolved by activation of transcoders, as appropriate.

#### AVC-Create-Multipoint-Channel

The AVC-Create-Multipoint-Channel request primitive is issued by the Node Controller at an AVC Terminal to create a multipoint communication path from a source node to one or more destination nodes.  The request is directed to the node’s Zone Manager, which is responsible for establishing the communication channel, in collaboration with other AVC Zone Managers.

FIGURE 39/T.132

AVC-Create-Multipoint-Channel - sequence of primitives

TABLE 50/T.132.

AVC-Create-Multipoint-Channel

| Content | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) | M(=) |
| Requesting Node |  | M | M(=) |  |
| Source Node | O | O(=) |  |  |
| Destination Node List | M | M(=) |  |  |
| Media Type | M | M(=) |  |  |
| Session Identifier | O | O(=) |  |  |
| Modification Rights | O | O(=) |  |  |
| Multipoint Channel Identifier |  |  | O | O(=) |
| QoS | O | O(=) | O | O(=) |
| Synchronised Multipoint Channels | O | O(=) |  |  |
| QoS Level |  |  | O | O(=) |
| Result |  |  | M | M(=) |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User ID of the requesting node.

Source Node:  AVC User Id of the node from which the multipoint channel is required to originate.  This parameter is omitted if a multicast channel is required.

Destination Node List:  A list of AVC User Ids corresponding to the nodes that are required to act as sinks on the multipoint channel.

Media Type: Indicates the type of media to be conveyed within the multipoint channel (i.e. audio, video, or real time data) and, optionally, the format of the media.

Session Identifier:  An optional parameter identifying the AVC Terminal Session in which the multipoint channel is to be used.

Modification Rights:  This parameter specifies the scope of AVC Terminals allowed to make modifications to or disband the multipoint channel.  The value of this parameter may be either Owner, Session, or Public.  The Owner setting specifies that only the owner may modify or disband the channel.  The Session setting specifies that any node which is part of the same AVC Terminal Session as the owner may modify or disband the channel.  The Public setting specifies that any AVC node enrolled in the conference may modify or disband the channel.

Multipoint Channel Id: A globally unique number identifying the multipoint channel. (Editor’s note: we need to specify how this is assigned)

QoS:  Desired Quality of Service is included in the request.  The actual QoS is provided in the confirm.

Synchronised Multipoint Channels: A list of multipoint channels with which the new multipoint channel is to be synchronised, and specifies the maximum relative latency between the channels.  (This parameter should specify a descriptor to apply meaning to the association e.g. two audio logical channels used to distribute a stereo signal, a video logical channel & its associated audio logical channels(s).)  Issue: association should probably be between paths and not logical channels.  Keep here pending a decision.

QoS Level:  An optional parameter indicating the level of service available on the channel.  It contains one of the following values - guaranteed  service, predictive service, best effort service.

Result:  Indicates whether the multipoint channel was created.  It contains one of the following results: successful, invalid source node, invalid destination node(s), unspecified failure, not authorised node.  (Editor’s note: do we need to explicitly identify errors such as attempting to route video to audio only nodes?)

#### AVC-Modify-Multipoint-Channel

The AVC-Modify-Multipoint-Channel request primitive is issued by the Node Controller at an AVC Terminal to modify the characteristics of an existing multipoint channel.  This includes adding or removing destination nodes and modifying the QoS of the channel.  The request is directed to the node’s Zone Manager, which is responsible for modifying the communication channel, in collaboration with other AVC Zone Managers.

FIGURE 40/T.132

AVC-Modify-Multipoint-Channel - sequence of primitives

TABLE 51/T.132.

AVC-Modify-Multipoint-Channel

| Content | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) | M(=) |
| Requesting Node |  | M | M(=) |  |
| Multipoint Channel Identifier | M | M(=) |  |  |
| Nodes to be Added | O | O(=) |  |  |
| Nodes to be Removed | O | O(=) |  |  |
| Session Identifier | O | O(=) |  |  |
| Modification Rights | O | O(=) | O | O(=) |
| QoS | O | O(=) | O | O(=) |
| Result |  |  | M | M(=) |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User ID of the requesting node.

Multipoint Channel Id: A globally unique number identifying the multipoint channel to be modified.

Nodes to be Added: A list of AVC User Ids corresponding to destination nodes that are to be added to the multipoint channel.

Nodes to be Removed: A list of AVC User Ids corresponding to destination nodes that are to be removed from the multipoint channel.

Session Identifier:  An optional parameter identifying the AVC Terminal Session in which the multipoint channel is to be used.

Modification Rights:  This optional parameter specifies the scope of AVC Terminals allowed to make modifications to or disband the multipoint channel.  The value of this parameter may be either Owner, Session, or Public.  The Owner setting specifies that only the owner may modify or disband the channel.  The Session setting specifies that any node which is part of the same AVC Terminal Session as the owner may modify or disband the channel.  The Public setting specifies that any AVC node enrolled in the conference may modify or disband the channel.  Only the owner of this multicast channel can change the modification rights.  If a non-owner attempts to change the modification rights, this change will not take place, however any modification to the multicast channel characteristics will be attempted as long as the requester has modification rights.

QoS:  This parameter is included in the request if a change in Quality of Service is required.  The actual QoS is provided in the confirm.

QoS Level:  An optional parameter indicating the level of service available on the channel.  It contains one of the following values - guaranteed  service, predictive service, best effort service.

Result:  Indicates whether the multipoint channel was modified.  It contains one of the following results: successful, unspecified failure, invalid multipoint channel id, invalid nodes to add list, invalid nodes to remove list, not authorised node.

#### AVC-Disband-Multipoint-Channel

The AVC-Disband-Multipoint-Channel request primitive is issued by the Node Controller at an AVC Terminal to disband an existing multipoint channel.  The request is directed to the node’s Zone Manager, which is responsible for disbanding the communication channel, in collaboration with other AVC Zone Managers.

FIGURE 41/T.132

AVC-Disband-Multipoint-Channel - sequence of primitives

TABLE 52/T.132.

AVC-Disband-Multipoint-Channel

| Content | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) | M(=) |
| Requesting Node |  | M | M(=) |  |
| Multipoint Channel Identifier | M | M(=) |  |  |
| Result |  |  | M | M(=) |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User ID of the requesting node.

Multipoint Channel Id: A globally unique number identifying the multipoint channel to be disbanded.

Result:  Indicates whether the multipoint channel was disbanded.  It contains one of the following results: successful, unspecified failure, invalid multipoint channel id, not authorised node.

#### AVC-Multipoint-Channel-Status

The AVC-Multipoint-Channel-Status indication primitive is generated by an AVC Provider following the creation of a new multipoint channel, or modification to or disbanding of an existing multipoint channel.  It is used to notify the members of a multipoint channel of the current status of the channel and is thus only issued at nodes which are members of the multipoint channel and nodes that are being removed from the multipoint channel.  Figures 39, 40 and 41 show the sequence of events leading to the generation of this primitive.

TABLE 53/T.132.

AVC-Multipoint-Channel-Status

| Content | Indication |
| --- | --- |
| Multipoint Channel Id | M |
| Channel Status | M |
| Source Node | O |
| Destination Nodes Added | O |
| Destination Nodes Removed | O |
| Session Identifier | O |

Multipoint Channel Id: A globally unique number identifying the multipoint channel.

Channel Status: This parameter indicates whether the multipoint channel is new, modified or disbanded.

Source Node: AVC User ID of the multipoint channel source node.

Destination Nodes Added: List of AVC User Ids corresponding to destination nodes added to the multipoint channel.

Destination Nodes Removed: List of AVC User Ids corresponding to destination nodes removed from the multipoint channel.

Session Identifier:  An optional parameter identifying the AVC Terminal Session in which the multipoint channel is to be used.

### Source Identification

This service exploits the T.120 capabilities to offer an enhanced functionality to a conference. It provides the capability for the origin and content of a video stream to be communicated.

Each node in the conference is uniquely identified by its GCC's User Id (or represented by a proxy service). If a node is only capable of originating a single video stream then its User Id is all that will be needed to uniquely identify it as the originating node for that stream. Where a node is capable of originating more than one stream of a particular media then this recommendation supports the unique identification of streams. The AVC database provides the means to map streams to Multipoint Channels. Streams may be identified by tags  embedded within the streams, or the identification may be implicit; based upon  some logical ordering.

In a multipoint conference, when an AVC aware node provides a  source  to a channel, it is required to send a source identification message to the NE Communication channel. If no identifier is received by the  NE(s), they  will assume the default  audio and/or video streams and extract a name if available from the GCC Conference Roster entry for that site. The identification message will allow the AVC Multipoint Channel, the stream source   and a string identifier to be defined to replace the default determination.

An AVC identification message can be associated with each  received stream  by an MCU or other network element. It is the MCU's responsibility to relayed the identification onward to correspond to the routing applied to the  stream. If the MCU carries out any manipulation of a stream it must generate a new AVC Identification message to correlate with the changes.

The information will be used to identify; to the receiver, who or what is being sent and also from what location the stream originates. For a video stream it may be used to generate a caption for displaying with the image.

TABLE 54/T.132.

AVC-Source- Identification  Primitive

|  | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Source AVC User ID | M | M= | O= |
| Source ID | O | O= | O= |
| AVC Multipoint Channel number (n) | M | M= | M= |
| Media Type | M | M= | M= |
| Stream Descriptor | O | O= |  |
| Reason Code | O | O= |  |
| Status |  |  | O= |

Source AVC User ID - Node identifier

Source ID - The device identifier as defined( as advertised in the application roster).

AVC Multipoint Channel Number -  1- n

Media Type - Audio/Video/Other

Stream Descriptor -

Reason Code - This code tell receiving site why image has changed

00 - Mode change

01 - Stream control assigned to node

Status -

FIGURE 42/T.132

AVC-Source-Service-Identification  - Sequence of primitives

Terminal Action.

In order for a  terminal to join the  Identification service it must send out its own video or audio identification message to the AVC Broadcast channel. Once this has been received it will  be included in the service. When ever the source it provides to the conference changes it must send a new identification with the source device and identifier string. If the optional fields are not completed the service will assume the source is the default video and/or audi streams and will use the user name string provided by the terminals 'GCC_Announce_Presence' message.

MCU Action

Each MCU must be able to associate each stream it receives with a stream  identifier. When ever an MCU provides a new video source to any of its out going ports, it must relay on the associated identifier(s). Each MCU needs to know the User ID of the terminals or NEs directly connected to its ports. If a connection is to another NE it must also know the user ID of all other terminals and ports that can be reach via that MCU in order than it can route the messages  using there User Ids.

### On-Air Indication Service

In a multipoint conference one or more Network Elements (MCUs  or routers) provide control and routing of audio and video streams. When a stream is routed onwards by an NE  to a terminal  capable of  interpreting the stream it must indicate to any peer NEs that source is 'On Air' . The NE directly connected to the source originator has the responsible for passing the 'On Air' indication to that terminal.

TABLE 55/T.132.

AVC-On-Air-Indication  Primitive

|  | Request | Indication |
| --- | --- | --- |
| Source AVC User ID | M | M= |
| Stream Id | M | M= |
| Source Descriptor | O | O= |
| Status |  |  |

Source AVC User Id -	The GCC User Id of the terminal originating the stream

Stream Id - Audio(n) or Video(n)

Source Device - A descriptor for  the originating the source at the terminal

FIGURE 43/T.132

AVC-On-Air-Indication - sequence of primitives

## AVC Supplementary Services

### Audio Mixing Service (AMS).

A conferencing typically requires the ability to mix audio streams from its participants in order to allow interactive discussions to take place. Any given conference may involve the requirement for multiple network elements to co-operate in delivering services. The Audio mixing service defines an generic management protocol to manage network elements and the delivery of an  Audio Mixing  service in  a homogeneous network and across network boundaries  in heterogeneous network environments. The Audio Mixing service is a supplementary AVC service. It may be specified as a requirement in the AVC Conference framework. When the management session is activated the network elements will receive the framework and shall  include the necessary support for the specified services and configurations unless it is incapable of providing that support. If the service is not required as part of the framework then  the network element should advertise its maximum capability  to provide services, when joining the conference. Based upon the Network element capabilities, the Top AVC provider is able to determine which services the AVC conference session(s) can support. Services that are available are made offered to participants at conventional nodes  by their Zone manager. Typically only a convenor or convenor privileged node would launch a service. If a service has been launched all conventional nodes are free to join it.

#### Audio Mixing Service -Capabilities

TABLE 56/T.132.

Audio Mixing Service -Functionality  & Requirement

| Audio Mixing Service | Parameter | Requirement |
| --- | --- | --- |
| Capabilities Network Elements: Terminal Nodes: | Number of Streams per port Number of ports Available Number of Separate mixers(sessions) Number of Streams per port Number of speakers per port {Remote management Control available} |  |
| AVC Audio Mixing Modes | Distribute Management by speaker detection Centrally Managed (by AVC Top Provider) Manual Control (by chair or convenor/CPN) Service Announcements Floor Controlled | Baseline (Default operation) Negotiable Available Available |

Default Operation

The audio mixing service by default (with no active roles in the session) operates in distributed management mode with each network element participating in the service providing mixer capability and  speaker detection. The Audio Mixing Service operates within the context of an AVC session. Each  Network Element participating in a session’s  audio mixing service takes the default audio stream from each session participant and includes it in it’s audio mix.  It provides a single audio output which is the sum of the inputs. Each participating Network Element is required to  monitor its audio inputs and provide speech detection, which is used to provide a local determination of a new speaker. The previous ‘New Speaker’ is then designated the ‘Previous Speaker’ for that node. In addition each network element should be capable of generating an Indication for each  active speaker that it detects and a list of those streams included in the mix. The locally determined ‘New Speaker’ and ‘Previous Speaker’ indications together with a list of active speakers is made available for use by the video services.  If the framework calls for it speaker indications are also distributed to the other network elements using the Audio Service Management Channel.

#### Chair Controlled Sessions

In the absence of framework requirements a chair role may be requested and assigned within a session as required, that chairman has the option to launch floor control as required.

Once floor control is launched other AVC services are informed. The Audio mixing service changes its operational mode, all  audio inputs to all mixers are muted except the chair and any participating convenor. Participants may request to speak, the floor control service will direct these request to the chair who can grant or refuse then or put them in a queue. Granted requests from the convenor and  relinquish floor requests from the chair or any other participant results in a Set-Audio-Mix request being sent to the service.

Where there is a defined framework this may call for the service to be launched automatically as soon as the chair joins the session, in either  case the operational mode is the same.

#### Manual Control Mode

The manual control mode is provided to allow the possibility of other applications controlling the mixing service.  The framework can require that this mode be prohibited or require convenor privileges.

#### Description of Abstract Services

##### AVC-Audio-Mix-Announce

When the Audio Mixing service has announcements enabled the AVC-Audio-Mix-Announce  primitive is issued  by  an AVC provider at  a network element which has an Audio mixer and is participating in an AVC Audio Mixing service.  . It is sent on the AVC-Audio-Service-Management channel and is received by all Network Elements enrolling with audio capabilities.

It provides the local determination of the sending node’s audio mixer as to which node it has nominated to be the new speaker, it may also send its previous speaker assignment and a list of streams included in the mix.

TABLE 57/T.132.

AVC-Audio-Mix-Announce - types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| AVC User ID | M | M= | M= |
| AVC Session Identifier | M | M= | M= |
| New Speaker GCC User-ID | M | M= | M= |
| New Speaker Microphone ID | O | O= | O= |
| New Speaker AVC Multipoint Channel Identifier (if not default) | C | C= | C= |
| New Speaker User Name string | O | O= | O= |
| Previous Speaker GCC User-ID | M | M= | M= |
| Previous Speaker Microphone ID | O | O= | O= |
| Previous Speaker AVC Multipoint Channel Identifier (if not default) | C | C= | C= |
| Previous Speaker User Name string | O | O= | O= |
| List of Nodes included in mix | O | O= | O= |
| List of Active Speakers | O | O= | O= |

AVC User ID:  Detecting Node GCC ID.

AVC  Session Identifier: The identifier for the AVC session

NewActive Speaker GCC User-ID:

New Speaker Microphone ID:

New Speaker AVC Multipoint Channel Identifier (if not default):

New Speaker User Name string:

Previous Speaker Microphone ID:

Previous Speaker AVC Multipoint Channel Identifier (if not default):

Previous Speaker User Name string:

List of Nodes included in mix:- List of Node User Ids, Mic Ids, Channel Ids identifyingall  streams being included in mixed

List of Active Speakers:- List of all  Node User Ids, Mic Ids, Channel Ids identifying as contributing to the output stream.

FIGURE 44/T.132

AVC-Audio-Mix-Announce- - Sequence of primitives

##### AVC-Audio-Mixer-Set

The AVC-Audio-Mixer-Set  primitive is issued  to provide a manual configuration of the sessions mixers. Each participating mixer will receive a list of streams to either include,  add, enable, mute or remove from the mix.

Only the convenor, convenor privileged zone or session chair are permitted to issue such requests.

The requests are  sent on the AVC-Audio-Service-Management channel and received by all Network Elements enrolling with audio capabilities.

Table 58 shows the parameters and types of this primitive.  Figure 45 shows the sequence of events when using this primitive.

TABLE 58/T.132.

AVC-Audio-Mixer-Set - types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| GCC User ID | M | M= | M= |
| AVC Session Identifier | M | M= | M= |
| Mode | M | M= | M= |
| Announcements | O | O= | O= |
| Add Node-Stream (List) | O | O= | O= |
| Enable Node-Stream (List) | O | O= | O= |
| Mute Node-Stream (List) | O | O= | O= |
| Include Node Stream(List | O | O= | O= |
| Remove Node-Stream List | O | O= | O= |

AVC User ID:  Detecting Node GCC ID.

AVC  Session Identifier: The identifier for the AVC session

Mode:- Manual/Default(distributed)/Centralised

Announcements:- flag

Add Node-Stream (List):  List of streams to be added to an existing mix

Enable Node-Stream (List):  A list of nodes to have mute removed

Mute Node-Stream (List):   A list of nodes to be Muted

Include Node Stream(List:   A list of the full set of nods to be mixed

Remove Node-Stream List:  A list of streams to be removed from mix.

FIGURE 45/T.132

AVC-Audio-Mixer-Set - Sequence of primitives

### Video Switching Service (VSS).

The video switching service operates on nodes that use video switches as a means to route video streams within a conference. It provide a standard mechanism for each participating network entity to co-operate in the delivery of a range of video switching  functions.

Within a  conference session which is utilising the video switching service, there may be a number of physical devices in different nodes that actually provide the video switching functionality. These devices may communicate with each other using a dedicated control channel. Terminals needing to communicate with the video service do so, by using the AVC management communication channel which provides a common interface independent of the conference configuration and  network components.

In a multipoint conference  the connection topology should always be arranged such that the Top Provider for the conference shall be an Network element with MCU functionality. In conference configurations where more than one NE is present, it shall be the top provider NE that arbitrates on if any given external request is to acted upon. Through  inter-NE communication the top provider can assign privileges to it client ' NEs to act independently, carrying out designated functions.

In a chaired session   the  video switching mode selection  may be restricted, or only available to the chair or the conference convenor.

#### Video Switching - Operation

A number of 'participant' rôles are utilised by the video switching service. These rôles are used to provide a focal point for the switching algorithms. Rôles may be pre-assigned in the AVC conference framework or they may be dynamically allocated,  if  the framework permits they may also be re-allocated at will during a session. The selected video switching mode  determines if and how the rôles are used to determine video routing. The following Roles are utilised by the Video switching service.

New-Speaker			Previous-Speaker		Presenter		Conference 	Conductor			Designated Stream	Group			Conference Convenor	Viewer

##### Rôle Definitions

The VSS can utilise both the session roles defined in this document and the  conference roles defined in T.124  allowing them to be used as the focus for the control algorithm.

New-Speaker

The "New-Speaker"  is a rôle where the holder changes dynamically according to some automatic algorithm which detects some type of designated activity e.g. audio, data or even video. The main purpose of this rôle is to support  Automatic Voice Switched mode where the determination of the New Speaker is provided by the Audio Mixing service and the rôle is dynamically allocated in sychronism with the corresponding Audio Mixer output. There can only be one New-Speaker at any given time.

Previous Speaker

The "Previous-Speaker" rôle is also a rôle where the holder changes dynamically according to some automatic algorithm as above, but the previous Speaker will typically be the last "New-Speaker". The main purpose of this rôle is also to support  Automatic Voice Switched mode where the algorithm is provided by the Audio Mixing service and rôle maps directly onto the corresponding Audio Mixer rôle assignment.  There can only be one Previous-Speaker at any given time.

Presenter(s)

The Presenter Rôle is defined  by AVC to give a representation of a Presenter presenting to a conference session. A conference with multiple AVC sessions may have multiple AVC Presenters. AVC also permits a session to have multiple presenters and the AVC group function can be used to define a set of intended presenters and if required an ordered sequence of presenters. This   functionality is normally only provided as part of the conference framework where the rôle holder(s) are normally known in advance and pre-assigned .. The typical use of this role is to provide control of the video to support the case where there is a participant who is presenting to a session. The presenters stream needs to be seen by all other  session participants.

Conductor

The conductor rôle is defined in recommendation T.124 which also provides the means to assign the rôle. The video switching service makes this rôle available as a point of focus. Unlike the Presenter and Chair roles this is a conference wide role.

Convenor

The Convenor rôle is defined in recommendation T.124, which also provides the means to assign the rôle. The convenor is not necessarily a conference participant, but in cases where the convenor is participating or there is a clear delegation of the role then the video switching service can makes this rôle available as a point of focus.

Viewer(s)

This rôle allows a sub group of session participants to be identified and handled in a specified way by the video switch. The viewer rôle is mainly  that of passive observation. An AVC viewer is assigned the role automatically if it enrols with  GCC as an anonymous  node.

Group Member

The AVC Group allows member nodes to be associated together for a particular application. The video switching service allows the group role to be used in its control matrix , this will allow all group members to be treated as a single entity in terms of what video they receive. It also allows the group members to provide the video source, however the results will only be correct in cases where there are sufficient receiving streams or the ability to combine the streams to match the capabilities of the designated receivers.

Designated Node(s)

This rôle is provided to allow any node to be specified as a point of focus in the switching service allowing access to the input and output options.

Override(s)

AVC must manage any interactions between different services using shared resources. The Video Switching Service (VSS) makes explicit provision for this with the ability to override the VSS  automated  use of  resources using by use of the Override feature. One example of the use of this mechanism would be the case where the Multipoint channel management Service is providing support for nodes to make specific video routing requests to it, in support of functions such as remote control. If these requests are permitted while the video Switching Service(VSS) is operating then AVC must manage any interactions between services. The Video Switching service designates these external resource requests as  Overrides and the requester becomes an override rôle holder. If permitted overriding requests take precedence over other settings but may also work alongside other selections where channel capacity permits.

#### Rôle Holder Input Options

There are a number of algorithms that can be selected from the video switching service to determine what a Rôle holder receives from the video switch. These are listed below:-

Rotate	- This causes the video switch to provide an output to the rôle holder which is a rotation of all other video streams.

Random	- This causes the video switch to provide an output to the rôle holder which is a random selection of all other video streams.

Loop back	- This causes the video switch to loop back the video from the rôle holder.

Specified Rôle	- This causes the video switch to provide an output to the rôle holder which is the video from an identified rôle holder. Note:- In the case of there being more than one holder of a given rôle then one may be selected or a rotation of the rôle holder may be done.

#### Rôle Holder's Output Options

The algorithm in use, also determines how and if the video switching service routes  the video originating from the rôle holder. The following choices are available:-

Broadcast to All	- This causes the video received from the rôle holder to be broadcast to all nodes in the conference.

Broadcast to others	- This causes the video received from the rôle holder to be broadcast to all nodes in the conference, except the originating node.

Specified Rôle		- This causes the video received from the rôle holder to be directed to the specific rôle holder identified, if the rôle holder identified is the same the video will be looped back to the originator.

Specific destination Stream(s) Group	- This allows a list of one or more receiving nodes to be specified (in terms of their AVC User ID's and stream ID's)

Authors Note 1:- MCUs are going to have to negotiate some parameters in order to support what is being proposed.

Authors Note 2:- In the case of a rôle holder having multiple streams this will have to be supported to.

#### Video Switching Matrix

The video switching service configuration and control options are defined in the form of a control matrix. A number of standard operating modes are defined using this matrix. The Video Switching Service also provides an option that when enabled, allows the potential to override the current operating mode to achieve a specific routing requirement.

The convenor may specify the requirement for the VSS in the AVC conference framework when the conference is created, this tells all NEs that the service is required.

The Video switching service has a management session component and one or more user session components corresponding to the number of conference sessions in which the VSS is operating. The VSS component needs to reside on each NE that has switching functionality contributing to the delivery of the conference. The VSS requires a  top VSS provider to arbitrate access to the control parameters and matrix, this should ideally  co-reside with the conference (GCC) Top provider.   The convenor  and any convenor privileged nodes for a conference always has access to the VSS control parameters.

TABLE 59/T.132.

Video Service Control Matrix

| VIDEO SWITCH |  | VIDEO OUTPUT TO RÔLE HOLDER |  |  |  | VIDEO OUTPUT FROM RÔLE HOLDER |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MODE RÔLE | Rotation |  | Random | Rôle | Specified Input | Broadcast to all | Broadcast to others | Rôle | Specified Outputs |
| New Speaker | flag |  | flag | rôle ID | User-Id | flag | flag | rôle ID | User-Id |
| Previous Speaker | flag |  | flag | rôle ID | User-Id | flag | flag | rôle ID | User-Id |
| Designated Presenter(s) | flag |  | flag | rôle ID | User-Id | flag | flag | rôle ID | User-Id |
| Group | flag |  | flag | rôle ID | User-Id | flag | flag | rôle ID | User-Id |
| Chair | flag |  | flag | rôle ID | User-Id | flag | flag | rôle ID | User-Id |
| Convenor | flag |  | flag | rôle ID | User-Id | flag | flag | rôle ID | User-Id |
| Designated Stream | flag |  | flag | rôle ID | User-Id | flag | flag | rôle ID | User-Id |
| Viewer(s) | flag |  | flag | rôle ID | User-Id | flag | flag | rôle ID | User-Id |
| Override | flag |  | flag | rôle ID | User-Id | flag | flag | rôle ID | User-Id |

Rotation : Flag indicating if the rôle holders received video should be rotated at the defined switching interval -True or false

Random : Flag indicating if the rôle holders video should be randomly switched at the defined switching interval -True or false

Input Rôle Identifier : indicating if the designated role holder (or receiving node)should see video from another role holder - 0=none; 1=Convenor; 2=Conductor; 3=Chair; 4=Presenter; 5=New speaker;  6=Previous Speaker; 7=Viewers; 8=Group;

Specified Input : GCC-User-ID of a node that is to provide the video

Broadcast to others :	Flag indicating if the identified rôle holders video stream should be broadcast to all nodes except the source note only one broadcast flag can be set in the control matrix.

Broadcast to all : Flag indicating if the identified rôle holders video stream should be broadcast to all nodes in session-  note only one broadcast flag can be set in the control matrix.

Output Rôle Identifier	: 0=none; 1=Convenor; 2=Conductor; 3=Chair; 4=Presenter;

5=New speaker;  6=Previous Speaker; 7=Viewers; 8=Group;

Specified Output(s) : GCC-User-ID or list of GCC-User-IDs of nodes to receive video stream

#### Standard Operating Modes

##### Voice Activated Video Switching Mode.

Automatic voice switching utilises the concept of a "New Speaker" and a "Previous Speaker" that are determined by the audio mixer(s) using some suitable algorithm (which is beyond the scope of this recommendation) to detect the currently active speaker. Once the active speaker has been determined, it is designated the "New-Speaker" site  and the previous site to be the "New-Speaker" is designated the "Previous-Speaker" site.  The Speaker detection indications are obtained from the Audio mixing service.  The video switch uses these parameters in its Voice Activated Video Switching Mode as follows:-

The Rôle holders will be dynamically changing as determined by the audio mixer, but on each switching operation the "New-Speaker"  will be broadcast to all other sites in the conference.

The "Previous-Speaker" video is only seen by  the "New-Speaker".

TABLE 60/T.132.

Video switching Service -Voice Switched Mode

| VIDEO SWITCH | VIDEO OUTPUT SEEN BY RÔLE HOLDER | VIDEO OUTPUT FROM RÔLE HOLDER TO:- |
| --- | --- | --- |
| RÔLE HOLDER |  |  |
| Current-Speaker | Previous-Speaker | Broadcast To Others |
| Previous-Speaker | Broadcast from New-Speaker | New-Speaker |

##### Presenter Mode

The Presenter Mode is provided to allow all the session participants to see the presenter, or if there is more than one, the current active presenter as determined by the audio mixing service speaker detection. The presenter(s) see the last new speaker who is not themselves. After the switching period has elapsed and no new activity is detected then the presenter(s) see the chair if there is one in the session.

TABLE 61/T.132.

Video switching Service -Presenter Mode

| VIDEO SWITCH | VIDEO OUTPUT TO RÔLE HOLDER | VIDEO OUTPUT FROM RÔLE HOLDER |
| --- | --- | --- |
| RÔLE |  |  |
| Presenter | New-Speaker/Chair/Rotation | Broadcast To Others |
| New Speaker | Broadcast from Presenter | New-Speaker |

##### Manual Control Mode

This mode effectively provides a bypass of the VSS control matrix, only allowing the Video switch Device to be used directly.

#### Mode Overrides

This mechanism allows  sites participating in the session to issue stream routing primitives in order to request specific routings of the video stream. The routing requests if permitted are also selected using the VSS control matrix, requesters are designated " Override " rôles and their incoming video is selected as a specified input. Overrides may be used in conjunction with any modes, however it will in most cases effect the operation of other modes when requests are in effect.

In a chaired session these requests should be sent to the chairpersons node for authorisation and only requests from the chairing node shall be processed by the  Video Switch.

#### Convenor Options

The convenor may request the video switching service  when creating an AVC conference. The convenor may elect to allow the video switching matrix to operate with its default settings or may reconfigure the matrix.

TABLE 62/T.132.

Video Service Control Matrix

| VIDEO SWITCH |  | VIDEO OUTPUT TO RÔLE HOLDER |  |  |  | VIDEO OUTPUT FROM RÔLE HOLDER |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MODE RÔLE | Rotation |  | Random | Rôle | Specified Input | Broadcast to all | Broadcast to others | Rôle | Specified Outputs |
| New Speaker | =0 |  | =false | PS | - | False | True | - | - |
| Previous Speaker | =0 |  | =false | NS | - |  |  | NS |  |
| Active Presenter | =0 |  | =false |  | rotation floor req | - | True | - | - |
| Group Members | =o |  | =false | NS | - | - | - | NS | - |
| Chair | =0 |  | =false | NS |  | - | True | - | - |
| Convenor | =0 |  | =false | - | - | - | - | - | - |
| Designated Stream | =0 |  | - | - | - | - | - | - | - |
| Viewer(s) | =0 |  | - | NS | - |  | - | - | - |
| Overrider | =0 |  | - | - | - | - | - | - | - |

#### Video Switching Service -Default Operation

The Video switching service may be launched without any additional configuration information or programming of the matrix. The service will by default start-up in voice activated video switching mode using the default video channels from each participating terminal in the session. A chair person may effect mode changes and re-programme the control matrix if desired. If a session has a chair and floor control activated this will restrict who is able to speak, speaker detection is still used to determine which video to switch. The default operation is based upon 1 video stream per terminal participating, and one session (the standard base session) per conference.  All participating MCUs need to be capable of supporting the service in order for it to be offered to a conference. If the capability is available the service will be offered to participating terminals by their node controller when the management session is started.

Default operation may be overridden by the AVC conference framework which allows the convenor to predefine the service configuration parameters and default operation. The framework can also define service requirements, which will cause the service to be launched and session participant may then join. The Switching matrix may be fully specified by the convenor automating the service completely.

#### Description of Abstract Services

##### AVC-VSS-Switch-Configure

The AVC Video Switching Service (VSS) has a number of configurable parameters plus a control matrix, and  the AVC-VSS-Switch-Configure primitive provides the means to access these parameters.

Access to some or all of the parameters may be restricted to convenor and CPNs.

Table 63 shows the parameters and types of this primitive.  Figure 46 shows the sequence of events when using this primitive.

TABLE 63/T.132.

AVC-VSS-Switch-Configure - types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Configuring Node ID | O | O= | O= |
| Max Number of Receiving channels per terminal | O | O= | O= |
| Required number of receiving Channels per terminal | O | O= | O= |
| Max number of transmitting channels per terminal | O | O= | O= |
| Max Number of transmitting channels per terminal | O | O= | O= |
| Permissions | O= | O= | O= |
| Overrides Permitted | O | O= | O= |
| Multiple presenters permitted | O | O= | O= |
| Rotation/ Random Switch interval default | O | O= | O= |
| VSS Control matrix -Rôle ID (List) | O | O= | O= |
| Input Mode | O | O= | O= |
| Output Mode | O | O= | O= |

Configuring Node ID - Gcc User ID or Rôle ID

Max number of receiving channels per terminal:- 0=determined by caps, 1-n

Required number of receiving channels per terminal - 0 - n

Max number of transmitting channels per terminal:- 0=determined by caps, 1-n

Required number of transmitting channels per terminal - 0 - n

Permissions to access VSS control Matrix: - Convenor by default plus any specified roles: Chair, Presenter, Conductor RDC, MCM

Overides Permitted;- FlaG

Multiple presenters permitted:- 0=NO 1-N =number

Rotation/ Random Switch interval default:-   1-255 seconds VSS Control matrix -Rôle ID (List) - Rôles New Spkr, Prev spkr, Presenter(s), Group, Convenor, Viewer(s), Override

Input Mode - Choice of: Rotation, Random, Rôle, specific node

Role ID or GCC User ID

Output Mode - Choice of Mode options:  Broadcast to all, broadcast, Rôle, specific User ID Role ID or GCC User ID

FIGURE 46/T.132

AVC-VSS-Switch-Configure - Sequence of primitives

##### AVC-VSS-Video-Switch-Set

TABLE 64/T.132.

AVC-VSS-Video-Switch-Set - types of primitives and their parameters

|  | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Site Identifier(AVC User ID) | M | M= | M= | M= |
| Video Switching Service Mode | O | O= | O= | O= |
| Rôle Identifier |  |  | O= | O= |
| Rôle Holders Incoming Stream Identifier List | O | O= | O= | O= |
| Rôle Holders Outgoing Stream Identifier list | O | O= | O= | O= |
| Service-Status Report |  |  | O | O= |

Site identifier	- Site originating the service mode selection request

Video Switching Service Mode - This parameter is used to select one of the standard modes.

Rôle Identifier	- This parameter identifies the rôle used as a point of focus for video switching.

Rôle Holders Incoming Stream identifier List	- This specifies which of the services options for  determining the input to the rôle holders node should be used.

Rôle Holders Outgoing Stream identifier List	- This specifies which of the services options for  determining where the output from the rôle holders node should go, is to  be used.

Service Status Report	- This provides feedback to requester under certain circumstances.

FIGURE 47/T.132

AVC-VSS-Video -Switch-Set  - Sequence of primitives

### "On Air" Indication Service

In a multipoint conference MCU(s) provide control and routing of audio and video streams. When a stream is routed onwards by an MCU  to a terminal  capable of  interpreting the stream it must indicate to any peer MCU's that source is 'On Air' . The MCU directly connected to the source originator has the responsible for passing the 'On Air' indication to that terminal.

TABLE 65/T.132.

AVC-Service-On-Air-Indication - types of primitives and their parameters

|  | Request | Indication |
| --- | --- | --- |
| Source AVC User ID | M | M= |
| Stream Id | M | M= |
| Source Device (n) | O | O= |
| Status |  |  |

Source AVC User Id -	The GCC User Id of the terminal originating the stream

Stream Id - Audio(n) or Video(n)

Source Device - The  Device Id of the Device originating the source at the terminal

FIGURE 48/T.132

AVC-Service-On-Air-Indication  - Sequence of primitives

### Enhanced Video Service

In face to face meeting scenario’s participants are typically able to see all of the other participants all of the time. As Teleconferencing solutions strive to provide better simulation of the face to face environment and even to improve on it, there is a need for improved video services.

However practical considerations may however restrict the ability to provide a true Continuous Presence where each of the participants can see all of the other participants all of the time, particularly in large conference situations. The following factors will all impact on what is achievable:-

- The throughput capacity of the video pipes to and from terminals and on any 	inter-MCU links

- The  topology of the conference

- Number of participants on Video Equipped Terminals.

- The video protocol and encoding in use.

- The ability of receivers to display the streams

AVC aims to provide tools to allow video processing to be used in support of various forms of enhanced video and Continuous Presence. The primary objective of Continuous Presence systems is to allow all of the participants to be seen all of the time. This can be achieved by distributing all of the participant streams to all of the participants. AVC supports this method of working, however this will impractical in most situations. AVC can be used to control the combining and processing of various source streams into one or more composite video output streams, allowing enhanced video capability with efficient use of bandwidth.

The example in Fig xxxxx below shows just one way in which AVC can be used to combine a number of video sources into a single composite stream. This particular example is based upon source streams in QCIF format and the receive stream based on Super CIF format . The current speaker is the focus for the receiver with a full CIF image in the centre of the display while other participants are positioned around the edges. The smaller images could be updated mush less frequently to reduce the bandwidth requirement.

Figure ___  Example of a composite video Stream Display

It is beyond the scope of this recommendation to define the protocols for specific CP systems, which may, in any case vary for different networks and bit rate capabilities. Other recommendations will define the formats and protocols for the video aspects of such a service. AVC will provide the control and management for these systems.

The T.120 Enhanced Video  service has the rôle of providing control and indications, in  support of  multipoint video conferences where some form of Enhanced video  system is required.

The AVC recommendation inherently supports many of the concepts required in order to support a  Continuous Presence service; however the requirements placed on MCU's and also on terminals to support CP may be significant.  The flexible approach to the provision of control adopted within this recommendation should not  constrain these or alternative methods from being used to provide such services. Nodes advertise their ability to support the various CP modes in the application roster. The AVC conference creator may predefine the mode used or an appropriate set of defaults may be defined.

#### Continuous Presence Multiplexer

In this mode video terminals provide  a single video stream on their 'up links' to an MCU. In each MCU a Time Division Multiplex(TDM) is used to combines the 'up link' streams received from participating video terminals to produce a multiplexed output  stream. Provided sufficient bit rate is available, the down link will contain a multiplex of  video streams from all other  terminals. A video multiplexer function is defined in this recommendation to support the control of such a method. This method provides a simple solution for MCU's; however terminals require as many codec receivers as there are images being received. Also this mode is the most difficult of the modes to scale up to larger conferences.  Figure 5/AVC shows an example where a terminal sources one stream and receives four. This configuration would also apply to the other four terminals in the conference.

#### Continuous Presence Transcoder

The transcoding method is based on a video transcoder in each MCU which decodes the incoming video received at the 'up links' from participants terminals, converting back the signal into its raw uncompressed format. This signal can then be re-coded to provide a resultant image combining any required combination of the input images in any desired position to any scale. However, considerations such image resolution, and  size of the resulting elements in a combined picture will impose practical constraints on what can be done.

This method provides the most flexibility to reprocess, resize and scale images, allowing  transcoders at MCUs the capability to adapt the images they provide, tailoring them to their terminals requirements or capabilities.

Transcoding will impose greater delays as an extra decoding and encoding process are added.

#### Continuous Presence Array Processor

This is method is similar to that of the video multiplexer except that it only deals with images in The Common Interchange format and resolutions. The Array method combines the individual 'up-link' pictures into a larger overall picture  frame or frames; these can be sent to receiving terminals and displayed as an array of pictures from different participants. This procedure can be directly applied to the picture coding hierarchy in ITU recommendation H.261/262 (see figure below), which defines a picture in terms of three Common Interface Formats(CIF). the smallest is  quarter CIF (QCIF), the next is CIF which is 4 times the size of QCIF. The largest format is Super-CIF (S-CIF) which is 4 times the size of CIF.  The array processor can combine  'Up link' images flexibly within the output stream(s) to provide a variety of display options.

## AVC Peer-to-Peer Functions

### Privacy

When a terminal endpoint wishes to temporarily suspend transmission of video and or audio to allow local consultation, the AVC-Privacy message is sent from that endpoint to the NE communication channel and the source removes its outgoing streams.  Zone managers for terminals who are receiving that stream will forward the  privacy indication to those endpoints. Likewise when the privacy mode is cancelled by the endpoint the same procedure is followed.  A zone manager could choose to provide a stream with the privacy indication rather than a message, if this was more appropriate for an endpoint.

TABLE 66/T.132.

AVC-Service-Privacy-Notify  Primitive

|  | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Source AVC User ID | M | M= | M= |
| Multipoint Channel ID | M | M= | M= |
| Privacy Status | M | M= | M= |

Source AVC User Id -	The GCC User Id of the terminal originating the stream

Multipoint Channel ID - Audio(n) or Video(n)

Privacy Status - On/Off

## Stream Servers { place-holder}

## Source Selection

AVC provides mechanisms to allow a node to advertise a list of sources which it is making available to the conference for selection. Examples of sources include streams introduced from a media server or ‘off air’ broadcast signals.  In this context, physical devices such as cameras are classified as sources.  The AVC Source Selection Service allows a node to request that a particular source be connected to a specific multipoint channel.  Unlike other AVC services, source selection is not dependent on support by Network Elements; it may thus be used in point-to-point operation.

Nodes advertise the presence of selectable sources as non-collapsing capabilities in the AVC Terminal Standard Base Session.  Templates for standardised source devices are defined in Annex C.  Sources may support the ability to be controlled by a remote node for which procedures are defined in §8.9.2.

### AVC-Source-Select

The AVC-Source-Select request primitive is issued by the Node Controller at an AVC Terminal to request that a particular source be connected to a specific multipoint channel.  The request is directed to the node at which the source is located.

FIGURE 49/T.132

AVC-Source-Select - sequence of primitives

TABLE 67/T.132.

AVC-Source-Select

| Content | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) | M(=) |
| Requesting Node |  | M | M(=) |  |
| Source Owner | M |  |  |  |
| Device Class | M | M(=) |  |  |
| Device Identifier | M | M(=) |  |  |
| Multipoint Channel Identifier | O | O(=) |  |  |
| Result |  |  | M | M(=) |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User ID of the requesting node.

Source Owner:  AVC User ID of the node at which the source to be selected is located.

Device Class: This parameter identifies the type of device required to act as a source.  Standardised device classes are defined in Annex C.

Device Identifier: Locally unique number, allowing discrimination between multiple devices of the same class at the source owner node.  The Device Identifier is extracted from the appropriate Device Entry (see), advertised as a non-collapsing capability in the AVC Terminal Standard Base Session.

Multipoint Channel Id: A globally unique number identifying the multipoint channel to which the selected source is to be connected.  This parameter is omitted to request that the selected source be connected to the default channel for the media type(s) generated by the source.  Default channels are assigned in the appropriate Network Specific Mappings.

Result:  Indicates whether the selected device was connected to the multipoint channel.  It contains one of the following results: successful, not authorised node, request denied, invalid source id, invalid multipoint channel id, device not available.

### Remote Device Control

AVC provides modelling and classification of devices for the purpose of allowing remote activation and control within a conference environment.

A number of standard device classes and a library of standard device attributes are defined by AVC (see Annex C). These aim to address mandatory or commonly required functionality for use in support of multipoint multimedia communication and in particular conferencing.

A device is normally a physical entity but may also be a software process and will typically reside at an AVC Terminal node. In both cases the AVC control model requires that the host node is the default owner of a given device and has responsibility for servicing remote device requests. It is not necessary for a host node to be co-located with the device it is hosting. A device can be a source or receiver of  a real time stream or a device may be both a source and a receiver such as in the case of a video cassette recorder.

Remote device control provides the following facilities to the conference :-

- declaration of devices and their associated attributes to enable a conference participant to determine a profile of device attached to a remote node.

- a mechanism to allow a participant to request exclusive access to a remote device in order to perform device control or configuration.

- mechanisms to allow a participant to control, configure, receive event notification and obtain the status of a remote device.

- the definition of a standard device class which allows a participant to select a set of remote devices which have their outputs combined together to form a single output stream.

Unlike other AVC services, remote device control is not dependent on support by Network Elements; it may thus be used in point-to-point operation.

At each node, each device is assigned a device class identifier, according to the type of device, and a unique numeric identifier to distinguish it from other devices with the same class identifier. The device class and identifier taken together with the AVC User ID of the node that owns the device provides a unique handle for each device within the conference.

Nodes may advertise devices that they will allow to be remotely accessed and controlled via Device entries in the AVC application roster. A device list is provided to GCC as a set of non-collapsing capabilities, which GCC distributes unchanged, to all other AVC nodes in the terminal session. A node must interrogate a remote node to obtain the device details before control or configuration of that device commences. A device does not require the existence of peer devices to be used in a conference.

Each device entry in the list has the following format.

| Device Entry Field | Description |
| --- | --- |
| Device Class | This is one of the standard classes defined in Annex C or a non-standard user class. |
| Device Identifier | This is a number ranging from 0 to 127 which identifies a particular device of this class that is connected to this node. |
| Source Stream Type | This identifies the type of stream or streams that this device may source. It will take one of the following values :- None, Video only, Audio only, Video and Audio |
| Sink Stream Type | This identifies the type of stream or streams that this device may sink. It will take one of the following values :- None, Video only, Audio only, Video and Audio |
| Device has Attributes | This flag is set to TRUE if the device has any attributes which may be used for device control. If this flag is FALSE then the device is only available to be selected as a stream source. |
| Device Name | This is an optional text string which contains the name of the device. |

If a device is declared as having attributes, these must be obtained by a remote node before control or configuration can commence.

A device that is declared as having a source stream type other than None is available for source selection.

It is permissible for a  device to be declared as controllable but not be available for source selection or for a device to be available for source selection but is not controllable.

If any of the attributes declared by a device change, a node must remove that device entry from the AVC application roster and re-advertise the device with a new identifier.  As the new instance of the application roster is distributed, peer nodes will see the old device entry disappear and the new device entry added. A node will then be required to interrogate a host node to obtain the new device details before control or configuration of that device commences. A device may not be removed from the application roster during the period that it is locked by a remote node.

#### AVC-Device-Attributes

An AVC application wishing to control, configure, obtain the status or receive events from a specific device at another node will first be required to obtain the device attributes using the AVC-Device-Attributes request primitive. This primitive is issued by the Node Controller at an AVC Node and is directed to the node at which the device is located. If the request is successful the AVC-Device-Attributes confirm primitive will contain the entire list of device attributes. Certain attributes will be accompanied with information describing  the range of values supported by the parameters used by that attribute. See Annex C for more details on device classes and their attributes.

Device attributes are divided into three main types :- control attributes,  status attributes and event attributes. Control attributes may be used to control a remote device and are used by the Device-Control primitive (see section ). Status attributes may be used to obtain the status of  a remote device and are used by the Device-Status primitive (See section ). Event attributes may be used to request notification of events from a remote device (See section ).

FIGURE 50/T.132

AVC-Device-Attributes - sequence of  primitives

TABLE 68/T.132.

AVC-Device-Attributes

| Content | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) | M(=) |
| Requesting Node |  | M | M(=) |  |
| Device Class | M | M(=) |  |  |
| Device Identifier | M | M(=) |  |  |
| List of device attributes |  |  | M | M(=) |
| Result |  |  | M | M(=) |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User ID of the requesting node.

Device Class: This parameter identifies the type of device for which a list of attributes is required.  Standardised device classes are defined in Annex C.

Device Identifier: Locally unique number, allowing discrimination between multiple devices of the same class at the source owner node.  The Device Identifier is extracted from the appropriate Device Entry, advertised as a non-collapsing capability in the AVC Terminal Standard Base Session.

List of Device Attributes: This is a list of device attributes. The format of these attributes is defined in Annex C.

Result:  Indicates whether the lock operation on the selected device was successful.  It contains one of the following results: successful,  request denied, unknown device, device not available.

### Device Control

An AVC node must lock a remote device to obtain exclusive access prior to performing remote control using the AVC-Device-Lock. Similarly if a node requires exclusive access to a local device which it is hosting in order to perform local control then it must lock this device. In this way a device will only be controlled by one node at one time.

An AVC node always had ultimate authority over a lock placed on any device that it is hosting.  It may choose to terminate a lock on a hosted device if access to the device is required and then inform the remote node that it has lost exclusive access. Alternatively it may interrogate the remote node to determine if the lock is still required and then terminate the lock if  it is no longer required.

A device does not need to be locked in order to obtain status information or to configure a device with an event list for event notification.

#### AVC-Device-Lock

The AVC-Device-Lock request primitive is issued by the Node Controller at an AVC node to configure the lock state of a particular device. The request is directed to the node at which the device is located. A device can not be locked if it has already been locked by another AVC node. An AVC node may only request a device be unlocked, if it originally locked that device.

FIGURE 51/T.132

AVC-Device-Lock - sequence of  primitives

TABLE 69/T.132.

AVC-Device-Lock

| Content | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) | M(=) |
| Requesting Node |  | M | M(=) |  |
| Device Class | M | M(=) |  |  |
| Device Identifier | M | M(=) |  |  |
| Lock / Unlock | M | M(=) |  |  |
| Result |  |  | M | M(=) |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User ID of the requesting node.

Device Class: This parameter identifies the type of device required to be locked or unlocked.  Standardised device classes are defined in Annex C.

Device Identifier: Locally unique number, allowing discrimination between multiple devices of the same class at the source owner node.  The Device Identifier is extracted from the appropriate Device Entry advertised as a non-collapsing capability in the AVC Terminal Standard Base Session.

Lock / Unlock: Set to TRUE if the request is for the device to be locked otherwise FALSE in order to unlock the device.

Result:  Indicates whether the lock operation on the selected device was successful.  It contains one of the following results: successful, ,request denied, unknown device, device not available.

#### AVC-Device-Lock-Inquire

If an AVC node requires a lock to be removed from one of its devices it may first wish to interrogate the remote AVC node that locked the device using the AVC-Device-Lock-Inquire request primitive to determine if the lock is still required. If the remote AVC node indicates that the device lock is no longer required then the device may be unlocked immediately.

FIGURE 52/T.132

AVC-Device-Lock-Inquire - sequence of primitives

TABLE 70/T.132.

AVC-Device-Lock-Inquire

| Content | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) | M(=) |
| Requesting Node |  | M | M(=) |  |
| Device Class | M | M(=) |  |  |
| Device Identifier | M | M(=) |  |  |
| Result |  |  | M | M(=) |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User ID of the requesting node.

Device Class: This parameter identifies the device type of the locked device.  Standardised device classes are defined in Annex C.

Device Identifier: Locally unique number, allowing discrimination between multiple devices of the same class at the source owner node.  The Device Identifier is extracted from the appropriate Device Entry, advertised as a non-collapsing capability in the AVC Terminal Standard Base Session.

Result:  Indicates whether the device lock on a particular device is still required.  It contains one of the following results: device lock may be released , device lock still required, unknown device.

#### AVC-Device-Lock-Terminated

An AVC node always has ultimate authority over the locks placed on any device owned by it. It may choose to terminate a device lock placed by a remote terminal. If a device lock is terminated without first interrogating the remote node using the AVC-Device-Lock-Inquire request primitive then the AVC-Device-Lock-Terminated request primitive should be used to inform the remote node that it no longer has exclusive access.

FIGURE 53/T.132

AVC-Device-Lock-Terminate - sequence of primitives

TABLE 71/T.132.

AVC-Device-Lock-Terminate

| Content | Request | Indication |  |
| --- | --- | --- | --- |
| Host Node |  | M |  |
| Device Class | M | M(=) |  |
| Device Identifier | M | M(=) |  |

Host Node:  AVC User ID of the node that is hosting the device on which the lock has been terminated.

Device Class: This parameter identifies the type of device on which the lock was terminated. Standardised device classes are defined in Annex C.

Device Identifier: Locally unique number, allowing discrimination between multiple devices of the same class.  The Device Identifier is extracted from the appropriate Device Entry, advertised as a non-collapsing capability in the AVC Terminal Standard Base Session.

#### AVC-Device-Control

The AVC-Device-Control request primitive is issued by the Node Controller at an AVC node to control a particular remote device that has been advertised in the AVC application roster. The request is directed to the node at which the device is located. The device should previously have been locked by the requesting node before device control can commence.

FIGURE 54/T.132

AVC-Device-Control - sequence of primitives

TABLE 72/T.132.

AVC-Device-Control

| Content | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M |  | M(=) |
| Requesting Node | M | M(=) |  |
| Device Class | M | M(=) |  |
| Device Identifier | M | M(=) |  |
| List of device control attributes | M | M(=) |  |
| Result |  |  | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User ID of the requesting node.

Device Class: This parameter identifies the type of device on which the lock was terminated. Standardised device classes are defined in Annex C.

Device Identifier: Locally unique number, allowing discrimination between multiple devices of the same class.  The Device Identifier is extracted from the appropriate Device Entry, advertised as a non-collapsing capability in the AVC Terminal Standard Base Session.

List of device control attributes: This is a list of device control attributes. The format of these attributes is defined in Annex C. This list should not contain attributes which would result in opposing actions being executed by the device.

Result:  Indicates whether the request was successful.  It contains one of the following results: successful,  unknown device, device attribute error, device not locked.

#### AVC-Device-Status

An AVC application can interrogate a remote device for its status using the AVC-Device-Status request primitive. This primitive contains a list of status attributes for which the status is required. Assuming a successful response, the AVC-Device-Status confirm primitive contains a list of these same status attributes with accompanying parameters indicating their status value.

The AVC-Device-Status request primitive is issued by the Node Controller at an AVC node to inquire of the status of a particular remote device that has been advertised in the AVC application roster. The request is directed to the node at which the device is located.

FIGURE 55/T.132

AVC-Device-Status - sequence of primitives

TABLE 73/T.132.

AVC-Device-Status

| Content | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) | M(=) |
| Requesting Node |  | M | M(=) |  |
| Device Class | M | M(=) |  |  |
| Device Identifier | M | M(=) |  |  |
| List of status attributes | M | M(=) |  |  |
| List of status attributes with associated responses |  |  | M | M(=) |
| Result |  |  | M | M(=) |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User ID of the requesting node.

Device Class: This parameter identifies the device type of the locked device.  Standardised device classes are defined in Annex C.

Device Identifier: Locally unique number, allowing discrimination between multiple devices of the same class at the source owner node.  The Device Identifier is extracted from the appropriate Device Entry, advertised as a non-collapsing capability in the AVC Terminal Standard Base Session.

List of device status attributes: This is a list of device status attributes for which the status is required. The format of these attributes is defined in Annex C.

List of device status attributes with associated responses:  This is the list of device status attributes with  associated status responses. The format of these attributes is defined in Annex C.

Result:  Indicates whether the device lock on a particular device is still required.  It contains one of the following results: successful, request denied, unknown device, device not available, device attribute error.

#### AVC-Configure-Device-Events

An AVC application can request that it is notified of event changes occurring on a remote device. A remote device can declare support for particular event attributes in the attribute list contained in the AVC-Device-Attributes confirm primitive. The AVC application can configure a remote device with a list of event attributes for which notification is required using the AVC-Configure-Device-Events primitive.

The AVC-Configure-Device-Events request primitive is issued by the Node Controller at an AVC node to configure an event list for a particular remote device that has been advertised in the AVC application roster. The request is directed to the node at which the device is located. If a node wishes to cancel an event list which it has previously configured then it should send this primitive with an empty list of events.

FIGURE 56/T.132

AVC-Configure-Device-Events - sequence of primitives

TABLE 74/T.132.

AVC-Configure-Device-Events

| Content | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Request Handle | M | M(=) | M(=) | M(=) |
| Requesting Node |  | M | M(=) |  |
| Device Class | M | M(=) |  |  |
| Device Identifier | M | M(=) |  |  |
| List of event attributes | M | M(=) |  |  |
| Result |  |  | M | M(=) |

Request Handle: Locally unique number to allow the requesting node to match responses.

Requesting Node: AVC User ID of the requesting node.

Device Class: This parameter identifies the device type of the locked device.  Standardised device classes are defined in Annex C.

Device Identifier: Locally unique number, allowing discrimination between multiple devices of the same class at the source owner node.  The Device Identifier is extracted from the appropriate Device Entry, advertised as a non-collapsing capability in the AVC Terminal Standard Base Session.

List of device event attributes: This is a list of device event attributes for which event notification is requested. The format of these attributes is defined in Annex C. This event list replaces any event list that has been previously  configured by the sending node.

Result:  Indicates whether the device lock on a particular device is still required.  It contains one of the following results: successful, request denied, unknown device, device not available, device attribute error.

#### AVC-Notify-Device-Event

A device may have an event list configured by a remote AVC node using the AVC-Configure-Device-Events primitive. When a listed event occurs, an AVC node can inform an interested remote node using the AVC-Notify-Device-Event request primitive. The request is directed to the remote node which configured the device event list. If more than one terminal has configured a device event list then this primitive will be sent to each terminal which has listed the event.

FIGURE 57/T.132

AVC-Device-Event-Notify - sequence of primitives

TABLE 75/T.132.

AVC-Notify-Device-Event

| Content | Request | Indication |  |
| --- | --- | --- | --- |
| Device Class | M | M(=) |  |
| Device Identifier | M | M(=) |  |
| List of event attributes | M | M(=) |  |

Device Class: This parameter identifies the type of device on which the lock was terminated. Standardised device classes are defined in Annex C.

Device Identifier: Locally unique number, allowing discrimination between multiple devices of the same class.  The Device Identifier is extracted from the appropriate Device Entry, advertised as a non-collapsing capability in the AVC Terminal Standard Base Session.

List of event attributes: This is a list of device event attributes indicating which events are being notified. The format of these attributes is defined in Annex C.

## Conducted Mode Behaviour

AVC is a conductor aware application protocol in that it has two modes of operation one for non-conducted mode and one for conducted mode, as described in this section.

Application become aware of a GCC Conference Conductor being assigned through receipt of a GCC-Conductor-Assign indication from its GCC provider. This includes the GCC User ID of the conducting node. Application must determine the identity of the AVC application protocol which inherits the conductors rôle.

Communications on the AVC-MCU-Control channel are not effected, nor are start-up or capability processes. No new use of functions other than privacy may be started without permission from the conductor.

# AVC Infrastructure Management Protocol Definition

## AVC Communication Channels

For a multipoint session AVC utilises 4 MCS static channels to provide management and control services for real time channels:-

·	AVC Broadcast Channel

·	AVC Network Element Control Channel

·	AVC Network Element Communication Channel

·	AVC  Top Provider Channel

For a point-to-point session only the AVC Broadcast Channel will be utilised.

### AVC Broadcast Channel

The AVC Broadcast Channel is joined by all AVC Providers.  It is used for broadcasting information to all AVC capable nodes.

### AVC Network Element Control Channel

The  AVC Network Element Control Channel is joined by all AVC Network Elements.  It shall not be joined or used by AVC Providers at AVC Terminals. Its use is reserved for communications between Network Elements.  AVC Network Elements providing time critical management functions are permitted to use this channel at Top priority (i.e. the priority that is normally reserved for GCC management  functions).

### AVC Network Element Communication Channel

The AVC Network Element Communication Channel is also joined by all AVC Network Elements. This channel may not be joined by AVC Terminals but in a multipoint session they may send messages to it in order to communicate with the AVC Network Elements.  It provides a single channel for communication to Network Elements that is independent of the number of  Network Elements and conference topology.  Effectively, it allows AVC Terminals to see the collection of AVC Network Elements as a single ‘virtual’  MCU.

### AVC Top Provider Channel

The AVC Top Provider Channel is joined by the Top AVC Provider only and is provided to allow dynamic transfer of the Top AVC Provider between nodes in future revisions.  AVC nodes may send messages to this channel to communicate with the Top Provider.

FIGURE 58/T.132

AVC communication channels in a point-to-point environment

FIGURE 59/T.132

AVC communication channels in a multipoint environment

## Nodes Entering an AVC Controlled Conference

In order to participate in a conference, an AVC Provider shall establish an MCSAP and a GCCSAP.  The method of creating SAPs is a local matter, outside the scope of this Recommendation.  When a node joins a conference, its AVC Provider shall wait until it receives a GCC-Application-Permission-To-Enroll indication.  If the node has been designated as an AVC Terminal, the AVC Provider shall then attempt to enroll in the AVC Registration Session as described in T.121; it may optionally join the AVC Terminal Standard Base Session as described in §9.2.1.   If the node has been designated as an AVC Network Element, it shall attempt to enroll in the AVC Network Element Standard Base Session, as described in §9.2.2.

The AVC Provider shall not attempt to enroll in any AVC Terminal Public or Private Sessions, unless explicitly requested to do so through receipt of an AVC-Session-Join request.

### Enrolling in the AVC Terminal Standard Base Session

An AVC Provider at a Terminal shall follow the procedures defined in T.121 to enroll in the AVC Terminal Standard Base Session.  It shall join the AVC Broadcast Channel.  The AVC Broadcast Channel is used as Session ID.  Any devices that are to be made available to the conference for remote control or selection are advertised in the non-collapsing capability list, as shown in tables 76 and 77.

Once the Node Controller has received a GCC-Application-Roster-Report indication containing an entry in the AVC Terminal Standard Base Session for its local AVC Provider, it may start to use AVC peer-to-peer functions.

At any time subsequently, the Node Controller may be notified that management of AVC services has been activated through receipt of an AVC-Announce-Zone-Manager indication primitive, identifying the AVC Network Element that is to act as its Zone Manager.  The Node Controller shall then advertise the capabilities and configuration of the node and its associated real time links.  For each real time link on which it is the upper node, the Node Controller shall issue an AVC-Announce-Link-Capabilities request and an AVC-Announce-Link-Configuration request, even if it has no capability or configuration information to report.  The Node Controller shall also issue an AVC-Announce-Node-Capabilities and an AVC-Announce-Node-Configuration to report any additional capability and configuration information pertaining to that node..  The Node Controller shall then wait until it receives an AVC-Service-Offer indication from its Zone Manager, offering it a range of services.

AVC Network Elements may enroll in the AVC Terminal Standard Base session in order to access AVC Terminal functionality.  Such nodes shall not advertise their AVC service capabilities and link and node capabilities and configuration when enrolling in this session.  They may, however, advertise any devices which they wish to make available for remote control.

Figure 60 illustrates a typical start up scenario for an AVC Terminal participating in a conference in which management of AVC services is in operation.

FIGURE 60/T.121

Typical Start-Up Procedure for AVC Session - AVC Terminal

TABLE 76/T.132.

AVC Terminal Standard Base Session Non-Collapsing Capability List Elements

| Capability Name | Capability Description | ID | Application Data |
| --- | --- | --- | --- |
| Camera | A list of cameras available for remote control | 1 | Device List |
| Microphone | A list of microphones available for remote control | 2 | Device List |
| Stream Player Recorder | A list of stream player recorders available for remote control | 3 | Device List |
| Slide Projector | A list of slide projectors available for remote control | 4 | Device List |
| Light Source | A list of light sources available for remote control | 5 | Device List |
| Document Scanner | A list of document scanners available for remote control | 6 | Device List |
| Source Combiner | A list of source combiners available for remote control | 7 | Device List |

TABLE 77/T.132.

Content of Device List Entry

| Parameter | Description | Type/Value |
| --- | --- | --- |
| Device Identifier | A locally unique number used to distinguish multiple devices of the same type at a node. | INTEGER (0..127) |
| Audio Source Flag | Flag indicating whether the device can source audio. | BOOLEAN |
| Audio Sink Flag | Flag indicating whether the device can sink audio. | BOOLEAN |
| Video Source Flag | Flag indicating whether the device can source video. | BOOLEAN |
| Video Sink Flag | Flag indicating whether the device can sink video. | BOOLEAN |
| Remote Control Flag | Flag indicating whether the device is available for remote control. | BOOLEAN |
| Device Name | An optional text string used to describe the device. | TextString |

### AVC Network Element Enrolment Procedure

An AVC Provider at an AVC Network Element shall follow the procedures defined in T.121 to enroll in the AVC Registration Session.  No capabilities shall be provided.

After receiving a GCC-Application-Roster-Report indication with an entry containing its own Application Protocol Entity ID, the AVC Provider shall follow the procedures defined in T.121 to enroll in the AVC Network Element Standard Base Session.  It shall join the AVC Broadcast Channel, AVC NE Control Channel and AVC NE Communication Channel.  The AVC NE Control Channel is used as Session ID.  If the AVC Provider is at the same node as the Top GCC Provider (i.e. it is the Top AVC Provider), it shall also join the AVC Top Provider Channel before enrolling active in the AVC Network Element Session with the ZoneManagerFlag set and including capabilities for each AVC service supported by the node.  Otherwise, the AVC Provider shall wait until it has received an InitiateManagementIndication AVCPDU from the Top AVC Provider.  On receipt of this PDU, an AVC Provider shall examine the Conference Management Mode parameter contained in the PDU and the EnableZoneManager flag set by its local Node Controller to determine whether it is to act as a Zone Manager and, if not, wait for another node to identify itself as Zone Manager

If the Conference Management Mode is Centralised, the Top AVC Provider is the only Zone Manager.

If the Conference Management Mode is Distributed-By Network, the AVC Provider is a Zone Manager if it is the AVC Network Element within its particular sub-network that is highest in the T.120 hierarchy (we need to specify how this can be determined).

If the Conference Management Mode is Distributed-Unrestricted, the AVC Provider is a Zone Manager if the AVC EnableZoneManagerFlag is set (this is determined locally, and may, for example depend on what type of Network Element the node is).

If the Network Element is a Zone Manager, the AVC Provider shall enroll active in the AVC Network Element session, with the ZoneManagerFlag set and including capabilities for each AVC service supported by the node.  Otherwise, the AVC Provider shall wait until it has received an AnnounceZoneManagerIndication AVCPDU, identifying the AVC Network Element that is to act as its Zone Manager.  It shall then enroll active in the AVC Network Element Session, with the ZoneManagerId capability set to the AVC User Id of the node’s Zone Manager and including appropriate capabilities for each AVC service supported by the node.

In all cases, the Node Controller shall then wait until it has received a GCC-Application-Roster-Report indication containing an entry in the AVC Network Element Standard Base Session for its local AVC Provider.  The Node Controller shall then advertise the capabilities and configuration of the node and its associated real time links.  For each real time link on which it is the upper node, the Node Controller shall issue an AVC-Announce-Link-Capabilities request primitive and an AVC-Announce-Link-Configuration request, even if it has no capability or configuration information to report.  The Node Controller shall also issue an AVC-Announce-Node-Capabilities and an AVC-Announce-Node-Configuration to report any additional capability and configuration information pertaining to that node.    (Editor’s Note:  Do we wish to consider integrating the Announce-Node/Link-Capabilities/Configuration primitives into a single primitive to avoid the need for all nodes to send 4 PDUs ?)

TABLE 78/T.132.

AVC Network Element Session Non-Collapsing Capability List Elements

| Capability Name | Capability Description | ID | Capability Type |
| --- | --- | --- | --- |
| ZoneManagerFlag | Set if the AVC Provider is a Zone Manager | 1 | - |
| ZoneManagerId | ZoneManagerFlag = FALSE and Conference Management Mode not Centralised AVC User Id of AVC Provider’s Zone Manager else Omitted | 2 | AVC User Id |

TABLE 79/T.132.

AVC Capability List Elements

| Capability Name Default Value and Description | ID | Class | APE Count Rule | Capability Value Range | Dependency |
| --- | --- | --- | --- | --- | --- |
| Editor’s Note: Insert AVC Service Capabilities here | 1 |  |  |  |  |
| Non-standard Capability (default: unspecified) This is used to negotiate non-standard services. Any number of these can appear in the Application Capability List, provided each has a unique Non-Standard Identifier. The interpretation of these capabilities is not defined in this recommendation. | Non-Standard Identifier | - | - | - | - |

APE Count Rule:

> 0 - if the numberOfEntities parameter returned by the GCC-Application-Roster-Report indication for this capability is greater than zero, then the result of the unsigned minimum operation is established, otherwise the default value of the capability is established.

= ALL - the numberOfEntities parameter returned by the GCC-Application-Roster-Report indication for this capability must equal the number of active File APEs enrolled in the current session, otherwise the default value of the capability is established.

= D - if the numberOfEntities parameter returned by the GCC-Application-Roster-Report indication for this capability is equal to the numberOfEntities parameter for the  capability on which it depends, then the result of the unsigned minimum operation is established, otherwise the default value of the capability is established.

If the node is a Zone Manager, the Node Controller should wait until the AVC service arbitration process is complete (i.e. it has received a GCC Application Roster Report in which all AVC Network Elements present in the conference have enrolled active in the AVC Network Element Session) before offering AVC services to the terminals within its Zone.  The Node Controller should not attempt to offer services to a terminal until it has received capability and configuration information about that terminal, to avoid the possibility of offering services that are incompatible with the terminal.  Services may be made available to the entire conference, or to individual AVC Terminals on a node by node basis, according to the outcome of the AVC service capability exchange.  The availability of services to an AVC Terminal may depend on the rôle of the AVC Terminal within the conference (e.g. chair, presenter, peer) and may be changed during the conference.

## Updating Late Joining Network Elements

The Top AVC Provider shall monitor the Application Roster for the AVC Network Element Session.  When a new AVC Network Element enrolls active in this session, the Top AVC Provider is responsible for supplying a complete copy of both the Link Roster and Node Roster to this node.  It shall do this by generating a RosterUpdateIndication AVCPDU and issuing an MCS-Send-Data request, specifying the AVC User ID of the incoming node as the Channel ID, specifying High data priority, and including the PDU in the Data field.

The Top AVC Provider forward a  copy of the conference framework to the new AVC Network Element by generating an InitiateManagementIndication AVCPDU and issuing an MCS-Send-Data request, specifying the AVC User ID of the incoming node as the Channel ID, specifying High data priority, and including the PDU in the Data field.

The Top Provider shall also notify the new AVC Network Element of any existing active AVC services by generating a ServiceLaunchReportIndication AVCPDU providing a list of all active services and the sessions in which they are active and setting the Refresh Flag to TRUE.  It shall then issue an MCS-Send-Data request, specifying the AVC User ID of the incoming node as the Channel ID, specifying High data priority, and including the PDU in the Data field.

## Zone Manager Identification

Each time a Zone Manager receives an Application Roster Report for the AVC Network Element Standard Base Session, it shall examine any new ZoneManagerID capability entries to determine if it is to assume Zone Management responsibility for the incoming node(s).  Similarly, each time a Zone Manager receives an Application Roster Report for the AVC Terminal Standard Base Session, it shall determine whether any new nodes have joined its Zone.  An AVC Terminal belongs to a Zone if it has an upward T.120 connection to a node that is already within that Zone.  If an AVC Provider determines that it is the Zone Manager of an incoming node, it shall send an AnnounceZoneManagerIndication AVCPDU to that node by issuing an MCS-Send-Data request, specifying the AVC User ID of the incoming node as the Channel ID, specifying High data priority, and including the PDU in the Data field.

TABLE 80/T.132.

AnnounceZoneManagerIndication AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| No Parameters | - | - |

On receipt of an AnnounceZoneManagerIndication AVCPDU, an AVC Provider shall generate an AVC-Announce-Zone-Manager indication and issue it to the Control AVCSAP.

## Session Management

### Joining an AVC Session

On receipt of an AVC-Session-Join request primitive via the Control AVCSAP, an AVC Provider shall examine the sessionID parameter to determine whether it is required to join an existing AVC session or attempt to create a new session.

If the sessionID parameter is omitted, the AVC Provider shall consult the conference framework (Editor’s Note: we need to specify the appropriate parameter) to determine whether it is permitted to create a new session.  If not, it shall generate an AVC-Session-Join confirm primitive with a negative result, indicating that the node is not authorised to perform this operation and issue it to the Control AVCSAP.  Otherwise, it shall examine the Session Type parameter.  If this is omitted or set to public, the AVC Provider shall follow the procedures defined in T.121 to enroll in a Public Session.  If the parameter is set to private, it shall follow the procedures defined in T.121 to enroll in a Private Session.  In each case, the AVC Provider shall use its existing MCS User Id when enrolling in the new session.  Other parameters shall be set in accordance with table 81.  If the AVC-Session-Join request includes a Session Context, the AVC Provider shall create a GCC Registry Entry for the Session Context as described in T.121.

If the sessionID parameter is present, the AVC Provider shall consult the GCC Application Roster to determine whether the AVC session with the specified Session Id exists.  If not, it shall generate an AVC-Session-Join confirm primitive with a negative result, indicating that an invalid session id was supplied and issue it to the Control AVCSAP.  Otherwise, it shall examine the application roster entry for the indicated session and, if the Start-up Channel is specified as Dynamic Private, the AVC Provider shall determine whether its local node has been admitted to the session.  If not, it shall generate an AVC-Session-Join confirm primitive with a negative result, indicating that the node is not authorised to join this session and issue it to the Control AVCSAP.

Once the AVC Provider has ascertained that the session exists and its local node is authorised to join the session, the AVC Provider shall follow the procedures defined in T.121 to join the session.  The AVC Provider shall use its existing MCS User Id when enrolling in the session and shall set the other parameters in accordance with table 81.

TABLE 81/T.132.

Parameters for enrolling in an AVC session

| Parameter | Value/Source |
| --- | --- |
| Enroll/Un-enroll | ENROLL |
| application-protocol-key | ASN.1 OBJECT IDENTIFIER for AVC |
| sessionType | As specified in AVC-Session-Join request Session Type parameter when creating a new AVC session. Derived from the GCC Application Roster when joining an existing AVC session. |
| sessionID | Omitted |
| admitList | Omitted |
| resourceList | Omitted |
| Conducting Operation Capable | Set if the APE is capable of becoming the session Conductor in conducted mode. |
| Non-Collapsing Capabilities List | Omitted |
| Application Capability List | Omitted |
| Expected Capability List | Omitted |

If the AVC Provider successfully enrolls in the chosen session, it shall then proceed to leave the session it was previously participating in as described in T.121.  It shall then generate an AVC-Session-Join confirm primitive with a successful result and issue it to the Control AVCSAP.  If a new AVC session has been created, the primitive shall also contain the AVC Session Id assigned to the session.

If the AVC Provider failed to enroll in the chosen session, it shall generate an AVC-Session-Join confirm primitive with a negative result, indicating that an unspecified failure occured, and issue it to the Control AVCSAP.

### Admitting Nodes to a Private AVC Session

On receipt of an AVC-Private-Session-Admit request primitive via the Control AVCSAP, an AVC Provider shall consult the application roster to determine whether the AVC session with the specified Session Id exists.  If not, it shall generate an AVC-Private-Session-Admit confirm primitive with a negative result, indicating that an invalid session id was supplied and issue it to the Control AVCSAP.  Otherwise, the AVC Provider shall consult its local database to determine whether its local node is the manager of the specified session.  If not, it shall generate an AVC-Private-Session-Admit confirm primitive with a negative result, indicating that the node is not session creator and issue it to the Control AVCSAP.  Otherwise it shall broadcast a PrivateSessionAdmitIndication AVCPDU to all nodes in the specified conference.  This is done by issuing an MCS-Send-Data request or MCS-Uniform-Send-Data request specifying the AVC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field.  Alternatively, the AVC Provider may issue one or more MCS-Send-Data request specifying the AVC User Id of one of the listed nodes as the Channel ID, also specifying High data priority, and including the PDU in the Data field.  In this case, it may leave the Invited Node List parameter in the PDU empty.

TABLE 82/T.132.

PrivateSessionAdmitIndication AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Session Id | Request | Indication |
| Invited Node List | Request | Destination AVC Providers |

On receipt of a PrivateSessionAdmitIndication PDU, an AVC Provider shall first determine if its local node is on the list of invited nodes.  If the list is either NULL, or if the local AVC User Id is present in the list, the AVC Provider shall generate an AVC-Private-Session-Admit indication primitive and issue it to the Control AVCSAP.  The Session Creator parameter is filled in from the Sender User ID field in the received MCS-Send-Data indication or MCS-Uniform-Send-Data indication.  If the local node is not on the list of invited nodes (if listed explicitly), no further action is taken.

### Expelling Nodes from a Private AVC Session

On receipt of an AVC-Private-Session-Expel request primitive via the Control AVCSAP, an AVC Provider shall consult the application roster to determine whether the AVC session with the specified Session Id exists.  If not, it shall generate an AVC-Private-Session-Expel confirm primitive with a negative result, indicating that an invalid session id was supplied and issue it to the Control AVCSAP.  Otherwise, the AVC Provider shall consult its local database to determine whether its local node is the manager of the specified session.  If not, it shall generate an AVC-Private-Session-Expel confirm primitive with a negative result, indicating that the node is not session creator and issue it to the Control AVCSAP.  Otherwise it shall broadcast a PrivateSessionExpelIndication AVCPDU to all nodes in the specified conference.  This is done by issuing an MCS-Send-Data request or MCS-Uniform-Send-Data request specifying the AVC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field.  Alternatively, the AVC Provider may issue one or more MCS-Send-Data request specifying the AVC User Id of one of the listed nodes  as the Channel ID, also specifying High data priority, and including the PDU in the Data field.  In this case, it may leave the Expelled Node List parameter in the PDU empty.

TABLE 83/T.132.

PrivateSessionExpelIndication AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Session Id | Request | Indication |
| Expelled Node List | Request | Destination AVC Providers |

On receipt of a PrivateSessionExpelIndication PDU, an AVC Provider shall first determine if its local node is on the list of nodes to be expelled.  If the list is either NULL, or if the local AVC User Id is present in the list, the AVC Provider shall enroll in the AVC Terminal Standard Base Session and shall leave the private session indicated in the PDU according to the procedures defined in T.121.  It shall then generate an AVC-Private-Session-Admit indication primitive and issue it to the Control AVCSAP.  The Session Creator parameter is filled in from the Sender User ID field in the received MCS-Send-Data indication or MCS-Uniform-Send-Data indication.  If the local node is not on the list of invited nodes (if listed explicitly), no further action is taken.

### Associating an AVC Session with Data Sessions

On receipt of an AVC-Session-Associate request primitive via the Control AVCSAP, an AVC Provider shall consult the application roster to determine whether the AVC session with the specified Session Id exists.  If not, it shall generate an AVC-Session-Associate confirm primitive with a negative result, indicating that an invalid session id was supplied and issue it to the Control AVCSAP.  Otherwise, the AVC Provider shall consult the application roster for the chosen session and determine whether it is a member of the session in the case of a Public session, or the manager of the session in the case of a private session.  If not, it shall generate an AVC-Private-Session-Associate confirm primitive with a negative result, indicating that the node is not authorised to define an association and issue it to the Control AVCSAP.  This primitive shall also be generated if the Session ID for the AVC Terminal Standard Base Session is specified.  (Editor’s note: What about the AVC NE Session?).

If the request is successfully validated, the AVC Provider shall examine the Associated Sessions parameter.  If present it shall issue a GCC-Registry-Set-Parameter request primitive to its local GCC Provider. The Registry Key is constructed by appending a Resource ID (constructed by encoding the 12 characters of the word “ASSOCIATIONS” into successive octets according to T.50) to the Session Key of the chosen AVC session.  If the Session is Private, the modification rights parameter shall be set to Owner, otherwise it shall be set to Session.  The structure of the registry parameter is TBD.

On receipt of the GCC-Application-Registry-Set-Parameter confirm primitive, the AVC Provider shall examine the result parameter and shall issue an AVC-Session-Associate confirm primitive to the Control AVCSAP with a corresponding result.

If the Associated Sessions parameter is absent, the AVC Provider shall attempt to remove the association entry from the GCC Registry for the selected AVC Session, by issuing a GCC-Registry-Delete-Entry to its local GCC Provider.  The Registry Key is constructed by appending a Resource ID (constructed by encoding the 12 characters of the word “ASSOCIATIONS” into successive octets according to T.50) to the Session Key of the chosen AVC session. The AVC Provider shall then issue an AVC-Session-Associate confirm primitive to the Control AVCSAP with a successful result.

### Impact on AVC Services

AVC Network Elements shall monitor the GCC application roster for nodes joining and leaving AVC Terminal sessions.  When a node joins a session it shall be subscribed to the active services within that session by the AVC Network Elements providing those services..  When a node leaves a session, it shall be removed from the services of that session.

## Activation and Termination of AVC Operation

### Activating AVC Control

On receipt of an AVC-Activate-Management request primitive via the Control AVCSAP, an AVC Provider shall generate an ActivateManagementRequest AVCPDU and send it to the Top AVC Provider.  This is done by issuing an MCS-Send-Data request request, specifying the AVC-Top-ProviderChannel as the Channel ID, specifying High data priority, and including the PDU in the Data field.

TABLE 84/T.132.

ActivateManagementRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| Convenor Password | Request | Indication |
| Conference Framework | Request | Top AVC Provider |

On receipt of an ActivateManagementRequest AVCPDU, the AVC Top Provider shall make a local copy of the conference framework and issue an AVC-Activate-Management indication primitive to the Control AVCSAP.

On receipt of an AVC-Activate-Management response primitive from the Control AVCSAP, the AVC Provider shall send an AVCActivateManagementResponse AVCPDU to the node that originated the request.  This is done by issuing an MCS-Send-Data request, specifying the AVC User ID of the originating AVC Provider as the Channel ID, specifying high data priority, and including the PDU in the Data field.  The AVC Provider shall also examine the result parameter in the response primitive and, if successful, shall generate an InitiateManagementIndication AVCPDU containing the conference framework from the ActivateManagementRequest AVCPDU and broadcast it to all Network Elements.  This is done by issuing an MCS-Send-Data request or MCS-Uniform-Send-Data request, specifying the AVC-NE-Communication Channel as the Channel ID, specifying high data priority, and including the PDU in the Data field.  Otherwise the AVC Provider shall delete the copy of the conference framework and take no further action.

On receipt of an ActivateManagementResponse AVCPDU, an AVC Provider shall generate an AVC-Activate-Management confirm primitive and issue it to the Control AVCSAP.

On receipt of an InitiateManagementIndication AVCPDU, an AVC Provider shall generate an AVC-Initiate-Management confirm primitive and issue it to the Control AVCSAP.  It shall also complete its enrolment in the AVC Network Element Session as described in §.

## Capability Reporting

### Overview

AVC assumes that the underlying NSCE performs the initial point-to-point capability exchange for a real time link. The Node Controller at the upper node of the real time link is responsible for notifying its local AVC Provider of the outcome of the capability exchange by issuing an AVC-Announce-Link-Capabilities request primitive.

Support for systems which do not have an underlying NSCE is for further study.

Each AVC Network Element shall maintain that subset of the Link and Node Rosters corresponding to the resources it owns.  They may optionally maintain a local copy of the Link and Node Rosters corresponding to resources owned by other AVC Network Elements.  This information is available from the owning AVC Network Element on request if required.

### Advertising Link Capabilities

On receipt of an AVC-Announce-Link-Capabilities request primitive via the Control AVCSAP, an AVC Provider shall examine the Link ID parameter and determine whether it is at the upper node of the real time link identified in the primitive.  If not, it shall generate an AVC-Announce-Link-Capabilities confirm primitive with a negative result, indicating that is not the upper node and issue it to the Control AVCSAP.  Otherwise, it shall update its local copy of the AVC Link Roster and send a RosterUpdateIndication AVCPDU to the AVC Network Elements.  If the AVC Management Mode is Distributed-Unrestricted or Distributed-By Network, the AVC Provider shall then broadcast a RosterUpdateIndication AVCPDU to all AVC Network Elements. This is done by issuing an MCS-Send-Data request or MCS-Uniform-Send-Data request, specifying the AVC-NE-Communication-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field.

If the AVC Management Mode is Centralised and the AVC Provider is not the Top AVC Provider for the specified conference, the AVC Provider shall then send a RosterUpdateIndication AVCPDU to notify the AVC Top Provider of the changes.  This is done by issuing an MCS-Send-Data request or MCS-Uniform-Send-Data request, specifying the AVC-Top-Provider Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field.

The AVC Provider shall then generate an AVC-Announce-Link-Capabilities confirm primitive indicating whether the requested operation was successful and issue it to the Control AVCSAP.

TABLE 85/T.132.

RosterUpdateIndication AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Updated Link Roster | Source AVC Provider | Destination AVC Providers |
| Updated Node Roster | Source AVC Provider | Destination AVC Providers |

On receipt of a RosterUpdateIndication AVCPDU, an AVC Provider shall update its copy of the Link and Node Rosters and issue an AVC-Roster-Report indication to the Control AVCSAP.

### Advertising Node Capabilities

On receipt of an AVC-Announce-Node-Capabilities request primitive via the Control AVCSAP, an AVC Provider shall update its local copy of the Node Roster if it is the Zone Manager of the node.  It shall then ascertain whether the conference is configured for centralised or distributed control.

If the AVC Management Mode is Distributed-Unrestricted or Distributed-By Network, the AVC Provider shall broadcast a RosterUpdateIndication AVCPDU to all AVC Network Elements.  This is done by issuing an MCS-Send-Data request or MCS-Uniform-Send-Data request, specifying the AVC-NE-Communication-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field.

If the AVC Management Mode is Centralised and the AVC Provider is not the Top AVC Provider for the specified conference, the AVC Provider shall send a RosterUpdateIndication AVCPDU to notify the AVC Top Provider of the node capabilities.  This is done by issuing an MCS-Send-Data request or MCS-Uniform-Send-Data request, specifying the AVC-Top-Provider Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field.

## Inheriting Existing Audio and Video Communications

### Inheriting an Existing Link Configuration

On receipt of an AVC-Announce-Link-Configuration request primitive via the Control AVCSAP, an AVC Provider shall examine the Link ID parameter and determine whether it is at the upper node of the real time link identified in the primitive.  If not, it shall generate an AVC-Announce-Link-Configuration confirm primitive with a negative result, indicating that is not the upper node and issue it to the Control AVCSAP.  Otherwise, it shall update its local copy of the AVC Link Roster and send a RosterUpdateIndication AVCPDU to the AVC Network Elements.  If the AVC Management Mode is Distributed-Unrestricted or Distributed-By Network, the AVC Provider shall then broadcast a RosterUpdateIndication AVCPDU to all AVC Network Elements. This is done by issuing an MCS-Send-Data request or MCS-Uniform-Send-Data request, specifying the AVC-NE-Communication-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field.

If the AVC Management Mode is Centralised and the AVC Provider is not the Top AVC Provider for the specified conference, the AVC Provider shall then send a RosterUpdateIndication AVCPDU to notify the AVC Top Provider of the changes.  This is done by issuing an MCS-Send-Data request or MCS-Uniform-Send-Data request, specifying the AVC-Top-Provider Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field.

### Inheriting an Existing Node Configuration

On receipt of an AVC-Announce-Node-Configuration request primitive via the Control AVCSAP, an AVC Provider shall update its local copy of the Node Roster if it is the Zone Manager of the node.  It shall then ascertain whether the conference is configured for centralised or distributed control.

If the AVC Management Mode is Distributed-Unrestricted or Distributed-By Network the AVC Provider shall broadcast an RosterUpdateIndication AVCPDU to notify all AVC Network Elements of the node configuration. This is done by issuing an MCS-Send-Data request or MCS-Uniform-Send-Data request, specifying the AVC-NE-Communication-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field.

If the AVC Management Mode is Centralised and the AVC Provider is not the Top AVC Provider for the specified conference, the AVC Provider shall send a RosterUpdateIndication AVCPDU to notify the AVC Top Provider of the node configuration.  This is done by issuing an MCS-Send-Data request or MCS-Uniform-Send-Data request, specifying the AVC-Top-Provider Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field.

## The Link and Node Rosters

Both the Link and Node Rosters are distributed amongst AVC Network Elements using the RosterUpdateIndication AVCPDU which can be used to send complete or partial roster information.  AVC Terminals submit information to the Link and Node Rosters, but, by default, do not receive copies of the rosters.

When a node joins a conference, it submits real time link and node configuration and capability information to the AVC Network Elements as a series of RosterReportIndication AVCPDUs.  Subsequent changes may be advertised by re-issuing a RosterUpdateIndication AVCPDU containing the revised information.

Each AVC Zone Manager is responsible for maintaining the subset of the Link and Node Rosters pertaining to the resources under its authority.  For resources owned by other AVC Zone Managers, the AVC Zone Manager shall as a minimum maintain a record of the resource identity and owner.  This enables it to retrieve the information from the appropriate AVC Zone Manager as required.  The Top AVC Provider shall maintain a complete copy of both Link and Node Rosters.

When a Network Element AVC Provider detects that a node has left the conference, it shall remove the node and all associated real time links from its local copy of the Node and Link Rosters respectively.

On receipt of a RosterUpdateIndication AVCPDU, an AVC Provider shall update its copy of the Link and Node Rosters and issue an AVC-Roster-Report indication primitive to the Control AVCSAP.

### Retrieving an Entry

On receipt of an AVC-Roster-Enquire request primitive, an AVC Provider shall examine the Zone Manager parameter.  If present, the AVC Provider shall send a RosterEnquireRequest AVCPDU to the Zone Manager indicated. This is done by issuing an MCS-Send-Data request, specifying the Zone Manager AVC User ID as the Channel ID, specifying High data priority, and including the PDU in the Data field.  If the Zone Manager parameter is absent, the AVC Provider shall broadcast a RosterEnquireRequest AVCPDU to all AVC Network Elements. This is done by issuing an MCS-Send-Data request or MCS-Uniform-Send-Data request, specifying the AVC-NE-Communication-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field.

TABLE 86/T.132.

RosterEnquireRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Destination AVC Provider |
| Roster Key | Request | Destination AVC Provider |
| Capability Flag | Request | Destination AVC Provider |
| Configuration Flag | Request | Destination AVC Provider |

On receipt of a RosterEnquireRequest AVCPDU on its own AVC User ID Channel, an AVC Provider shall examine the Roster Key.  If it is present and corresponds to resource(s) which the AVC Provider owns, the AVC Provider shall generate an appropriate list of roster entries.  If the Roster Key is absent, the AVC Provider shall generate a list of roster entries for all resources within its own zone.  In both cases, the AVC Provider shall take account of the setting of the Capability Flag and Configuration Flag, when assembling the list.  The AVC Provider returns the list of roster entries to the requesting node by sending a RosterEnquireResponse AVCPDU to the requesting node by issuing an MCS-Send-Data request, specifying the AVC User Id of  the requester as the Channel ID, specifying High data priority, and including the PDU in the Data  field.  The result parameter is specified as successful if the Roster Entry List contains at least one valid entry, otherwise it contains a negative result indicating the reason for failure.

On receipt of a RosterEnquireRequest AVCPDU on the AVC-NE-Communication-Channel, an AVC Provider shall first determine whether it is a Zone Manager.  If not, it shall ignore the request.  Otherwise, the AVC Provider shall examine the Roster Key.  If it is present and corresponds to resource(s) which the AVC Provider owns, the AVC Provider shall generate an appropriate list of roster entries.  If the Roster Key is absent, the AVC Provider shall generate a list of roster entries for all resources within its own zone.  In both cases, the AVC Provider shall take account of the setting of the Capability Flag and Configuration Flag, when assembling the list.  The AVC Provider returns the list of roster entries to the requesting node by sending a RosterEnquireResponse AVCPDU to the requesting node by issuing an MCS-Send-Data request, specifying the AVC User Id of  the requester as the Channel ID, specifying High data priority, and including the PDU in the Data  field.  The result parameter is specified as successful if the Roster Entry List contains at least one valid entry, otherwise it contains a negative result indicating the reason for failure.

TABLE 87/T.132.

RosterEnquireResponse AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Source AVC Provider | Confirm |
| Roster Key | Source AVC Provider | Confirm |
| List of Roster Entries | Source AVC Provider | Confirm |
| Result | Source AVC Provider | Confirm |

On receipt of a RosterEnquireResponse AVCPDU, an AVC Provider shall generate an AVC-Roster-Enquire confirm primitive containing the roster list included in the PDU and issue it to the Control AVCSAP.

### Monitoring an Entry

On receipt of an AVC-Roster-Monitor request primitive, an AVC Provider shall a RosterMonitorRequest AVCPDU to the Zone Manager indicated by issuing an MCS-Send-Data request, specifying the Zone Manager AVC User ID as the Channel ID, specifying High data priority, and including the PDU in the Data field.  The content of the RosterMonitorRequest AVCPDU is shown in Table 88.

TABLE 88/T.132.

RosterMonitorRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Destination AVC Provider |
| Roster Key | Request | Destination AVC Provider |
| Capability Flag | Request | Destination AVC Provider |
| Configuration Flag | Request | Destination AVC Provider |
| Media Type | Request | Destination AVC Provider |

On receipt of a RosterMonitorRequest AVCPDU, an AVC Provider shall attempt to change the monitoring state of any roster entries associated with the Roster Key included in the PDU.

If the Roster Key includes an entity id, the AVC Provider shall check whether there is a roster entry corresponding to this key and shall set the monitoring state of this entry for the requesting node in accordance with the Enable/Disable Flag.

If the roster key does not include an entity id, the request applies to all roster entries of the specified entity type.  If the Enable/Disable Flag is set to Disable, the AVC Provider shall disable monitoring of all entities of the specified entity type for the requesting node, regardless of whether or how monitoring had previously been enabled for such entries.  If the Enable/Disable Flag is set to Enable, the AVC Provider shall enable monitoring for all entities of the specified entity type for the requesting node.  This applies to roster entries created subsequent to the processing of this request (Editor’s Note : Is this unnecessarily complex ?)

A change in the content of any roster entry being monitored shall cause the AVC Provider to generate an RosterReportIndication AVCPDU and issue it to the appropriate monitoring node(s).  For each node monitoring the registry entry that has changed, the AVC Provider issues an MCS-Send-Data request, specifying the AVC User ID of the monitoring node as the Channel ID and including the PDU in the Data Field.  Changes to multiple roster entries may be aggregated into a single PDU.

## Resource Configuration

### Configuring a Link Multiplex

On receipt of an AVC-Set-Link-Multiplex request primitive, an AVC Provider shall consult the Link ID parameter and determine whether it is the Zone Manager of the real time link identified in the primitive.  If not, it shall generate an AVC-Set-Link-Multiplex confirm primitive with a negative result indicating that  the node is not the Zone Manager of the link and issue it to the Control AVCSAP.

Otherwise the AVC Provider shall send an SetLinkMultiplexRequest AVCPDU to the node which is required to modify its outgoing multiplex.  If the Direction parameter specified in the AVC-Set-Link-Multiplex request primitive is Up, then the Lower Node of the Real Time Link is required to modify it outgoing multiplex; if the Direction is Down the Upper Node of the Real Time Link is required to modify its outgoing multiplex.  In either case, the AVC Provider extracts the appropriate destination node AVC User ID from the Real Time Link Id parameter in the request primitive and issues an MCS-Send-Data request to this node, specifying High data priority, and including the PDU in the Data field.

TABLE 89/T.132.

SetLinkMultiplexRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| Real Time Link Id | Request | Indication |
| Multiplex Configuration Table | Request | Indication |

On receipt of a SetLinkMultiplexRequest AVCPDU, an AVC Provider shall generate an AVC-Set-Link-Multiplex indication primitive and issue it to the Control AVCSAP.

It is the responsibility of the Node Controller to interact with the appropriate local NSCE to configure the multiplex.  It is assumed that an NSCE is responsible for configuring the outgoing multiplex of its real time link(s) and notifying other link endpoints which then set their incoming multiplexes.  The Node Controller shall then generate an AVC-Set-Link-Multiplex response primitive indicating whether or not the operation succeeded and issue it to the Control AVCSAP.

On receipt of an AVC-Set-Link-Multiplex response primitive from the Control AVCSAP, the AVC Provider shall send a SetLinkMultiplexResponse AVCPDU to the node that originated the request.  This is done by issuing an MCS-Send-Data request, specifying the AVC User ID of the originating AVC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field.

TABLE 90/T.132.

SetLinkMultiplexResponse AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Response | Confirm |
| Result | Response | Confirm |

On receipt of a SetLinkMultiplexResponse AVCPDU, an AVC Provider shall generate an AVC-Set-Link-Multiplex confirm primitive indicating whether the requested operation was successful and issue it to the Control AVCSAP.

### Configuring the Media Streams Transported by a Link

On receipt of an AVC-Set-Link-Transmission-Mode request primitive, an AVC Provider shall consult the Link ID parameter and determine whether it is the Zone Manager of the real time link identified in the primitive.  If not, it shall generate an AVC-Set-Link-Transmission-Mode confirm primitive with a negative result indicating that the node is not the Zone Manager of the link and issue it to the Control AVCSAP.

Otherwise the AVC Provider shall send an SetLinkTransmissionModeRequest AVCPDU to the node which is required to modify its outgoing media streams.  If the Direction parameter specified in the AVC-Set-Link-Transmission-Mode request primitive is Up, then the Lower Node of the Real Time Link is required to modify it outgoing media streams; if the Direction is Down the Upper Node of the Real Time Link is required to modify its outgoing media streams.  In either case, the AVC Provider extracts the appropriate destination node AVC User ID from the Real Time Link Id parameter in the request primitive and issues an MCS-Send-Data request to this node, specifying High data priority, and including the PDU in the Data field.

TABLE 91/T.132.

SetLinkTransmissionModeRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| Real Time Link Id | Request | Indication |
| Media Configuration Table | Request | Indication |

On receipt of a SetLinkTransmissionModeRequest AVCPDU, an AVC Provider shall generate an AVC-Set-Link-Transmission-Mode indication primitive and issue it to the Control AVCSAP.

It is the responsibility of the Node Controller to interact with the appropriate local NSCE to configure the outgoing media streams. The Node Controller shall then generate an AVC-Set-Link-Transmission-Mode response primitive indicating whether or not the operation succeeded and issue it to the Control AVCSAP.

On receipt of an AVC-Set-Link-Transmission-Mode response primitive from the Control AVCSAP, the AVC Provider shall send a SetLinkTransmissionModeResponse AVCPDU to the node that originated the request.  This is done by issuing an MCS-Send-Data request, specifying the AVC User ID of the originating AVC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field.

TABLE 92/T.132.

SetLinkTransmissionModeResponse AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Response | Confirm |
| Result | Response | Confirm |

On receipt of a SetLinkTransmissionModeResponse AVCPDU, an AVC Provider shall generate an AVC-Set-Link-Transmission-Mode confirm primitive indicating whether the requested operation was successful and issue it to the Control AVCSAP.

### Configuring a Node

On receipt of an AVC-Set-Node-Configuration request primitive, an AVC Provider shall first ascertain whether it is the Zone Manager of the Destination Node.  If not, it shall generate an AVC-Set-Node-Configuration confirm primitive with a negative result, indicating that the node does not have the appropriate authority and issue it to the Control AVCSAP.  Otherwise, the AVC Provider shall send a SetNodeConfiguration AVCPDU to the Destination Node by issuing an MCS-Send-Data request, specifying the AVC User ID of the destination node as the Channel ID, specifying High data priority, and including the PDU in the Data field.

On receipt of a SetNodeConfiguration AVCPDU, an AVC Provider shall generate an AVC-Set-Node-Configuration indication and issue it to the Control AVCSAP.

It is the responsibility of the Node Controller to implement the node configuration contained in the AVC-Set-Node-Configuration indication primitive and to issue an AVC-Set-Node-Configuration response primitive, indicating whether or not the operation succeeded.   If the bridge configuration table within the indication primitive contains any invalid parameters, the node configuration shall remain unchanged and the Node Controller shall identify the reason for failure in the response primitive.

On receipt of an AVC-Set-Node-Configuration response primitive from the Control AVCSAP, the AVC Provider shall send a SetNodeConfigurationResponse AVCPDU to the node that originated the request.  This is done by issuing an MCS-Send-Data request, specifying the AVC User ID of the originating AVC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field.

On receipt of a SetNodeConfigurationResponse AVCPDU, an AVC Provider shall generate an AVC-Set-Node-Configuration confirm primitive indicating whether the requested operation was successful and issue it to the Control AVCSAP.

TABLE 93/T.132.

SetNodeConfigurationRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| Bridge Configuration Table | Request | Indication |

TABLE 94/T.132.

SetNodeConfigurationResponse AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Response | Confirm |
| Result | Response | Confirm |

### Opening a Logical Channel

On receipt of an AVC-Open-Logical-Channel request primitive, an AVC Provider shall consult the Link ID parameter and determine whether it is the Zone Manager of the real time link identified in the primitive.  If not, it shall generate an AVC-Open-Logical-Channel confirm primitive with a negative result indicating the node is not the Zone Manager of the link and issue it to the Control AVCSAP.

Otherwise the AVC Provider shall send an OpenLogicalChannelRequest AVCPDU to the node which is required to open the logical channel.  If the Direction parameter specified in the AVC-Open-Logical-Channel request primitive is Up, then the Lower Node of the Real Time Link is required to open the logical channel; if the Direction is Down the Upper Node of the Real Time Link is required to open the logical channel.  In either case, the AVC Provider extracts the appropriate destination node AVC User ID from the Real Time Link Id parameter in the request primitive and issues an MCS-Send-Data request to this node, specifying High data priority, and including the PDU in the Data field.

TABLE 95/T.132.

OpenLogicalChannelRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| Real Time Link Id | Request | Indication |
| Direction | Request | Indication |
| Path Id | Request | Indication |
| Media Type | Request | Indication |
| Multiplex Parameters | Request | Indication |
| Logical Channel QoS | Request | Indication |
| Associated Logical Channels | Request | Indication |

On receipt of an OpenLogicalChannelRequest AVCPDU, an AVC Provider shall generate an AVC-Open-Logical-Channel indication primitive and issue it to the Control AVCSAP.  It is the responsibility of the Node Controller to interact with the appropriate local NSCE to open the logical channel.  The Node Controller shall then generate an AVC-Open-Logical-Channel response primitive indicating whether or not the operation succeeded and issue it to the Control AVCSAP.

On receipt of an AVC-Open-Logical-Channel response primitive, the AVC Provider shall send an OpenLogicalChannelResponse AVCPDU to the node that originated the request.  This is done by issuing an MCS-Send-Data request, specifying the AVC User ID of the originating AVC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field.

TABLE 96/T.132.

OpenLogicalChannelResponse AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Response | Confirm |
| Logical Channel Number | Response | Confirm |
| Logical Channel QoS | Response | Confirm |
| Result | Response | Confirm |

On receipt of an OpenLogicalChannelResponse AVCPDU, an AVC Provider shall generate an AVC-Open-Logical-Channel confirm primitive indicating whether the requested operation was successful and issue it to the Control AVCSAP.

### Closing a Logical Channel

On receipt of an AVC-Close-Logical-Channel request primitive, an AVC Provider shall consult the Link ID parameter and determine whether it is the Zone Manager of the real time link identified in the primitive.  If not, it shall generate an AVC-Close-Logical-Channel confirm primitive with a negative result indicating that the node is not the Zone Manager of the link and issue it to the Control AVCSAP.  Otherwise, the AVC Provider shall consult its Link Roster to determine whether the logical channel identified in the primitive exists.  If not, it shall generate an AVC-Close-Logical-Channel confirm primitive with a negative result indicating that an invalid logical channel ID was provided and issue it to the Control AVCSAP

If both these criteria are met, the AVC Provider shall send a CloseLogicalChannelRequest AVCPDU to the node which is required to close the logical channel.  If the Direction parameter specified in the AVC-Close-Logical-Channel request primitive is Up, then the Lower Node of the Real Time Link is required to close the logical channel; if the Direction is Down the Upper Node of the Real Time Link is required to close the logical channel.  In either case, the AVC Provider extracts the appropriate destination node AVC User ID from the Real Time Link Id parameter in the request primitive and issues an MCS-Send-Data request to this node, specifying High data priority, and including the PDU in the Data field.

TABLE 97/T.132.

CloseLogicalChannelRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| Real Time Link Id | Request | Indication |
| Direction | Request | Indication |
| Logical Channel Number | Request | Indication |

On receipt of a CloseLogicalChannelRequest AVCPDU, an AVC Provider shall generate an AVC-Close-Logical-Channel indication primitive and issue it to the Control AVCSAP.  It is the responsibility of the Node Controller to interact with the appropriate local NSCE to close the logical channel.  The Node Controller shall then generate an AVC-Close-Logical-Channel response primitive indicating whether or not the operation succeeded and issue it to the Control AVCSAP.

On receipt of an AVC-Close-Logical-Channel response primitive, the AVC Provider shall send an CloseLogicalChannelResponse AVCPDU to the node that originated the request.  This is done by issuing an MCS-Send-Data request, specifying the AVC User ID of the originating AVC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field.

TABLE 98/T.132.

CloseLogicalChannelResponse AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Response | Confirm |
| Result | Response | Confirm |

On receipt of a CloseLogicalChannelResponse AVCPDU, an AVC Provider shall generate an AVC-Close-Logical-Channel confirm primitive indicating whether the requested operation was successful and issue it to the Control AVCSAP.

## Resource Arbitration

### Reserving Resources

On receipt of an AVC-Reserve-Resource request primitive, an AVC Provider shall consult its local database to determine whether it is a Zone Manager.  If not, it shall generate an AVC-Reserve-Resource confirm primitive with a negative result, indicating that it is not a Zone Manager and issue it to the Control AVCSAP.  Otherwise, the AVC Provider shall examine the Destination Node parameter and verify that it corresponds to a Zone Manager.  If this is not the case, it shall generate an AVC-Reserve-Resource confirm primitive with a negative result, indicating that an Invalid Destination Node ID was provided and issue it to the Control AVCSAP.

If all these criteria are met the AVC Provider shall send a ReserveResourceRequest AVCPDU to the Destination Node by issuing an MCS-Send-Data request, specifying High data priority, and including the PDU in the Data field.

TABLE 99/T.132.

ReserveResourceRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| Resource Descriptor | Request | Indication |

On receipt of a ReserveResourceRequest AVCPDU, an AVC Provider shall generate an AVC-Reserve-Resource indication primitive and issue it to the Control AVCSAP.  It is the responsibility of the Node Controller to process the request and to respond, indicating whether or not the request can be fulfilled.  How this is determined is a local matter.  The Node Controller should not permit reservation of resources that are in use, or are already reserved.  The exception to this when a reservation is not confirmed within a reasonable period of time.  In this instance, the Node Controller may allow the reservation to lapse and release the associated resources.  The period of time after which this may occur is a local matter.

On receipt of an AVC-Reserve-Resource response primitive, the AVC Provider shall send a ReserveResourceResponse AVCPDU to the node that originated the request.  This is done by issuing an MCS-Send-Data request, specifying the AVC User ID of the originating AVC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field.

TABLE 100/T.132.

ReserveResourceResponse AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Response | Confirm |
| Reservation Handle | Response | Confirm |
| QoS Level | Response | Confirm |
| Result | Response | Confirm |

On receipt of a ReserveResourceResponse AVCPDU, an AVC Provider shall generate an AVC-Reserve-Resource confirm primitive indicating the result included in the PDU and issue it to the Control AVCSAP.  If the result is successful, the AVC Provider shall store the Reservation Handle and the AVC User Id of the Zone Manager that assigned it in order to validate subsequent requests relating to the reservation.

### Cancelling a Reservation

On receipt of an AVC-Cancel-Resource-Reservation request primitive, an AVC Provider shall consult its local database to determine whether it is a Zone Manager.  If not, it shall generate an AVC-Cancel-Resource-Reservation confirm primitive with a negative result, indicating that it is not a Zone Manager and issue it to the Control AVCSAP.  Otherwise, the AVC Provider shall examine the Destination Node parameter and Reservation Handle and verify that the pairing corresponds to a valid previous reservation transaction.  If this is not the case, it shall generate an AVC-Cancel-Resource-Reservation confirm primitive with a negative result, indicating that an Invalid Reservation was provided and issue it to the Control AVCSAP.

If all these criteria are met the AVC Provider shall send a CancelResourceReservationRequest AVCPDU to the Destination Node by issuing an MCS-Send-Data request, specifying High data priority, and including the PDU in the Data field.  It shall also delete any locally held information relating to the reservation.

TABLE 101/T.132.

CancelResourceReservationRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| Reservation Handle | Request | Indication |

On receipt of a CancelResourceReservationRequest AVCPDU, an AVC Provider shall generate an AVC-Cancel-Resource-Reservation indication primitive and issue it to the Control AVCSAP.  It is the responsibility of the Node Controller to release any resources associated with the reservation.

### Confirming a Reservation

On receipt of an AVC-Confirm-Resource-Reservation request primitive, an AVC Provider shall consult its local database to determine whether it is a Zone Manager.  If not, it shall generate an AVC-Confirm-Resource-Reservation confirm primitive with a negative result, indicating that it is not a Zone Manager and issue it to the Control AVCSAP.  Otherwise, the AVC Provider shall examine the Destination Node parameter and Reservation Handle and verify that the pairing corresponds to a valid previous reservation transaction.  If this is not the case, it shall generate an AVC-Confirm-Resource-Reservation confirm primitive with a negative result, indicating that an Invalid Reservation was provided and issue it to the Control AVCSAP.

If all these criteria are met the AVC Provider shall send a ConfirmResourceReservationRequest AVCPDU to the Destination Node by issuing an MCS-Send-Data request, specifying High data priority, and including the PDU in the Data field.

TABLE 102/T.132.

ConfirmResourceReservationRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| Reservation Handle | Request | Indication |

On receipt of a ConfirmResourceReservationRequest AVCPDU, an AVC Provider shall generate an AVC-Confirm-Resource-Reservation indication primitive and issue it to the Control AVCSAP.  It is the responsibility of the Node Controller to arrange for the appropriate resources to be configured to support the request and then to issue an AVC-Confirm-Resource-Reservation response primitive, indicating whether or not the operation was successful.

On receipt of an AVC-Confirm-Resource-Reservation response primitive, the AVC Provider shall send a ConfirmResourceReservationResponse AVCPDU to the node that originated the request.  This is done by issuing an MCS-Send-Data request, specifying the AVC User ID of the originating AVC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field.

TABLE 103/T.132.

ConfirmResourceReservationResponse AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Response | Confirm |
| Result | Response | Confirm |

On receipt of a ConfirmResourceReservationResponse AVCPDU, an AVC Provider shall generate an AVC-Confirm-Resource-Reservation confirm primitive indicating the result included in the PDU and issue it to the Control AVCSAP.

### Relinquishing a Reservation

On receipt of an AVC-Relinquish-Resource-Reservation request primitive, an AVC Provider shall consult its local database to determine whether it is a Zone Manager.  If not, it shall generate an AVC-Relinquish-Resource-Reservation confirm primitive with a negative result, indicating that it is not a Zone Manager and issue it to the Control AVCSAP.  Otherwise, the AVC Provider shall examine the Destination Node parameter and Reservation Handle and verify that the pairing corresponds to a valid previous reservation transaction.  If this is not the case, it shall generate an AVC-Relinquish-Resource-Reservation confirm primitive with a negative result, indicating that an Invalid Reservation was provided and issue it to the Control AVCSAP.

If all these criteria are met the AVC Provider shall send a RelinquishResourceReservationRequest AVCPDU to the Destination Node by issuing an MCS-Send-Data request, specifying High data priority, and including the PDU in the Data field.  It shall also delete any locally held information relating to the reservation.

TABLE 104/T.132.

RelinquishResourceReservationRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| Reservation Handle | Request | Indication |

On receipt of a RelinquishResourceReservationRequest AVCPDU, an AVC Provider shall generate an AVC-Relinquish-Resource-Reservation indication primitive and issue it to the Control AVCSAP.  It is the responsibility of the Node Controller to release any resources associated with the reservation.  The Node Controller may optionally choose to reconfigure the AVC infrastructure to allow reuse of these resources.  This is left as a local matter.

# AVC Service Management Protocol Definition

## AVC Service Access

Zone Managers may offer AVC services to their respective terminals following receipt of an InitiateManagement AVCPDU from the Top AVC Provider.  AVC services may only be offered to terminals that have enrolled in the AVC Terminal Standard Base Session.  It may thus be necessary for the Node Controller at a Zone Manager node to use the GCC-Application-Invoke mechanism to prompt terminals within its zone that have indicated their support for AVC (by enrolling in the AVC  Registration Session) to enroll in the AVC Terminal Standard Base Session.  This is done by issuing a GCC-Application-Invoke request primitive, specifying the Session Key of the AVC Terminal Standard Base Session and no capabilities in the Application Protocol Entity Invoke List, and providing in the Destination Nodes parameter a list of Node Ids corresponding to the terminals within the Zone that are required to enroll in this session.

Terminals that do not enroll in the AVC Terminal Standard Base Session may still have access to AVC services through a proxy mechanism.  The definition of such a protocol is outside the scope of this recommendation.

(Editor’s Note:  Alternatively, the Top AVC Provider could issue a GCC-Application-Invoke request  inviting all nodes to enroll in the AVC Terminal Standard Base Session coincident with broadcasting the InitiateManagement AVCPDU.  This has the disadvantage of generating potentially unnecessary network traffic).

### Offering AVC Services to Terminals

On receipt of an AVC-Service-Offer request primitive, an AVC Provider shall consult its local database to determine whether it is a Zone Manager.  If not, it shall generate an AVC-Service-Offer confirm primitive with a negative result, indicating that it is not a Zone Manager and issue it to the Control AVCSAP.  Otherwise, the AVC Provider shall verify that it may offer AVC services (InitiateManagement AVCPDU received from the Top AVC Provider).  If not, it shall generate an AVC-Service-Offer confirm primitive with a negative result, indicating that AVC management is not active and issue it to the Control AVCSAP.  Otherwise, it shall examine the Destination Node parameter and verify that it corresponds to a node within its own Zone.  If this is not the case, it shall generate an AVC-Service-Offer confirm primitive with a negative result, indicating that an Invalid Destination Node ID was provided and issue it to the Control AVCSAP.

If all these criteria are met the AVC Provider shall send a ServiceOfferRequest AVCPDU to the Destination Node by issuing an MCS-Send-Data request, specifying High data priority, and including the PDU in the Data field. It shall then generate an AVC-Service-Offer confirm primitive with a positive result and issue it to the Control AVCSAP.

TABLE 105/T.132.

ServiceOfferRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| AVC Active Service List | Request | Indication |
| AVC Inactive Service List | Request | Indication |

On receipt of a ServiceOfferRequest AVCPDU, an AVC Provider shall generate an AVC-Service-Offer indication primitive and issue it to the Control AVCSAP.

### Subscribing to AVC Services

On receipt of an AVC-Service-Subscribe request primitive, an AVC Provider shall consult its local copy of the GCC application roster to determine whether the indicated AVC session exists.  If not, it shall generate an AVC-Service-Subscribe confirm primitive with a negative result, indicating that it an invalid AVC session id was provided and issue it to the Control AVCSAP.  Otherwise, it shall send a ServiceSubscribeRequest AVCPDU to its Zone Manager by issuing an MCS-Send-Data request, specifying the AVC User Id of the Zone Manager as the Channel Id, specifying High data priority, and including the PDU in the Data field.

On receipt of a ServiceSubscribeRequest AVCPDU, an AVC Provider shall consult the conference framework to determine whether the requested service is available and if the node issuing the request is authorised to subscribe to the service.  If these criteria are not met, it shall send an ServiceSubscribeResponse AVCPDU to the node that originated the request, indicating the reason for rejection.  This is done by issuing an MCS-Send-Data request, specifying the AVC User ID of the originating AVC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field.  Otherwise, the AVC Provider shall generate an AVC-Service-Subscribe indication and issue it to the Control AVCSAP and shall send a ServiceSubscribeResponse AVCPDU to the node that originated the request, indicating that the subscription was successful.

If the AVC Service is not configured for automatic subscription, the AVC Provider shall then broadcast a ServiceSubscribeReportIndication AVCPDU to notify all AVC Network Elements of the change in service subscription.  This is done by issuing an MCS-Send-Data request or MCS-Uniform-Send-Data request, specifying the AVC-NE-Control-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field.

TABLE 106/T.132.

ServiceSubscribeRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| AVC Service | Request | Indication |
| AVC Session ID | Request | Indication |
| Subscribe/Unsubscribe Flag | Request | Indication |

TABLE 107/T.132.

ServiceSubscribeResponse AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Response | Confirm |
| Result | Response | Confirm |

TABLE 108/T.132.

ServiceSubscribeReportIndication AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| AVC User ID | ServiceSubscribeRequest PDU | Indication |
| AVC Service | ServiceSubscribeRequest PDU | Indication |
| AVC Session ID | ServiceSubscribeRequest PDU | Indication |
| Subscribe/Unsubscribe Flag | ServiceSubscribeRequest PDU | Indication |

On receipt of a ServiceSubscribeResponse AVCPDU, an AVC Provider shall generate an AVC-Service-Subscribe confirm primitive indicating the result included in the PDU and issue it to the Control AVCSAP.

### Activating AVC Services

On receipt of an AVC-Service-Activate request primitive, an AVC Provider shall consult its local copy of the GCC application roster to determine whether the indicated AVC session exists.  If not, it shall generate an AVC-Service-Activate confirm primitive with a negative result, indicating that it an invalid AVC session id was provided and issue it to the Control AVCSAP.  Otherwise, it shall send a ServiceActivateRequest AVCPDU to the Top AVC Provider by issuing an MCS-Send-Data request, specifying the AVC-Top-Provider Channel as the Channel Id, specifying High data priority, and including the PDU in the Data field. (Editor’s Note: The local AVC Provider could validate the selected service and authorisation. This would increase the complexity of AVC Terminal implementations, but reduce network traffic for invalid requests.)

On receipt of a ServiceActivateRequest AVCPDU, the Top AVC Provider shall consult the conference framework to determine whether the requested service is available and if the node issuing the request is authorised to launch the service.  If these criteria are not met, it shall send an ServiceActivateResponse AVCPDU to the node that originated the request, indicating the reason for rejection.  This is done by issuing an MCS-Send-Data request, specifying the AVC User ID of the originating AVC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field.  Otherwise, the AVC Provider shall generate an AVC-Service-Activate indication and issue it to the Control AVCSAP.

TABLE 109/T.132.

ServiceActivateRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| AVC Service | Request | Indication |
| AVC Session ID | Request | Indication |
| Activate/De-activate Flag | Request | Indication |

On receipt of an AVC-Service-Activate response primitive, the AVC Provider shall send an ServiceActivateResponse AVCPDU to the node that originated the request.  This is done by issuing an MCS-Send-Data request, specifying the AVC User ID of the originating AVC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field.

TABLE 110/T.132.

ServiceActivateResponse AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Response | Confirm |
| Result | Response | Confirm |

On receipt of a ServiceActivateResponse AVCPDU, an AVC Provider shall generate an AVC-Service-Activate confirm primitive indicating the result included in the PDU and issue it to the Control AVCSAP.

### Launching AVC Services

On receipt of an AVC-Service-Launch request primitive, an AVC Provider shall consult its local database to determine whether it is the Top AVC Provider.  If not, it shall generate an AVC-Service-Launch confirm primitive with a negative result, indicating that it is not the Top AVC Provider and issue it to the Control AVCSAP.  Otherwise, it shall broadcast a ServiceLaunchRequest AVCPDU to all AVC Network Elements.   This is done by issuing an MCS-Send-Data request or MCS-Uniform-Send-Data request specifying the AVC-NE-Control-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field.  Alternatively, the AVC Provider may issue one or more MCS-Send-Data request specifying the Node ID of one of the listed destination nodes as the Channel ID, also specifying High data priority, and including the PDU in the Data field.  In this case, it may leave the Destination Node List parameter in the PDU empty.

TABLE 111/T.132.

ServiceLaunchRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| AVC Service | Request | Indication |
| AVC Session ID | Request | Indication |
| Activate/De-activate Flag | Request | Indication |

On receipt of a ServiceLaunchRequest AVCPDU, an AVC Provider shall generate an AVC-Service-Offer indication primitive and issue it to the Control AVCSAP.  The Node Controller shall attempt to configure its node to support the service indicated in the PDU and shall then generate an AVC-Service-Launch response primitive indicating whether or not the configuration was successful and issue it to the Control AVCSAP.

On receipt of an AVC-Service-Launch response primitive, the AVC Provider shall send an ServiceLaunchResponse AVCPDU to the Top AVC Provider.  This is done by issuing an MCS-Send-Data request, specifying the AVC-Top-Provider Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field.

TABLE 112/T.132.

ServiceLaunchResponse AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Response | Confirm |
| Result | Response | Confirm |

On receipt of a ServiceLaunchResponse AVCPDU, the Top AVC Provider shall generate an AVC-Service-Launch confirm primitive indicating the result included in the PDU and issue it to the Control AVCSAP.

### Reporting the Launch of AVC Services

On receipt of an AVC-Service-Launch-Report request primitive, an AVC Provider shall consult its local database to determine whether it is the Top AVC Provider.  If not, it shall generate an AVC-Service-Launch-Report confirm primitive with a negative result, indicating that it is not the Top AVC Provider and issue it to the Control AVCSAP.  Otherwise, it shall examine the Destination Nodes parameter.  If this is NULL, it shall broadcast a ServiceLaunchReportIndication AVCPDU to all AVC Network Elements.   This is done by issuing an MCS-Send-Data request specifying the AVC-NE-Control-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field.  Otherwise, the AVC Provider shall send a ServiceLaunchReportIndication AVCPDU to the selected AVC Network Element.   This is done by issuing an MCS-Send-Data request specifying the AVC User ID of the selected AVC Network Element as the Channel ID, specifying High data priority, and including the PDU in the Data field.

TABLE 113/T.132.

ServiceLaunchReportIndication AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| AVC Service Table | Request | Indication |
| Full Refresh Flag | Request | Indication |

On receipt of a ServiceLaunchReportIndication AVCPDU, an AVC Provider shall generate an AVC-Service-Report indication primitive and issue it to the Control AVCSAP.

## Multipoint Channel Management

### Creating a Multipoint Channel

On receipt of an AVC-Create-Multipoint-Channel request primitive, an AVC Provider shall send a CreateMultipointChannelRequest AVCPDU to its Zone Manager by issuing an MCS-Send-Data request, specifying the AVC User Id of the Zone Manager as the Channel Id, specifying High data priority, and including the PDU in the Data field.

On receipt of a CreateMultipointChannelRequest AVCPDU, an AVC Provider shall consult the conference framework to determine whether the node issuing the request is authorised to use the service.  If not, it shall send a CreateMultipointChannelResponse AVCPDU to the node that originated the request, indicating the reason for rejection.  This is done by issuing an MCS-Send-Data request, specifying the AVC User ID of the originating AVC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field.  Otherwise, the AVC Provider shall generate an AVC-Create-Multipoint-Channel indication and issue it to the Control AVCSAP.

It is the responsibility of the Node Controller to assess the request and to co-ordinate with other Zone Managers, as appropriate, to create the multipoint channel.  The Node Controller shall then generate an AVC-Create-Multipoint-Channel response primitive, indicating whether or not the operation succeeded and issue it to the Control AVCSAP.

On receipt of an AVC-Create-Multipoint-Channel response, the AVC Provider shall send a CreateMultipointChannel Response AVCPDU to the node that originated the request.  This is done by issuing an MCS-Send-Data request, specifying the AVC User ID of the originating AVC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field.  If the result is successful, the AVC Provider shall determine and store the state of Modification Rights for the multipoint channel.  If the Modification Rights parameter was included in the request PDU, the Modification Type is set to the indicated value, otherwise, the value Public is assumed.  If the result is successful, the AVC Provider shall also generate a MultipointChannelStatusIndication AVCPDU with the Status parameter set to New and including the AVC User ID of the source node and all destination nodes of the new multipoint channel.  The AVC Provider shall send this  PDU  to all nodes joined to the new channel.   For each of these nodes, this is done by issuing an MCS-Send-Data request, specifying the AVC User ID of the node as the Channel ID, specifying High data priority, and including the PDU in the Data field.

TABLE 114/T.132.

CreateMultipointChannelRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| Source Node | Request | Indication |
| Destination Node List | Request | Indication |
| Media Type | Request | Indication |
| Session Identifier | Request | Indication |
| Modification Rights | Request | Indication |
| QoS Parameters | Request | Indication |
| Synchronised Multipoint Channels | Request | Indication |

TABLE 115/T.132.

CreateMultipointChannelResponse AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Response | Confirm |
| Multipoint Channel Identifier | Response | Confirm |
| QoS Parameters | Response | Confirm |
| QoS Level | Response | Confirm |
| Result | Response | Confirm |

On receipt of a CreateMultipointChannelResponse AVCPDU, an AVC Provider shall generate an AVC-Create-Multipoint-Channel confirm primitive indicating the result included in the PDU and issue it to the Control AVCSAP.

### Modifying a Multipoint Channel

On receipt of an AVC-Modify-Multipoint-Channel request primitive, an AVC Provider shall send a ModifyMultipointChannelRequest AVCPDU to its Zone Manager by issuing an MCS-Send-Data request, specifying the AVC User ID of the Zone Manager as the Channel ID, specifying High data priority, and including the PDU in the Data field.

On receipt of a ModifyMultipointChannelRequest AVCPDU, an AVC Provider shall consult the conference framework to determine whether the node issuing the request is authorised to use the service.  If so, it shall verify that the multipoint channel exists and shall check the modification rights to determine whether the requesting node is authorised to modify the multipoint channel.

The determination of whether the requesting node has modification rights is by the following rules:  If the current Modification Rights attribute of the multipoint channel is set to Owner, the requester, as indicated by the Entity ID and the Node ID from the Sender User ID field of the received MCS-Send-Data indication, must be the owner or the multipoint channel must currently be un-owned for the request to succeed. If the current Modification Rights attribute of the entry is set to Session, the requester must be part of the same AVC Terminal Session for this request to succeed.  This is determined by finding the entry in the current Application Roster for the requester and examining the Session Key.  The Session Key must be identical to that of the owner.  Again, if the entry is not owned, this restriction does not apply.  If the current Modification Rights attribute of the multipoint channel is set to Public, there are no restrictions  for the request to succeed.

If any of these criteria are not met, it shall send a ModifyMultipointChannelResponse AVCPDU to the node that originated the request, indicating the reason for rejection.  This is done by issuing an MCS-Send-Data request, specifying the AVC User ID of the originating AVC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field.  Otherwise, the AVC Provider shall generate an AVC-Modify-Multipoint-Channel indication and issue it to the Control AVCSAP.  If the requester is the current owner of the multipint channel, or if the channel is currently unowned, the AVC Provider shall change the Modification Rights state to that indicated in the request PDU, if present.  Otherwise, the Modification Rights state is not changed.

It is the responsibility of the Node Controller to assess the request and to co-ordinate with other Zone Managers, as appropriate, to modify the multipoint channel.  The Node Controller shall then generate an AVC-Modify-Multipoint-Channel response primitive, indicating whether or not the operation succeeded and issue it to the Control AVCSAP.

On receipt of an AVC-Modify-Multipoint-Channel response, the AVC Provider shall send a ModifyMultipointChannelResponse AVCPDU to the node that originated the request.  This is done by issuing an MCS-Send-Data request, specifying the AVC User ID of the originating AVC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field.  If present in the request PDU, the response PDU shall include the updated Modification Rights, regardless of the outcome of the attempt to modify the multipoint channel.  If the response primitive contains a successful result, the AVC Provider shall also generate a MultipointChannelStatusIndication AVCPDU with Status set to Modified and containing the AVC User Ids of all nodes that have been added to or removed from the multipoint channel.  It shall issue this PDU to all nodes currently joined to the channel and to any nodes that have been removed from the channel as a result of processing the ModifyMultipointChannelRequest AVCPDU.   For each of these nodes, this is done by issuing an MCS-Send-Data request, specifying the AVC User ID of the node as the Channel ID, specifying High data priority, and including the PDU in the Data field.

TABLE 116/T.132.

ModifyMultipointChannelRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| Multipoint Channel Identifier | Request | Indication |
| Nodes to Add List | Request | Indication |
| Nodes to Remove List | Request | Indication |
| Session Identifier | Request | Indication |
| Modification Rights | Request | Indication |
| QoS Parameters | Request | Indication |

TABLE 117/T.132.

ModifyMultipointChannelResponse AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Response | Confirm |
| Modification Rights | Source AVC Provider | Confirm |
| QoS Parameters | Response | Confirm |
| QoS Level | Response | Confirm |
| Result | Response | Confirm |

On receipt of a ModifyMultipointChannelResponse AVCPDU, an AVC Provider shall generate an AVC-Modify-Multipoint-Channel confirm primitive indicating the result included in the PDU and issue it to the Control AVCSAP.

### Disbanding a Multipoint Channel

On receipt of an AVC-Disband-Multipoint-Channel request primitive, an AVC Provider shall send a DisbandMultipointChannelRequest AVCPDU to its Zone Manager by issuing an MCS-Send-Data request, specifying the AVC User Id of the Zone Manager as the Channel Id, specifying High data priority, and including the PDU in the Data field.

On receipt of a DisbandMultipointChannelRequest AVCPDU, an AVC Provider shall consult the conference framework to determine whether the node issuing the request is authorised to use the service.  It shall also verify that the multipoint channel exists and shall check the modification rights (see §10.2.2) to determine whether the requesting node is authorised to disband the multipoint channel.  If any of these criteria are not met, it shall send a DisbandMultipointChannelResponse AVCPDU to the node that originated the request, indicating the reason for rejection.  This is done by issuing an MCS-Send-Data request, specifying the AVC User ID of the originating AVC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field.  Otherwise, the AVC Provider shall generate an AVC-Disband-Multipoint-Channel indication and issue it to the Control AVCSAP.

It is the responsibility of the Node Controller to assess the request and to co-ordinate with other Zone Managers, as appropriate, to disband the multipoint channel.  The Node Controller shall then generate an AVC-Disband-Multipoint-Channel response primitive, indicating whether or not the operation succeeded and issue it to the Control AVCSAP.

On receipt of an AVC-Disband-Multipoint-Channel response, the AVC Provider shall send a DisbandMultipointChannel Response AVCPDU to the node that originated the request.  This is done by issuing an MCS-Send-Data request, specifying the AVC User ID of the originating AVC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field.  If the response primitive contains a successful result, the AVC Provider shall also generate a MultipointChannelStatusIndication AVCPDU with Status set to Disbanded.  Source Node and Destination Node parameters are omitted.  The AVC Provider shall issue this PDU to all nodes joined to the channel prior to processing the DisbandMultipointChannelRequest AVCPDU.   For each of these nodes, this is done by issuing an MCS-Send-Data request, specifying the AVC User ID of the node as the Channel ID, specifying High data priority, and including the PDU in the Data field.

TABLE 118/T.132.

DisbandMultipointChannelRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| Multipoint Channel Identifier | Request | Indication |

TABLE 119/T.132.

DisbandMultipointChannelResponse AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Response | Confirm |
| Result | Response | Confirm |

On receipt of a DisbandMultipointChannelResponse AVCPDU, an AVC Provider shall generate an AVC-Disband-Multipoint-Channel confirm primitive indicating the result included in the PDU and issue it to the Control AVCSAP.

### Multipoint Channel Status

On receipt of a MultipointChannelStatusIndication AVCPDU, an AVC Provider shall generate an AVC-Multipoint-Channel-Status indication primitive and issue it to the Control AVCSAP.

TABLE 120/T.132.

MultipointChannelStatusIndication AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Multipoint Channel Identifier | Source AVC Provider | Indication |
| Channel Status | Source AVC Provider | Indication |
| Source Node | Source AVC Provider | Indication |
| Destination Nodes Added | Source AVC Provider | Indication |
| Destination Nodes Removed | Source AVC Provider | Indication |
| Session Identifier | Source AVC Provider | Indication |

## Source Selection

On receipt of an AVC-Source-Select request primitive, an AVC Provider shall send a SourceSelectRequest AVCPDU to the Source Owner by issuing an MCS-Send-Data request, specifying the AVC User Id of the Source Owner as the Channel Id, specifying High data priority, and including the PDU in the Data field.

On receipt of a SourceSelectRequest AVCPDU, an AVC Provider shall determine whether the device selected exists at its local node and whether the multipoint channel selected originates from the local node. If these criteria are not met, it shall send an SourceSelectResponse AVCPDU to the node that originated the request, indicating the reason for rejection.  This is done by issuing an MCS-Send-Data request, specifying the AVC User ID of the originating AVC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field.  Otherwise, the AVC Provider shall generate an AVC-Source-Select indication and issue it to the Control AVCSAP.

The Node Controller is responsible for processing the source selection request and for issuing an AVC-Source-Select response to notify the originating node of the outcome.  On receipt of an AVC-Source-Select response primitive, the AVC Provider shall generate a SourceSelectResponse AVCPDU and send it to the node that originated the request.

TABLE 121/T.132.

SourceSelectRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| Device Class | Request | Indication |
| Device Identifier | Request | Indication |
| Multipoint Channel Id | Request | Indication |

TABLE 122/T.132.

SourceSelectResponse AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Response | Confirm |
| Result | Response | Confirm |

On receipt of a SourceSelectResponse AVCPDU, an AVC Provider shall generate an AVC-Source-Select confirm primitive indicating the result included in the PDU and issue it to the Control AVCSAP.

## Remote Device Control

### Requesting Device Attributes

On receipt of an  AVC-Device-Attribute request primitive, an AVC provider shall send a DeviceAttributeRequest AVCPDU to the device owner by issuing an MCS-Send-Data request, specifying the AVC User ID of the device owner as the Channel ID, specifying normal priority, and including the PDU in the Data field.

TABLE 123/T.132.

DeviceAttributeRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| Device Class | Request | Indication |
| Device Identifier | Request | Indication |

The DeviceAttributeResponse AVCPDU is issued by an AVC application in reply to receiving the DeviceAttributeRequest AVCPDU. A successful response contains the requested device attributes. The response is sent with the channel ID of the PDU set to the user ID of the AVC application that made the original request.

TABLE 124/T.132.

DeviceAttributeResponse AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Response | Confirm |
| Device Attribute List | Response | Confirm |
| Result | Response | Confirm |

The format of each attribute is as follows:-

| Parameter | Description |
| --- | --- |
| Device Attribute | This is an attribute for this device class. (See Annex C for more details.) |

On receiving the DeviceAttributeResponse AVCPDU from the responding node, the AVC provider should make a copy of the received device attribute list. It shall then use this attribute list to verify the contents of an attribute list contained in other primitive requests.  The AVC provider shall delete this attribute list if the device entry is removed from the application roster. It may also delete this list due to local circumstances (for example the available memory is low and the device has not been remotely accessed for a long period of time). In this case further request primitives containing device attributes destined for that device will be rejected until another successful DeviceAttributeResponse AVCPDU has been received.

### Locking Devices

On receipt of an  AVC-Device-Lock request primitive, an AVC provider shall send a DeviceLockRequest AVCPDU to the device owner by issuing an MCS-Send-Data request, specifying the AVC User ID of the device owner as the Channel ID, specifying normal priority, and including the PDU in the Data field.

TABLE 125/T.132.

DeviceLockRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| Device Class | Request | Indication |
| Device Identifier | Request | Indication |
| Lock / Unlock | Request | Indication |

The DeviceLockResponse AVCPDU is issued by an AVC application in reply to receiving the DeviceLock Request AVCPDU. The response is sent with the channel ID of the PDU set to the user ID of the AVC application that made the original request.

TABLE 126/T.132.

DeviceLockResponse AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Response | Confirm |
| Result | Response | Confirm |

The AVC provider must maintain a list of those devices which it has locked. The provider can only send an DeviceControlRequest AVCPDU to a device which it has already successfully locked.

### Device Lock Inquire

On receipt of an  AVC-Device-Lock-Inquire request primitive, an AVC provider shall send a DeviceLockInquire Request AVCPDU to the device owner by issuing an MCS-Send-Data request, specifying the AVC User ID of the device owner as the Channel ID, specifying normal priority, and including the PDU in the Data field.

TABLE 127/T.132.

DeviceLockInquireRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| Device Class | Request | Indication |
| Device Identifier | Request | Indication |

The DeviceLockInquireResponse AVCPDU is issued by an AVC application in reply to receiving the DeviceLockInquire Request AVCPDU. A successful response indicates the node’s requirement for the device lock to continue. The response is sent with the channel ID of the PDU set to the user ID of the AVC application that made the original request.

TABLE 128/T.132.

DeviceLockInquireResponse AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Response | Confirm |
| Result | Response | Confirm |

On receipt of the DeviceLockInquireResponse AVCPDU, the AVC provider must update its list of devices which it has locked.

### Device Lock Terminated Indication

On receipt of an AVC-Device-Lock-Terminated request primitive, an AVC Provider shall send a DeviceLockTerminatedIndication AVCPDU to the node which had originally locked the device. The sending node shall issue an MCS-Send-Data request, specifying the AVC User Id of the destination node as the Channel Id, specifying High data priority, and include the PDU in the Data field.

TABLE 129/T.132.

DeviceLockTerminatedIndication AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Device Class | Request | Indication |
| Device Identifier | Request | Indication |

On receipt of an DeviceLockTerminatedIndication AVCPDU, an AVC Provider shall generate an AVC-Device-Lock-Terminated indication and issue it to the Control AVCSAP. A node may no longer send a DeviceControlRequest AVCPDU specifying a particular remote device once a node has indicated that the lock on that device has been terminated.

### Device Control Request

On receipt of an AVC-Device-Control request primitive, an AVC Provider shall send determine that it has ownership of a lock placed on the destination device. It should also verify the contents of the list of device control attributes against the attribute information previously received in the DeviceAttributeResponse AVCPDU. If any attribute is incorrect or a device lock has not previously been obtained then the request primitive must be rejected by returning an AVC-Device-Control confirm primitive with the appropriate result. If this is not the case it shall send a DeviceControlRequest AVCPDU by issuing an MCS-Send-Data request, specifying the AVC User Id of the node which owns the device as the Channel Id, specifying High data priority, and include the PDU in the Data field.

TABLE 130/T.132.

DeviceControlRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Device Class | Request | Indication |
| Device Identifier | Request | Indication |
| List of Device Control attributes | Request | Indication |

On receipt of an DeviceControlRequest AVCPDU, an AVC Provider shall generate an AVC-Device-Control indication and issue it to the Control AVCSAP. The User Application should then parse the list of device control attributes and command the specified device to perform the appropriate actions.  All actions should be processed by the device in the same order as the reception of  AVC-Device-Control indication primitives.

### Device Status Inquire

On receipt of an  AVC-Device-Status-Inquire request primitive, an AVC provider shall verify the contents of the list of device status attributes against the attribute information previously received in the DeviceAttributeResponse AVCPDU. If any attribute is incorrect, the request primitive must be rejected by returning an AVC-Device-Status-Inquire confirm primitive with the appropriate result. If the attribute list is correct, the AVC provider shall send a DeviceStatusInquire Request AVCPDU to the device owner by issuing an MCS-Send-Data request, specifying the AVC User ID of the device owner as the Channel ID, specifying normal priority, and including the PDU in the Data field.

TABLE 131/T.132.

DeviceStatusInquireRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| Device Class | Request | Indication |
| Device Identifier | Request | Indication |
| List of device status attributes | Request | Indication |

The DeviceStatusInquire Response AVCPDU is issued by an AVC application in reply to receiving the DeviceStatusInquire Request AVCPDU. A successful response contains a list of status attributes with accompanying status fields containing the current state of the device. The response is sent with the channel ID of the PDU set to the user ID of the AVC application that made the original request.

TABLE 132/T.132.

DeviceStatusInquireResponse AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Response | Confirm |
| List of device status attributes | Response | Confirm |
| Result | Response | Confirm |

### Configuring Device Events

An AVC application can register an interest in event notifications from a specific remote device using the AVC-Configure-Device-Events primitive. On receipt of an  AVC-Configure-Device-Events request primitive, an AVC provider shall verify the contents of the list of device event attributes against the attribute information previously received in the DeviceAttributeResponse AVCPDU. If any attribute is incorrect, the request primitive must be rejected by returning an AVC-Configure-Device-Events confirm primitive with the appropriate result. If the attribute list is correct, the AVC provider shall send a ConfigureDeviceEventsRequest AVCPDU to the device owner by issuing an MCS-Send-Data request, specifying the AVC User ID of the device owner as the Channel ID, specifying normal priority, and including the PDU in the Data field.

TABLE 133/T.132.

ConfigureDeviceEventsRequest AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Request | Indication |
| Device Class | Request | Indication |
| Device Identifier | Request | Indication |
| List of device event attributes | Request | Indication |

The ConfigureDeviceEventsResponse AVCPDU is issued by an AVC application in reply to receiving the ConfigureDeviceEventsRequest AVCPDU.  The response is sent with the channel ID of the PDU set to the user ID of the AVC application that made the original request.

TABLE 134/T.132.

ConfigureDeviceEventsResponse AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Request Handle | Response | Confirm |
| Result | Response | Confirm |

### Device Event Notify

On receipt of an AVC-Device-Event-Notify request primitive, an AVC Provider shall send a DeviceEventNotifyIndication AVCPDU to the node which had registered an interest in receiving a notification  of that device event. The sending node shall issue an MCS-Send-Data request, specifying the AVC User Id of the destination node as the Channel Id, specifying High data priority, and include the PDU in the Data field.

TABLE 135/T.132.

DeviceEventNotifyIndication AVCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Device Class | Request | Indication |
| Device Identifier | Request | Indication |
| List of event attributes | Request | Indication |

On receipt of an DeviceEventNotifyIndication AVCPDU, an AVC Provider shall generate an AVC-Device-Event-Notify indication and issue it to the Control AVCSAP.

# AVC PDU Definitions

The structure of AVC PDUs is specified as follows using the notation ASN.1 of ITU Recommendation X.680.  All AVC PDUs shall be encoded for transmission by applying the Packed Encoding Rules of ITU-T Recommendation X.691 using the Basic Aligned variant.  AVC PDUs are encoded and placed in the data field of either MCS-Send-Data or MCS-Uniform-Send-Data primitives.  The bit string generated by the ASN.1 encoding is placed in the OCTET STRING used by MCS in the order such that for each octet, the leading bit is placed in the most significant bit position and the trailing bit is placed in the least significant bit position.

AVC-PROTOCOL DEFINITIONS AUTOMATIC TAGS::=

BEGIN

IMPORTS			ChannelID,
				StaticChannelID,
				DynamicChannelID,
				UserID,
				TokenID,
				StaticTokenID,
				DynamicTokenID,
				Key,
				H221NonStandardIdentifier,
				NonStandardParameter,
				TextString,

Password

FROM GCC-PROTOCOL {itu-t recommendation t 124 version(0) 1},
				CapabilityTableEntry,
				CapabilityDescriptor,
				MultiplexCapability,
				ModeElement,
				LogicalChannelNumber,

OpenLogicalChannel,

OpenLogicalChannelAck

OpenLogicalChannelConfirm

OpenLogicalChannelReject,

MultiplexEntrySend

FROM MULTIMEDIA-SYSTEM-CONTROL{itu-t (0) recommendation (0) h (8) 245 version (0) 2};

-- Export all symbols

-- ==========================================================================

-- Part 1:  Message Components

-- ==========================================================================

Handle							::= INTEGER (0..4294967295)

NonStandardIdentifier				::= CHOICE

{

object              					OBJECT IDENTIFIER,

h221nonStandard     				H221NonStandardIdentifier

}

MultipointChannelID				::= INTEGER (0..65535)

LinkID							::= SEQUENCE

{

LowerNodeID				UserID,

UpperNodeID				UserID

}

MediaType						::= CHOICE

{

Audio					NULL,

Video					NULL,

Data						NULL,

...

}

NodeType						::= CHOICE

{

Terminal					NULL,

MCU					NULL,

MultipointController		NULL,

MultipointProcessor		NULL,

Gateway					NULL,

ConferenceServer		NULL,

...

}

SynchronisedChannelList		::= SEQUENCE (SIZE (1..256)) OF SEQUENCE

{

logicalChannelNumber		LogicalChannelNumber,

positiveSkew				INTEGER (0..4095),		-- units milliseconds

negativeSkew			INTEGER (0..4095),		-- units milliseconds

...

}

QoSParameters					::= SEQUENCE

{

meanBitrate				INTEGER (0..MAX) OPTIONAL,	-- units bits/s

peakBitrate				INTEGER (0..MAX) OPTIONAL,	-- units bits/s

defaultBitrate				INTEGER (0..MAX) OPTIONAL,	-- units bits/s

meanLatency				INTEGER (0..4095) OPTIONAL,	-- units milliseconds

maximumLatencyVariation	INTEGER (0..1023) OPTIONAL,	-- units milliseconds

...

}

QoSLevel					::= CHOICE

{

guaranteed				NULL,

predictive				NULL,

bestEffort				NULL,

...

}

MLPDataRate						::= CHOICE

{

mlp-4k					NULL,

mlp-6.4k					NULL,

mlp-14.4k				NULL,

mlp-16k					NULL,

mlp-22.4k				NULL,

mlp-24k					NULL,

mlp-30.4k				NULL,

mlp-32k					NULL,

mlp-38.4k				NULL,

mlp-40k					NULL,

mlp-46.4k				NULL,

mlp-62.4k				NULL,

mlp-64k					NULL,

mlp-128k					NULL,

mlp-192k					NULL,

mlp-256k					NULL,

mlp-320k					NULL,

mlp-384k					NULL,

...

}

(Editor’s Note: Would it be possible to just specify the max data rate supported?)

H221MultiplexCapability			::= SEQUENCE

{

transferRate			CHOICE

{

56k				NULL,

128k			NULL,

192k			NULL,

256k			NULL,

320k			NULL,

384k			NULL,

512k			NULL,

768k			NULL,

1152k			NULL,

1472k			NULL,

1536k			NULL,

1920k			NULL,

...

},

mlpDataRate			SET OF MLPDataRate,

(Editor’s Note: More parameters?)

...

}

H221MultiplexConfiguration			::= SEQUENCE

{

transferRate			CHOICE

{

64k				NULL,

128k			NULL,

192k			NULL,

256k			NULL,

320k			NULL,

384k			NULL,

512k			NULL,

768k			NULL,

1152k			NULL,

1472k			NULL,

1536k			NULL,

1920k			NULL,

...

},

mlpDataRate			MLPDataRate,

(Editor’s Note: More parameters?)

...

}

LinkCapabilitySet					::= SEQUENCE

{

linkID					LinkID,

multiplexCapability		CHOICE

{

h245MultiplexCapability	MultiplexCapability,

h221MultiplexCapability	H221MultiplexCapability,

None				NULL,

...

},

networkType				(Editor’s note: Do we need this?),

upLinkQoS				QoSParameters OPTIONAL	-- QoS from lower node to

-- higher node

downLinkQoS			QoSParameters OPTIONAL	-- QoS from higher node to

-- lower node

mediaCapabilityTable		SET SIZE (1..256) OF CapabilityTableEntry OPTIONAL,

capabilityDescriptors		SET SIZE (1..256) OF CapabilityDescriptor OPTIONAL,

...

}

...

}

PortCapabilities 					::= SET OF CHOICE

{

singlePort				PortDescriptor,

aggregatedPorts		SEQUENCE

{

portsList				SET OF CHOICE

{

isdnPort		ISDNPort,	--Editor’s Note: Implies aggregation between

csdnPort		CSDNPort,	-- different port types.  Is this intentional?

permanentPort		PermanentPort,

...

},

channelAggregation	ChannelAggregationMethod OPTIONAL,

profilesSupported		SET OF Profile OPTIONAL,

mediaSupported		MediaList OPTIONAL,

...

},

...

}

PortDescriptor 					::= CHOICE

{

gstnPort					GSTNPort,

csdnPort					CSDNPort,

isdnPort					ISDNPort,

psdnPort				PSDNPort,

atmPort					??? Distinguish between CBR and VBR modes ???,

mobilePort				??? Seen as a GSTN port ???,

permanentPort			PermanentPort,	-- leased line or LAN interface

...

}

GSTNPort 						::= SEQUENCE

{

networkAddress		ExtendedE164NetworkAddress,

profilesSupported		SET OF Profile OPTIONAL,

mediaSupported		MediaList OPTIONAL,

...

}

CSDNPort 						::= SEQUENCE

{

baseChannelRate	CHOICE

{

digital-56k			NULL,

digital-64k			NULL,

…,

},

numberOfChannels		INTEGER (0..???),

networkAddresses		SET (SIZE (1..???)) OF ExtendedE164NetworkAddress,

channelAggregation		ChannelAggregationMethod OPTIONAL,

profilesSupported		SET OF Profile OPTIONAL,

mediaSupported		MediaList OPTIONAL,

...

}

ISDNPort 						::= SEQUENCE

{

interfaceType			CHOICE

{

basic				NULL,

primary				NULL,

…

},

networkCapabilities		SEQUENCE

{

digital-64k			BOOLEAN,

digital-2x64k			BOOLEAN,

digital-384k			BOOLEAN,

digital-1472k			BOOLEAN,

digital-1536k			BOOLEAN,

digital-1920k			BOOLEAN,

multirate-base-64k		INTEGER (1..30)	-- See note 1

} OPTIONAL,

networkAddresses		SET (SIZE (1..???)) OF ExtendedE164NetworkAddress,

channelAggregation		ChannelAggregationMethod OPTIONAL,

profilesSupported		SET OF Profile OPTIONAL,

mediaSupported		MediaList OPTIONAL,

...

}

-- Note 1	:	digital-2x64k differs from multirate-base-64k with a multiplier value of 2 ; in the first case the

--			network is requested an 8 kHz integrity with Restricted Differential Time Delay (RDTD) ; in the

--			second case the network is requested a Time Slot Sequence integrity (see Rec. ITU-T/Q.931,

--			clause 4.5.5)

PermanentPort 					::= SEQUENCE

{

connectedNodeID		NodeID,

networkAddress	CHOICE

{

networkAddresses	SET (SIZE (1..???)) OF ExtendedE164NetworkAddress,

transportAddress		SEQUENCE

{

nsapAddress		OCTET STRING (SIZE (1..20)),

transportSelector	OCTET STRING OPTIONAL

},

nonstandard		NonStandardParameter

},

profilesSupported		SET OF Profile OPTIONAL,

mediaSupported		MediaList OPTIONAL,

...

}

MediaList						:: = SEQUENCE

{

audio					BOOLEAN,

video					BOOLEAN,

data						BOOLEAN,

...

}

ChannelAggregationMethod		::= CHOICE

{

nonStandard				NonStandardParameter,

h221					NULL,

h244					NULL,

iso-iec-13871				NULL,	-- The actual mode of bonding is dynamically

-- selected according to the procedures described in

--ISO/IEC 13871.

...

}

Profile 							::= CHOICE

{

simpleProfile				CHOICE

{

-- Basic transfer modes or information types :

speech				NULL,	-- Simple telephony

telephony-3kHz			NULL,	-- G.711

telephony-7kHz			NULL,	-- G.722

voice-band				NULL,	--V.series Modems

frameRelay,				NULL,

-- T.120-only data profiles (T.123) :

t123-pstn-basic			NULL,

t123-psdn-basic			NULL,

t123-b-isdn-basic			NULL,

…

},

-- Multimedia profiles :

multimediaProfile			SEQUENCE

{

profile				CHOICE

{

h310			NULL,

h320			NULL,

h321			NULL,

h322			NULL,

h323			NULL,

h324			NULL,

h324m		NULL,

v61				NULL,	--asvd

v70				NULL,	--dsvd

…

},

t120Data				BOOLEAN

},

dsmccDownloadProfile		NULL,

nonStandard				NonStandardParameter,

...

}

ExtendedE164NetworkAddress 		::= SEQUENCE

{

internationalNumber		DialingString,

subAddress				SubAddressString OPTIONAL,

extraDialing				ExtraDialingString OPTIONAL,

...

}

TranscoderTableEntry				::=SEQUENCE

{

inputMediaFormat		ModeElement,

outputMediaFormat		ModeElement,

bidirectionalFlag		BOOLEAN,		--TRUE if capable of transcoding

...									-- from output format to input format

}

NodeCapabilitySet				::= SEQUENCE

{

nodeID					UserID,

nodeType				NodeType,

bridgeCapabilityTable		SET OF MediaType OPTIONAL,

transcoderTable		SET OF TranscoderTable Entry OPTIONAL,

portCapabilityTable		SET OF PortCapabilities,

associatedDevices		???,

mediaCapabilityTable		SET SIZE (1..256) OF CapabilityTableEntry OPTIONAL,

capabilityDescriptors		SET SIZE (1..256) OF CapabilityDescriptor OPTIONAL,

...

}

MultiplexConfiguration				::= CHOICE

{

h223MultiplexConfiguration	MultiplexEntrySend,

h221MultiplexConfiguration	H221MultiplexConfiguration,

...

}

LinkConfiguration				::= SEQUENCE

{

linkID					LinkID,

upMultiplexConfiguration	MultiplexConfiguration,	-- Lower node to upper node

downMultiplexConfiguration	MultiplexConfiguration,	-- Upper node to lower node

upMediaConfiguration		MediaConfigurationTable,-- Lower node to upper node

downMediaConfiguration	MediaConfigurationTable,-- Upper node to lower node

...

}

MediaConfigurationTable		::= SET SIZE (1..256) MediaConfiguration

MediaConfiguration				::= SEQUENCE

{

modeElement				ModeElement,

logicalChannelNumber		LogicalChannelNumber,

multiplexParameters		MultiplexParameters OPTIONAL,

synchronisedChannelList	SynchronisedChannelList OPTIONAL,

...

}

BridgeConfigurationEntry		::= SEQUENCE

{

incomingLinkID			LinkID,

input			CHOICE

{

incomingLogicalChannel		LogicalChannelNumber,

incomingMultipointChannel		MultipointChannelID

},

outgoingLinkID			LinkID,

output		CHOICE

{

outgoingLogicalChannel		LogicalChannelNumber,

outgoingMultipointChannel		MultipointChannelID

},

...

}

NodeConfiguration				::= SEQUENCE

{

nodeID					UserID,

bridgeConfiguration		SET OF BridgeConfigurationEntry OPTIONAL,

...

}

TransmissionModeElement		::= SEQUENCE

{

modeElement				ModeElement,

multipointChannelID		MultipointChannelID OPTIONAL,

...

}

LinkRosterUpdate					::= SEQUENCE

{

fullCapabilityRefresh		BOOLEAN,

fullConfigurationRefresh	BOOLEAN,

linksRemoved			SET SIZE (1..256) OF LinkID OPTIONAL,

linkCapsAdded			SET SIZE (1..256) OF LinkCapabilitySet OPTIONAL,

linkCapsModified		SET SIZE (1..256) OF LinkCapabilitySet OPTIONAL,

linkConfigsAdded		SET SIZE (1..256) OF LinkConfiguration OPTIONAL,

linkConfigsModified		SET SIZE (1..256) OF LinkConfiguration OPTIONAL,

...

}

NodeRosterUpdate				::= SEQUENCE

{

fullCapabilityRefresh		BOOLEAN,

fullConfigurationRefresh	BOOLEAN,

nodesRemoved			SET SIZE (1..256) OF UserID OPTIONAL,

nodeCapsAdded		SET SIZE (1..256) OF NodeCapabilitySet OPTIONAL,

nodeCapsModified		SET SIZE (1..256) OF NodeCapabilitySet OPTIONAL,

nodeConfigsAdded		SET SIZE (1..256) OF NodeConfiguration OPTIONAL,

nodeConfigsModified		SET SIZE (1..256) OF NodeConfiguration OPTIONAL,

...

}

RosterKey						::= SEQUENCE

{

(Editor’s Note: to be defined)

}

RosterEntry						::= CHOICE

{

linkCapability				LinkCapabilitySet,

linkConfiguration		LinkConfiguration,

nodeCapability			NodeCapabilitySet,

nodeConfiguration		NodeConfiguration,

...

}

AVCServiceLIst					::= SET SIZE (0..31)

{

avcService					AVCService,

avcSessionID					ChannelID OPTIONAL,

statusflag					BOOLEAN,	-- TRUE for activated,

…

}

AVCService						::= CHOICE

{

audioMixing				NULL,

videoSwitching			NULL,

continuousPresence		NULL,

floorControl				NULL,

nonStandardService		NonStandardIdentifier,

...

}

ActiveAVCService					::= SEQUENCE

{

avcService				AVCService,

avcSessionID				ChannelID OPTIONAL,

subscriptionFlag		BOOLEAN,		-- TRUE if the node is subscribed

...

}

ResourceDescriptor				::= CHOICE

{

nonStandardDescriptor		NonStandardParameter,

multipointChannel		MultipointChannel,

...

}

MultipointChannel					::= SEQUENCE

{

multipointChannelID		MultipointChannelID,

mediaType				MediaType,

sourceNodeID			UserID,

mediaFormat				ModeElement OPTIONAL,

destinationNodeList		SET (SIZE 1..65536) OF SEQUENCE

{

destinationNodeID		UserID,

mediaFormat		ModeElement OPTIONAL,

...

},

qosParameters			QoSParameters OPTIONAL,

synchronisation		SynchronisedMultipointChannelList OPTIONAL,

...

}

SynchronisedMultipointChannelList	::= SEQUENCE (SIZE (1..256)) OF SEQUENCE

{

multipointChannelID		MultipointChannelID,

positiveSkew				INTEGER (0..4095),		-- units milliseconds

negativeSkew			INTEGER (0..4095),		-- units milliseconds

...

}

ModificationRights				::= CHOICE

{

owner					NULL,

session					NULL,

public					NULL

}

DeviceClass						::= INTEGER (0..511)

DeviceID							::= INTEGER (0..127)

DeviceList						::= SET SIZE (1..256)

{

deviceID					DeviceID,

audioSourceFlag		BOOLEAN,

audioSinkFlag			BOOLEAN,

videoSourceFlag		BOOLEAN,

videoSinkFlag			BOOLEAN,

remoteControlFlag		BOOLEAN,

deviceName				TextString OPTIONAL,

…

}

-- ==========================================================================

-- Part 2:  PDU Messages

-- ==========================================================================

ActivateManagementRequest			::= SEQUENCE

{

requestHandle				Handle,

convenorPassword			Password,

conferenceFramework			ConferenceFramework, -- to be defined

...

}

ActivateManagementResponse			::= SEQUENCE

{

requestHandle				Handle,

result		CHOICE

{

successful				NULL,

notAuthorisedNode			NULL,

managementAlreadyActive		NULL,

...

},

...

}

InitiateManagementIndication			::= SEQUENCE

{

conferenceFramework			ConferenceFramework, -- to be defined

...

}

PrivateSessionAdmitIndication			::= SEQUENCE

{

sessionID					ChannelID,

invitedNodes					SET (SIZE (1..65536)) OF UserID OPTIONAL,

...

}

PrivateSessionExpelIndication			::= SEQUENCE

{

sessionID					ChannelID,

expelledNodes				SET (SIZE (1..65536)) OF UserID OPTIONAL,

...

}

AnnounceZoneManagerIndication		::= SEQUENCE

{

-- No parameters

...

}

RosterUpdateIndication				::= SEQUENCE

{

linkRosterUpdate			LinkRosterUpdate	OPTIONAL,

nodeRosterUpdate			NodeRosterUpdate	OPTIONAL,

...

}

RosterEnquireRequest					::= SEQUENCE

{

requestHandle				Handle,

rosterKey					RosterKey,

capabilityFlag				BOOLEAN,	-- TRUE to retrieve capability details

configurationFlag			BOOLEAN,	-- TRUE to retrieve configuration

-- details

...

}

RosterEnquireResponse				::= SEQUENCE

{

requestHandle				Handle,

rosterKey					RosterKey,

rosterEntries					SET SIZE (1..256) OF RosterEntry OPTIONAL,

result		CHOICE

{

successful				NULL,

insufficientPrivilege		NULL,

entryNotFound			NULL,

notEntryOwner			NULL,

...

},

...

}

RosterMonitorRequest					::= SEQUENCE

{

requestHandle				Handle,

rosterKey					RosterKey,

capabilityFlag				BOOLEAN,	-- TRUE to monitor capability details

configurationFlag			BOOLEAN,	-- TRUE to monitor configuration

-- details

mediatype					SET OF MediaType OPTIONAL,

...

}

SetLinkMultiplexRequest				::= SEQUENCE

{

requestHandle				Handle,

linkID						LinkID,

multiplexConfiguration			MultiplexConfiguration,

...

}

SetLinkMultiplexResponse			::= SEQUENCE

{

requestHandle				Handle,

result		CHOICE

{

successful				NULL,

unspecifiedFailure			NULL,

...

},

...

}

SetLinkTransmissionModeRequest		::= SEQUENCE

{

requestHandle			Handle,

linkID					LinkID,

transmissionMode		SEQUENCE SIZE (1..256) OF TransmissionModeElement,

...

}

SetLinkTransmissionModeResponse		::= SEQUENCE

{

requestHandle				Handle,

result		CHOICE

{

successful				NULL,

modeUnavailable			NULL,

multipointConstraint		NULL,

requestDenied			NULL,

unspecifiedFailure			NULL,

...

},

...

}

SetNodeConfigurationRequest			::= SEQUENCE

{

requestHandle				Handle,

nodeID						UserID,

addToBridge					SET OF BridgeConfigurationEntry OPTIONAL,

removeFromBridge			SET OF BridgeConfigurationEntry OPTIONAL,

...

}

SetNodeConfigurationResponse			::= SEQUENCE

{

requestHandle				Handle,

result		CHOICE

{

successful				NULL,

InvalidBridgeEntry		NULL,

unspecifiedFailure			NULL,

...

},

...

}

OpenLogicalChannelRequest			::= SEQUENCE

{

requestHandle				Handle,

linkID						LinkID,

pathID						PathID,

CHOICE

{

openH245LogicalChannel		OpenLogicalChannel,

...						-- for non-H.245 environments

},

qosParameters				QoSParameters OPTIONAL,

synchronisedChannelList		SynchronisedChannelList OPTIONAL,

...

}

OpenLogicalChannelResponse			::= SEQUENCE

{

requestHandle				Handle,

result	CHOICE

{

(Editor’s Note: Check this- may be better to replace with AVC result codes)

openH245LogicalChannelAck	OpenLogicalChannelAck,

openH245LogicalChannelConfirm	OpenLogicalChannelConfirm,

openH245LogicalChannelReject	OpenLogicalChannelReject,

... 						-- for non-H.245 environments

},

qosParameters				QoSParameters OPTIONAL,

(Editor’s Note: Is this in the right place, given it is conditional on a successful result?)

qosLevel						QoSLevel OPTIONAL,

...

}

CloseLogicalChannelRequest			::= SEQUENCE

{

requestHandle				Handle,

linkID						LinkID,

logicalChannelNumber			LogicalChannelNumber,

...

}

CloseLogicalChannelResponse			::= SEQUENCE

{

requestHandle				Handle,

result		CHOICE

{

successful				NULL,

unspecifiedFailure			NULL,

...

},

...

}

ReserveResourceRequest			::= SEQUENCE

{

requestHandle				Handle,

resourceDescriptor			ResourceDescriptor,

...

}

ReserveResourceResponse			::= SEQUENCE

{

requestHandle				Handle,

reservationHandle			INTEGER  (0..65535) OPTIONAL,

(Editor’s Note: Are these in the right place, given they are conditional on a successful result?)

qosLevel						QoSLevel OPTIONAL,

result		CHOICE

{

successful				NULL,

invalidResourceDescriptor		NULL,

insufficientResources		NULL,

unspecifiedFailure			NULL,

...

},

...

}

CancelResourceReservationRequest		::= SEQUENCE

{

requestHandle				Handle,

resourceDescriptor			ResourceDescriptor,

...

}

ConfirmResourceReservationRequest		::= SEQUENCE

{

requestHandle				Handle,

reservationHandle			INTEGER  (0..65535),

...

}

ConfirmResourceReservationResponse		::= SEQUENCE

{

requestHandle				Handle,

result		CHOICE

{

successful				NULL,

reservationLapsed			NULL,

unspecifiedFailure			NULL,

...

},

...

}

RelinquishResourceRequest			::= SEQUENCE

{

requestHandle				Handle,

resourceDescriptor			ResourceDescriptor,

...

}

ServiceOfferRequest					::= SEQUENCE

{

requestHandle				Handle,

activeAVCServiceList			SET OF ActiveAVCService OPTIONAL,

inactiveAVCServiceList			SET OF AVCService OPTIONAL,

...

}

ServiceSubscribeRequest			::= SEQUENCE

{

requestHandle				Handle,

avcService					AVCService,

avcSessionID					ChannelID OPTIONAL,

subscribeFlag				BOOLEAN,	-- TRUE to subscribe

...

}

ServiceSubscribeResponse			::= SEQUENCE

{

requestHandle				Handle,

result		CHOICE

{

successful				NULL,

notAuthorisedNode			NULL,

serviceNot Available		NULL,

invalidSessionID			NULL,

...

},

...

}

ServiceSubscribeReportIndication		::= SEQUENCE

{

nodeID						UserID,

avcService					AVCService,

avcSessionID					ChannelID OPTIONAL,

subscribeFlag				BOOLEAN,	-- TRUE to subscribe

...

}

ServiceActivateRequest				::= SEQUENCE

{

requestHandle				Handle,

avcService					AVCService,

avcSessionID					ChannelID OPTIONAL,

activateFlag					BOOLEAN,	-- TRUE to activate

...

}

ServiceActivateResponse			::= SEQUENCE

{

requestHandle				Handle,

result		CHOICE

{

successful				NULL,

notAuthorisedNode			NULL,

serviceNot Available		NULL,

invalidSessionID			NULL,

...

},

...

}

ServiceLaunchRequest				::= SEQUENCE

{

requestHandle				Handle,

avcService					AVCService,

avcSessionID					ChannelID OPTIONAL,

activateFlag					BOOLEAN,	-- TRUE to activate

...

}

ServiceLaunchResponse				::= SEQUENCE

{

requestHandle				Handle,

result		CHOICE

{

successful				NULL,

notTopAVCProvider		NULL,

serviceNot Available		NULL,

invalidSessionID			NULL,

unspecifiedFailure			NULL,

...

},

...

}

ServiceLaunchReportIndication			::= SEQUENCE

{

avcServiceList				AVCServiceList,

fullRefresh					BOOLEAN,	-- TRUE for full refresh

...

}

CreateMultipointChannelRequest			::= SEQUENCE

{

requestHandle				Handle,

sourceNodeID				UserID OPTIONAL,

destinationNodeList			SET SIZE (1..256) OF UserID,

mediaType					MediaType, (Editor’s Note: this doesn’t allow media format to be selected)

avcSessionID					ChannelID OPTIONAL,

modificationRights			ModificationRights OPTIONAL,

qosParameters				QoSParameters OPTIONAL,

synchronisedMultipointChannelList	SynchronisedMultipointChannelList OPTIONAL,

...

}

CreateMultipointChannelResponse		::= SEQUENCE

{

requestHandle				Handle,

multipointChannelID			MultipointChannelID OPTIONAL,

qosLevel						QoSLevel OPTIONAL,

result		CHOICE

{

successful				NULL,

notAuthorisedNode			NULL,

invalidSourceNode			NULL,

invalidDestinationNodeList		NULL,

unspecifiedFailure			NULL,

...

},

...

}

ModifyMultipointChannelRequest			::= SEQUENCE

{

requestHandle				Handle,

multipointChannelID			MultipointChannelID OPTIONAL,

addedNodeList				SET SIZE (1..256) OF UserID OPTIONAL

removedNodeList			SET SIZE (1..256) OF UserID OPTIONAL

avcSessionID					ChannelID OPTIONAL,

modificationRights			ModificationRights OPTIONAL,

qosParameters				QoSParameters OPTIONAL,

...

}

ModifyMultipointChannelResponse		::= SEQUENCE

{

requestHandle				Handle,

result		CHOICE

{

successful				NULL,

invalidMultipointChannelID		NULL,

notAuthorisedNode			NULL,

invalidAddedNodeList		NULL,

invalidRemovedNodeList		NULL,

unspecifiedFailure			NULL,

...

},

...

}

DisbandMultipointChannelRequest		::= SEQUENCE

{

requestHandle				Handle,

multipointChannelID			MultipointChannelID OPTIONAL,

...

}

DisbandMultipointChannelResponse		::= SEQUENCE

{

requestHandle				Handle,

result		CHOICE

{

successful				NULL,

invalidMultipointChannelID		NULL,

notAuthorisedNode			NULL,

unspecifiedFailure			NULL,

...

},

...

}

MultipointChannelStatusIndication		::= SEQUENCE

{

multipointChannelID			MultipointChannelID,

status		CHOICE

{

new						NULL,

modified					NULL,

disbanded				NULL,

...

},

sourceNodeID				UserID OPTIONAL,

addedNodeList				SET SIZE (1..256) OF UserID OPTIONAL

removedNodeList			SET SIZE (1..256) OF UserID OPTIONAL

avcSessionID					ChannelID OPTIONAL,

...

}

SourceSelectRequest					::= SEQUENCE

{

requestHandle				Handle,

deviceClass					DeviceClass,

deviceID						DeviceID,

multipointChannelID			MultipointChannelID OPTIONAL,

...

}

SourceSelectResponse				::= SEQUENCE

{

requestHandle				Handle,

result		CHOICE

{

successful				NULL,

requestDenied			NULL,

invalidSourceID			NULL,

invalidMultipointChannelID		NULL,

notAuthorisedNode			NULL,

deviceUnavailable			NULL,

...

},

...

}

DeviceAttributeRequest				::= SEQUENCE

{

requestHandle				Handle,

deviceClass					DeviceClass,

deviceID						DeviceID,

...

}

DeviceAttributeResponse			::= SEQUENCE

{

requestHandle				Handle,

deviceAttributeList			SET SIZE (1..???) OF DeviceAttribute OPTIONAL,

result		CHOICE

{

successful				NULL,

requestDenied			NULL,

unknownDevice			NULL,

notAuthorisedNode			NULL,

deviceUnavailable			NULL,

...

},

...

}

DeviceLockRequest					::= SEQUENCE

{

requestHandle				Handle,

deviceClass					DeviceClass,

deviceID						DeviceID,

lockFlag						BOOLEAN,	-- TRUE to lock

...

}

DeviceLockResponse					::= SEQUENCE

{

requestHandle				Handle,

result		CHOICE

{

successful				NULL,

requestDenied			NULL,

unknownDevice			NULL,

notAuthorisedNode			NULL,

deviceUnavailable			NULL,

...

},

...

}

DeviceLockEnquireRequest			::= SEQUENCE

{

requestHandle				Handle,

deviceClass					DeviceClass,

deviceID						DeviceID,

...

}

DeviceLockEnquireResponse			::= SEQUENCE

{

requestHandle				Handle,

result		CHOICE

{

lockRequired			NULL,

LockNotRequired			NULL,

unknownDevice			NULL,

...

},

...

}

DeviceLockTerminatedIndication			::= SEQUENCE

{

deviceClass					DeviceClass,

deviceID						DeviceID,

...

}

DeviceControlRequest					::= SEQUENCE

{

requestHandle				Handle,

deviceClass					DeviceClass,

deviceID						DeviceID,

controlAttributeList			SET SIZE (1..???) OF ControlAttribute,

...

}

DeviceStatusEnquireRequest			::= SEQUENCE

{

requestHandle				Handle,

deviceClass					DeviceClass,

deviceID						DeviceID,

StatusAttributeIdentifierList		SET SIZE (1..???) OF StatusAttributeIdentifier,

...

}

DeviceStatusEnquireResponse			::= SEQUENCE

{

requestHandle				Handle,

statusAttributeList			SET SIZE (1..???) OF StatusAttribute OPTIONAL,

result		CHOICE

{

successful				NULL,

requestDenied			NULL,

unknownDevice			NULL,

notAuthorisedNode			NULL,

deviceUnavailable			NULL,

...

},

...

}

ConfigureDeviceEventsRequest			::= SEQUENCE

{

requestHandle				Handle,

deviceClass					DeviceClass,

deviceID						DeviceID,

deviceEventIdentifierList		SET SIZE (1..???) OF DeviceEventIdentifier,

...

}

ConfigureDeviceEventsResponse		::= SEQUENCE

{

requestHandle				Handle,

result		CHOICE

{

successful				NULL,

requestDenied			NULL,

unknownDevice			NULL,

notAuthorisedNode			NULL,

deviceUnavailable			NULL,

...

},

...

}

DeviceEventNotifyIndication			::= SEQUENCE

{

deviceClass					DeviceClass,

deviceID						DeviceID,

deviceEventList			SET SIZE (1..???) OF DeviceEvent,

...

}

NonStandardPDU						::= SEQUENCE

{

nonStandardData			NonStandardParameter,

...

}

-- ==========================================================================

-- Part 3:  Messages sent using MCS-Send-Data or MCS-Uniform-Send-Data

-- ==========================================================================

AVCPDU 							::= CHOICE
{
	request					RequestPDU,
	response					ResponsePDU,
	indication					IndicationPDU
}

RequestPDU ::= CHOICE
{

activateManagementRequest		ActivateManagementRequest,
	rosterEnquireRequest			RosterEnquireRequest,
	rosterMonitorRequest			RosterMonitorRequest,
	setLinkMultiplexRequest		SetLinkMultiplexRequest,
	setLinkTransmissionModeRequest	SetLinkTransmissionModeRequest,
	setNodeConfigurationRequest		SetNodeConfigurationRequest,
	openLogicalChannelRequest		OpenLogicalChannelRequest,
	closeLogicalChannelRequest		CloseLogicalChannelRequest,
	reserveResourceRequest		ReserveResourceRequest,
	cancelResourceReservationRequest	CancelResourceReservationRequest,
	confirmResourceReservationRequest	ConfirmResourceReservationRequest,
	relinquishResourceRequest		RelinquishResourceRequest,
	serviceOfferRequest			ServiceOfferRequest,
	serviceSubscribeRequest		ServiceSubscribeRequest,
	serviceActivateRequest			ServiceActivateRequest,
	launchPermissionRequest		LaunchPermissionRequest,
	serviceLaunchRequest			ServiceLaunchRequest,

createMultipointChannelRequest	CreateMultipointChannelRequest,

modifyMultipointChannelRequest	ModifyMultipointChannelRequest,

disbandMultipointChannelRequest	DisbandMultipointChannelRequest,

deviceAttributeRequest			DeviceAttributeRequest,

deviceLockRequest			DeviceLockRequest,

deviceLockEnquireRequest		DeviceLockEnquireRequest,

deviceControlRequest			DeviceControlRequest,

deviceStatusEnquireRequest		DeviceStatusEnquireRequest,

configureDeviceEventsRequest		ConfigureDeviceEventsRequest,
	nonStandardRequest			NonStandardPDU,
	...
}

ResponsePDU ::= CHOICE
{

activateManagementResponse		ActivateManagementResponse,
	rosterEnquireResponse			RosterEnquireResponse,
	rosterMonitorResponse			RosterMonitorResponse,
	setLinkMultiplexResponse		SetLinkMultiplexResponse,
	setLinkTransmissionModeResponse	SetLinkTransmissionModeResponse,
	setNodeConfigurationResponse		SetNodeConfigurationResponse,
	openLogicalChannelResponse		OpenLogicalChannelResponse,
	closeLogicalChannelResponse		CloseLogicalChannelResponse,
	reserveResourceResponse		ReserveResourceResponse,
	cancelResourceReservationResponse	CancelResourceReservationResponse,
	confirmResourceReservationResponse	ConfirmResourceReservationResponse,
	serviceSubscribeResponse		ServiceSubscribeResponse,
	launchPermissionResponse		LaunchPermissionResponse,
	serviceSubscribeResponse		ServiceSubscribeResponse,
	serviceActivateResponse		ServiceActivateResponse,
	serviceLaunchResponse		ServiceLaunchResponse,
	createMultipointChannelResponse	CreateMultipointChannelResponse,

modifyMultipointChannelResponse	ModifyMultipointChannelResponse,

disbandMultipointChannelResponse	DisbandMultipointChannelResponse,

deviceAttributeResponse		DeviceAttributeResponse,

deviceLockResponse			DeviceLockResponse,

deviceLockEnquireResponse		DeviceLockEnquireResponse,

deviceStatusEnquireResponse		DeviceStatusEnquireResponse,

configureDeviceEventsResponse	ConfigureDeviceEventsResponse,
	nonStandardResponse			NonStandardPDU,
	...
}

IndicationPDU ::= CHOICE
{

initiateManagementIndication		InitiateManagementIndication,

privateSessionAdmitIndication		PrivateSessionAdmitIndication,
	privateSessionExpelIndication		PrivateSessionExpelIndication,
	rosterUpdateIndication			RosterUpdateIndication,

serviceSubscribeReportIndication	ServiceSubscribeReportIndication,
	serviceLaunchReportIndication		ServiceLaunchReportIndication,

multipointChannelStatusIndication	MultipointChannelStatusIndication,

deviceLockTerminatedIndication	DeviceLockTerminatedIndication,

deviceEventNotifyIndication		DeviceEventNotifyIndication,
	nonStandardIndication			NonStandardPDU,
	...
}

END

Annex A

Framework Parameters for Conference Models

(this annex forms an integral part of this Recommendation)

This Annex specifies conference framework parameters for the standardised AVC conference models.  Parameters not specified are left to the discretion of the conference convenor.

TABLE A.1/T.132.

Framework parameters for conventional meeting

| Parameter | Value |
| --- | --- |
| Maximum Number of Anonymous Nodes | 0 |
| Public Session Flag | TRUE |
| Private Session Flag | TRUE |
| Primary Session Flag | TRUE |
| Chair Rôle Flag | TRUE |
| Presenter Rôle Flag | TRUE |
| Additional Sessions: Chair Rôle Flag | FALSE |
| Additional Sessions: Presenter Rôle Flag | FALSE |

TABLE A.2/T.132.

Framework parameters for virtual meeting rooms

| Parameter | Value |
| --- | --- |
| Maximum Number of Anonymous Nodes | 0 |
| Public Session Flag | TRUE |
| Private Session Flag | TRUE |
| Primary Session Flag | FALSE |
| Chair Rôle Flag | TRUE |
| Presenter Rôle Flag | TRUE |
| Additional Sessions: Chair Rôle Flag | TRUE |
| Additional Sessions: Presenter Rôle Flag | TRUE |

TABLE A.3/T.132.

Framework parameters for conventional conference

| Parameter | Value |
| --- | --- |
| Public Session Flag | TRUE |
| Private Session Flag | TRUE |
| Primary Session Flag | TRUE |
| Chair Rôle Flag | TRUE |
| Presenter Rôle Flag | TRUE |

TABLE A.4/T.132.

Framework parameters for broadcast conference

| Parameter | Value |
| --- | --- |
| Public Session Flag | FALSE |
| Private Session Flag | FALSE |
| Chair Rôle Flag | TRUE |
| Presenter Rôle Flag | TRUE |

TABLE A.5/T.132.

Framework parameters for gathering

| Parameter | Value |
| --- | --- |
| Public Session Flag | TRUE |
| Private Session Flag | TRUE |
| Primary Session Flag | FALSE |
| Chair Rôle Flag | FALSE |
| Presenter Rôle Flag | FALSE |

Annex B

Object Identifier Assignments

(this annex forms an integral part of this Recommendation)

TABLE B- lists the assignment of Object Identifiers defined for use by this Recommendation.

TABLE B-1/T.132

T. 132 object identifier assignment

| Object Identifier Value | Description |
| --- | --- |
| {itu-t recommendation t 132 version(0) 1} | This Object Identifier is used to indicate the version of this Recommendation. At this time there is a single standardised version defined. |

Annex C

Standard Device Classes and Device Control Specifications

(this annex forms an integral part of this Recommendation)

- Standard Device Classes

This following table  lists the standard device classes.

Device Class TABLE

|  | Device Class Name |
| --- | --- |
|  | Camera |
|  | Microphone |
|  | Stream player recorder |
|  | Slide Projector |
|  | Light Source |
|  | Document scanner |
|  | Source Combiner |

- C.1.1 Device Number

00 -127  available for nodes to allocate locally

Standard Device Control Specifications

Device class identifiers are assigned by this document. Each node may support up to 128 devices from any class, identified by a numeric index (0 to 127).

A number of standard device classes are defined. A library  of  standard  device attributes is defined for use by the device classes. For each standard class a set of attributes  which may be used by that class is listed. Where appropriate, it is permissible to use other standard attributes which are not listed as part of a device class. Where a priori information is available a node may use non-standard attributes as part of a standard device class. A node may also declare a non-standard device class using standard or non-standard attributes.

Each attribute is described individually with a table showing all the available attribute fields. The availability of each field depends upon the service primitive in which the attribute is used.

- C.1.2 Camera Device Class

In a video conferencing environment, cameras are normally the principle source of  video. The camera device class therefore provides a comprehensive remote control capability allowing control over the camera position and viewing angle as well as the ability to manipulate  parameters relating to the image.

Control Attributes

Set Device state

Select Device Pre-set

Configure Date

Configure Time

Set Iris Mode

Set Focus Mode

Select Camera Lens

Select Camera Filter

Go To Camera Home Position

Illuminate External Camera Light

Clear Camera Lens

Set Camera Pan Speed

Set Camera Tilt Speed

Configure Camera Back light Setting

Configure Camera White Balance Alignment

Focus Image

Pan Continuous

Tilt Continuous

Zoom Continuous

Focus Continuous

Iris Continuous

Set Zoom Position

Set Focus Position

Set Iris Position

Set Pan Position

Set Tilt Position

Status Attributes

Read Device State

Read Selected Device Pre-set

Read Iris Mode

Read Focus Mode

Read Selected Camera Lens

Read Selected Camera Filter

Read Selected External Camera Light

Read Camera Pan Speed

Read Camera Tilt Speed

Read Camera Back light Setting

Read Camera White Balance Alignment

Read Zoom Position

Read Focus Position

Read Iris Position

Read Pan Position

Read Tilt Position

Event Attributes

Device Lock State Changed

Device Availability Changed

Camera Panned to Limit

Camera Tilted to Limit

Camera Zoomed to Limit

Camera Focused to Limit

- C.1.3 Slide Presenter Device Class.

The slide presenter device class provides a means for controlling, configuring and collecting the status of  a remote slide presenter. The device class  may also contain camera control attributes where the slide presenter  contains a camera.

Control Attributes

Set Device state

Select slide

Select next slide

Set slide show mode

Play automatic slide show

Set automatic slide display time

Status Attributes

Read Device State

Read automatic slide display time

Event Attributes

Device Lock State Changed

Device Availability Changed

- C.1.4 Microphone Device Class

In a conferencing environment, a microphone is normally the principle source of  audio. The microphone device class provides a means for controlling and collecting the status of a remote microphone.

Control Attributes

Set Device state

Status Attributes

Read Device State

Event Attributes

Device Lock State Changed

Device Availability Changed

- C.1.5 Light Source Device Class.

The light source device class provides a means for controlling and collecting the status of a remote light source that can be considered a device in its own right and is not part of another device. An example of when this device class may be used is when a light source is part of an external security installation. The light source will need to be remotely activated when the natural light fades and removed when the natural light returns.

Control Attributes

Set Device state

Status Attributes

Read Device State

Event Attributes

Device Lock State Changed

Device Availability Changed

- C.1.6 Stream Player Recorder Device Class

The Stream Player Recorder device class provides a means for controlling, configuring and collecting the status of  a remote Record / Playback device. This device may be connected to both a video and audio stream or only a video stream or only an audio stream. The Device Entry in the AVC application roster should be consulted to determine this. This device class may be used to describe such devices as a Video Cassette Recorder or a CD Player.

This device class allows the device to partition the playback media into sections which can be regarded as a complete. Each partition is referred to as a program.  In the case of a VCR a program is  the cassette tape, or for a CD player it may be a track. The stream positioning parameters then occur within the bounds of the selected program. The is a concept of Normal Play Time which is a measurement of the amount of play time when played at normal speed. The Normal Play Time is measured from the start of the current program. This class may additionally provides a means for recording an incoming stream.

Control Attributes

Set Device state

Configure Date

Configure Time

Continuous Rewind

Continuous Fast Forward

Search Backwards

Search Forwards

Pause

Select Program

Next Program

Go To Normal Play Time Point

Configure Continuous Play Back Mode

Play

Set Audio Output State

Play to Normal Play Time Point

Record

Record for Duration

Status Attributes

Read Device State

Read Continuous Play Mode

Read Stream Player Recorder State

Read Current Program Duration

Read Selected Program

Event Attributes

Device Lock State Changed

Device Availability Changed

Stream Player Recorder State Change

Stream Player Recorder Program Change

- C.1.7 Source Combiner Device Class

The source selection device class provides a means for selecting the source output of one or more devices to be combined together to form one output stream.

Control Attributes

Set Device state

Configure Device Inputs

Status Attributes

Read Device State

Read Configured Device Inputs

Event Attributes

Device Lock State Changed

Device Availability Changed

- C.1.8 Document Scanner Device Class

The document scanner device class provides a means for controlling, configuring and collecting the status of  a remote document scanner.

Editor’s note - This section is for further study

- C.1.9  Device Control Attributes

- C.1.9.1 General Control Attributes

These control attributes are general attributes for use by any device class as appropriate.

Set Device State

This attribute is used to set a device to an active or inactive state. If a device class supports this control attribute then the Read Device State status attribute should also be supported to allow a remote node to determine the current state.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Device state | Active / Inactive |  | request indication |

Configure Device Date

This attribute is used configure the current date on a device.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Day | 1..31 |  | request indication |
| Month | 1..12 |  | request indication |
| Year | 1980..2999 |  | request indication |

Configure Device Time

This attribute is used configure the current time on a device.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Hours | 0..23 |  | request indication |
| Minutes | 0..59 |  | request indication |

Select Device Pre-set

This attribute allows the selection of a particular device pre-set where the device has more than one pre-set available. Each pre-set has a text label associated with it which may be used to provide descriptive text about the pre-set to a remote user. Each text is limited to a size of  32 characters.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Maximum number available | 1..255 | response confirm |  |
| List of pre-set text labels | List of up to 255 pre-set labels | response confirm |  |
| Select pre-set | 1..255 |  | request indication |

- C.1.9.2  Camera Control Attributes

The attributes in this group may be used for controlling a camera. Camera control movement parameters may be specified relative to the current position or as an absolute value. The terms left and right used in this section are used to describe movement as viewed from the camera’s perspective.

Set Iris Mode

Configures the camera iris mode to be automatic or manual. If a device class supports this control attribute then the Read Iris Mode status attribute should also be supported to allow a remote node to determine the current setting.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Mode | Manual / Auto |  | request indication |

Set Focus Mode

Configures the camera focusing mode to be automatic, manual or a single shot auto mode. If a device class supports this control attribute then the Read Focus Mode status attribute should also be supported to allow a remote node to determine the current setting.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Mode | Manual / Auto |  | request indication |

Select Camera Lens

This attribute allows the selection of a particular camera lens where the camera has more than one lens available. If a device class supports this control attribute then the Read Selected Camera Lens status attribute should also be supported to allow a remote node to determine the current setting.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Maximum number available | 1..255 | response confirm |  |
| Select lens number | 1..255 |  | request indication |

Select Camera Filter

This attribute allows the selection of a particular camera filter where the camera has more than one filter available. If a device class supports this control attribute then the Read Selected Camera Filter status attribute should also be supported to allow a remote node to determine the current setting.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Maximum number available | 1..255 | response confirm |  |
| Select filter number | 1..255 |  | request indication |

Go To Camera Home Position

This attribute commands the camera to move to its default home position.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |

Illuminate External Camera Light

This attribute is used to control an external light source that is associated with a camera. An light source may be switched on in order to illuminate the camera’s field of view.  Up to 10 lights may be available. The Select field selects which light is illuminated. Only one light source may be illuminated at one time so any  previously illuminated light is switched off. A value of 0 in the select field will turn off all lights. If a device class supports this control attribute then the Read Illuminated External Light status attribute should also be supported to allow a remote node to determine the current setting.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Maximum number of available lights | 1..10 | response confirm |  |
| Select | 1..10 |  | request indication |

Clear Camera Lens

This attribute commands the camera to clear the lens. For example, this attribute may be used by a camera situated in an outside location which has an external wiper attached to clear rain and dirt from the lens.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |

Set Camera Pan Speed

This attribute configures the speed of camera movement when it is panning. If a device class supports this control attribute then the Read Camera Pan Speed status attribute should also be supported to allow a remote node to determine the current setting.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Max. pan speed | 0..1023 | response confirm |  |
| Set pan speed | 0..1023 |  | request indication |

Set Camera Tilt Speed

This attribute configures the speed of camera movement when it is tilting. If a device class supports this control attribute then the Read Camera Tilt Speed status attribute should also be supported to allow a remote node to determine the current setting.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Max. tilt speed | 0..1023 | response confirm |  |
| Set tilt Speed | 0..1023 |  | request indication |

Configure Camera Back Light Setting

This attribute configures the setting of the camera back light. If a device class supports this control attribute then the Read Camera Back Light status attribute should also be supported to allow a remote node to determine the current setting.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Maximum back light setting | 0..255 | response confirm |  |
| Configure back light setting | 0..255 |  | request indication |

Configure Camera White balance Alignment

This attribute configures the setting of the camera white balance alignment. If a device class supports this control attribute then the Read Camera White Balance Alignment status attribute should also be supported to allow a remote node to determine the current setting.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Max. manual white balance setting | 0..255 | response confirm |  |
| Configure white balance setting | 0..255 / Auto |  | request indication |

Focus  Image

This attribute instructs the camera to bring the current image into focus. Once the camera has focused it will maintain that focus position regardless of changes occurring in the current image.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |

- C.1.9.3  Camera Control Attributes Used For Start Stop Movement

These attributes start a continuous movement and have a maximum time out as part of the attribute parameter. The continuous movement is started in the selected direction until the time-out expires. If another control service primitive is received before the time-out expires, the movement will be stopped immediately if the primitive contains the same attribute with a stop parameter. If the primitive contains the same attribute with a new time-out, the timer will be immediately restarted with the new time-out.

Pan Continuous

Starts or stops a continuous horizontal panning movement with a maximum time-out.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Pan direction | Left / Right / Stop |  | request indication |
| Pan time-out | 100..1000ms |  | request indication |

Tilt Continuous

Starts or stops a continuous vertical tilting movement with a maximum time-out.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Tilt direction | Up / Down / Stop |  | request indication |
| Tilt time-out | 100..1000ms |  | request indication |

Zoom Continuous

Starts or stops a continuous zoom movement with a maximum time-out.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Zoom direction | Telescopic / Wide / Stop |  | request indication |
| Zoom time-out | 100..1000ms |  | request indication |

Focus Continuous

Starts or stops a continuous focus movement with a maximum time-out.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Focus direction | Near / Far / Stop |  | request indication |
| Focus time-out | 100..1000ms |  | request indication |

Iris Continuous

Starts or stops a continuous iris movement with a maximum time-out.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Iris direction | Darker / Lighter / Stop |  | request indication |
| Iris time-out | 100..1000ms |  | request indication |

- C.1.9.4  Camera Control Attributes Using Fixed Range Positioning

This group of attributes use a position mechanism with fixed ranges. The receiving AVC application must map this fixed positioning to the actual positioning system used by the camera. For example, the zoom attributes have a fixed co-ordinate range of -127 to +127. Zoom position +127 represents the maximum zoom position. Zoom position -127 is the maximum unzoomed position and 0 represents the mid point. This method is not used for panning and tilting.

A fixed range value maps to the following camera positions depending upon the attribute with which it is used.

| Maximum negative value | 0 | Maximum positive value |
| --- | --- | --- |
| Maximum un-zoomed | Midpoint Zoom position | Maximum Zoom |
| Maximum Focus Near | Midpoint Focus position | Maximum Focus Far |

Set Zoom Position

This attribute sets the camera zoom position. If a device class supports this control attribute then the Read Zoom Position status attribute should also be supported to allow a remote node to determine the current setting.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Zoom to position | -1023..+1023 |  | request indication |
| Positioning mode | Relative / Absolute |  | request indication |

Set Focus Position

This attribute sets the camera focus position. If a device class supports this control attribute then the Read Focus Position status attribute should also be supported to allow a remote node to determine the current setting.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Focus to position | -127..+127 |  | request indication |
| Positioning mode | Relative / Absolute |  | request indication |

Set Iris Position

This attribute sets the camera iris position. If a device class supports this control attribute then the Read Iris Position status attribute should also be supported to allow a remote node to determine the current setting.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Iris position | -127..+127 |  | request indication |
| Positioning mode | Relative / Absolute |  | request indication |

- C.1.9.5  Camera Control Attributes Using Spatial Positioning

This group of attributes use a  positioning mechanism where the positioning system represents the absolute camera position in degrees. The receiving AVC application must map these absolute degree positions to the camera positioning system.

These parameter values express the angle in degrees to which the camera is to be positioned assuming that the mid point of the camera pan and tilt range is 0°.

A absolute position value maps to the following camera positions depending upon the attribute with which it is used.

| Negative value | 0° | Positive value |
| --- | --- | --- |
| Number of degrees to pan to the left of the midpoint. | Midpoint Pan position | Number of degrees to pan to the right of the midpoint. |
| Number of degrees to Tilt downwards from the midpoint. | Midpoint Tilt position | Number of degrees to Tilt upwards from the midpoint |

A range from  -18000 to +18000 is used to represent a position from -180° to +180° in steps of  0.01° (= 0° 0’ 36”). The camera should position itself to the nearest position possible.

Set Pan Position

This attribute is used to pan a camera to a particular position. This attribute sets the camera pan position. If a device class supports this control attribute then the Read Pan Position status attribute should also be supported to allow a remote node to determine the current setting.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Max. pan range Left | 0..+18000 | response confirm |  |
| Max. pan range right | 0..-18000 | response confirm |  |
| Min camera step size | 0..+18000 | response confirm |  |
| Pan to position | -18000..+18000 |  | request indication |
| Positioning mode | Relative / Absolute |  | request indication |

Set Tilt Position

This attribute may be used to set the tilt position. The Position mode field may be set to relative or absolute. In relative mode, the new tilt position is relative to the current camera position. This attribute sets the camera tilt position. If a device class supports this control attribute then the Read Tilt Position status attribute should also be supported to allow a remote node to determine the current setting.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Max. tilt range up | 0..+18000 | response confirm |  |
| Max. tilt range down | 0..-18000 | response confirm |  |
| Min camera step size | 0..+18000 | response confirm |  |
| Tilt to position | -18000..+18000 |  | request indication |
| Positioning mode | Relative / Absolute |  | request indication |

- C.1.9.6 Slide Presenter Control Attributes

Select Slide

This attribute allows the selection of a particular slide for viewing. If a device class supports this control attribute then the Read Selected Slide status attribute should also be supported to allow a remote node to determine the current setting.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Maximum number available | 0..1023 | response confirm |  |
| Select slide number | 0..1023 |  | request indication |

Select Next Slide

This attribute allows the selection of a slide for viewing adjacent to the currently selected slide.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Direction | Forwards / Backwards |  | request indication |

Set Slide Show Mode

Configures the slide presenter mode to be automatic or manual.  If a device class supports this control attribute then the Read Slide Show Mode status attribute should also be supported to allow a remote node to determine the current setting.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Mode | Manual / Auto |  | request indication |

Play Automatic Slide Show

Starts, stops or pauses an automatic slide show

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Mode | Start / Stop / Pause |  | request indication |

Set Automatic Slide Display Time

Configures the time period in seconds for which a slide is displayed when an automatic slide show is in progress

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Maximum slide time available | 1..255 | response confirm |  |
| Slide display time in seconds | 1..255 |  | request indication |

- C.1.9.7 Stream Player Recorder Control Attributes for Stream Positioning

Continuous Rewind

This attribute is used to start or stop a rewinding operation. The output stream is not visible during rewinding.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Mode | Start / Stop |  | request indication |

Continuous Fast Forward

This attribute is used to start or stop a fast forwarding operation. The output stream is not visible during fast forwarding.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Mode | Start / Stop |  | request indication |

Search Backwards

This attribute is used to starts or stops reversing at greater than normal playing speed through the  program whilst still showing the output.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Mode | Start / Stop |  | request indication |

Search Forwards

This attribute is used to starts or stops forwarding at greater than normal playing speed through the  program whilst still showing the output.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Mode | Start / Stop |  | request indication |

Pause

This attribute sets or releases the pause function.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Mode | Start / Stop |  | request indication |

Select Program

This attribute selects the current program.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Maximum number of programs available | 1..1023 | response confirm |  |
| Select | 1..1023 |  | request indication |

Next  Program

This attribute selects an adjacent program to the current selected program.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Select | -1 / +1 |  | request indication |

Go To Normal Play Time Point

This instructs the player to move to this Normal Play Time point within the current program. The Normal Play Time is measured from the start of the program and is expressed in seconds.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Maximum play time point available | 0 .. 65535 | response confirm |  |
| Go to normal play time point | 0 .. 65535 |  | request indication |

- C.1.9.8 Stream Player Recorder Attributes for Configuration

Configure Continuous Play Back Mode

This attribute configures the setting of the continuous playback mode.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Mode | on / off |  | request indication |

- C.1.9.9 Stream Player Recorder Attributes for Playback

Play

This attribute starts or stops stream playback.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Mode | Start / Stop |  | request indication |

Set Audio Output State

This attribute activates or mutes the audio output from the player.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| State | Active / Muted |  | request indication |

Play to Normal Play Time Point

This instructs the player to start playing until the Normal Play Time point within the current program is reached. The Normal Play Time is measured from the start of the program and is expressed in seconds and microseconds.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Maximum play time point available | 0 .. 65535 | response confirm |  |
| Play to normal play time point | 0 .. 65535 |  | request indication |

- C.1.9.10 Stream Player Recorder Control Attributes for Recording

This attributes may be used by a device that is capable of recording an incoming stream.

Record

This attribute starts or stops recording an incoming stream.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Mode | Start / Stop |  | request indication |

Record for Duration

This attribute starts or stops recording an incoming stream for a specified duration which is expressed in seconds.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Max. duration available in seconds | 0..65535 | response confirm |  |
| Duration in seconds | 0..65535 |  | request indication |

- C.1.9.11 Source Combiner Control Attributes

Configure Device Inputs

This attribute configures the list of  devices that will source an input into the source combiner device. The attribute declares the maximum number of inputs available for this source combiner. It also declares a list of available devices which can be used as an input. This list may exceed the actual number of available inputs. When this attribute forms part of an AVC-Device-Control primitive it contains a list of devices which are to act as inputs to this source combiner. This list should not exceed the number of available inputs. If the number of entries in this list  is less than the number of available inputs then some inputs will be unconnected.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Control |
| --- | --- | --- | --- |
| Maximum number of device inputs | 0..64 | response confirm |  |
| Number of entries in available devices list | 0..64 | response confirm |  |
| List of available devices | List of device classes and identifiers | response confirm |  |
| Output format | For further study | response confirm |  |
| Number of entries in device input list | 0..64 |  | request indication |
| Device input list | List of device classes and identifiers |  | request indication |

- C.1.10 Device Status Attributes

- C.1.10.1 General Status Attributes

These status attributes may be used by any device class as appropriate.

Read Device State

This attribute returns the current device state.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Device | Active / Inactive / Unknown |  | response confirm |

Read Device Date

This attribute is used to read the current device date.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Day | 1..31 / Unknown |  | response confirm |
| Month | 1..12 / Unknown |  | response confirm |
| Year | 1980..2999 / Unknown |  | response confirm |

Read Device Time

This attribute is used to read the current device time.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Hour | 0..23 / Unknown |  | response confirm |
| Minutes | 0..59 / Unknown |  | response confirm |

Read Selected Device Pre-set

If the device is currently in a state that is defined by a device pre-set then this attribute will return the number of the pre-set. If the device is in a state that is define by a device pre-set then the value returned will be 0.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Current selected pre-set | 1..255 / Unknown |  | response confirm |

- C.1.10.2 Camera Status Attributes

The following table show the status attributes.

Read Iris Mode

Returns the current camera iris mode.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Mode | Manual / Auto / Unknown |  | response confirm |

Read Focus Mode

Returns the current camera focus mode.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Mode | Manual / Auto / Unknown |  | response confirm |

Read Pan Motor Speed

Returns the current configuration of the camera pan motor speed.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Current pan motor speed | 0..1023 / Unknown |  | response confirm |

Read Tilt Motor Speed

Returns the current configuration of the camera tilt motor speed.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Current tilt speed | 0..1023 / Unknown |  | response confirm |

Read Illuminated External Camera Light

Returns the number of the illuminated external camera light. If no light is currently illuminated  then the Illuminated light field will indicate 0.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Illuminated light | 0..10 / Unknown |  | response confirm |

Read Camera Back light Setting

This attribute returns the setting of the camera back light.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Current back light setting | 0..255 / Unknown |  | response confirm |

Read Camera White balance Alignment

This attribute returns the setting of the camera white balance alignment.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Current white balance alignment | 0..255 / Unknown |  | response confirm |

Read Selected Camera Lens

This attribute returns the number of the last selected camera lens.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Selected lens number | 1..255 / Unknown |  | response confirm |

Read Selected Camera Filter

This attribute returns the number of the last selected camera filter.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Selected filter number | 1..255 / Unknown |  | response confirm |

- C.1.10.3 Camera Status Attributes Using Fixed Range Positioning

Read Zoom Position

This attribute returns the current zoom position.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Zoom position | -1023..+1023 / Unknown |  | response confirm |

Read Focus Position

This attribute returns the current focus position.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Focus position | -127..+127 / Unknown |  | response confirm |

Read Iris Position

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Iris position | -127..+127 / Unknown |  | response confirm |

- C.1.10.4 Camera Status Attributes Using Absolute Positions

A range from  -18000 to +18000 is used to represent a position from -180° to +180° in steps of  0.01°.

Read Pan Position

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Pan position | -18000..+18000 / Unknown |  | response confirm |

Read Tilt Position

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Tilt position | -18000..+18000 / Unknown |  | response confirm |

- C.1.10.5 Slide Presenter Status Attributes

Read Automatic Slide Display Time

This attribute returns the current setting of the slide display time.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Slide display time | 1..255 / Unknown |  | response confirm |

- C.1.10.6 Stream Player Recorder Status Attributes

Read Continuous Play Mode

This attribute returns the current setting of the continuous play mode.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Continuous play mode | On / Off / Unknown |  | response confirm |

Read Stream Player Recorder State

This attribute returns the current state of the stream player recorder.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| State | Playing / Recording / Paused / Paused / record / Paused play / Rewinding / Fast Forwarding / Searching Forwards / Searching Backwards / Stopped / Program Unavailable / Unknown |  | response confirm |

Read Current Program Duration

This attribute returns the total duration of the currently active program in seconds and microseconds

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Seconds | 0..65535 / Unknown |  | response confirm |

Read Current Program Duration

This attribute returns the current normal play time point in seconds and microseconds

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Seconds | 0.. 65535 / Unknown |  | response confirm |

Read Selected Program

If the device is currently playing a selected program then this attribute will return the number of the program. If the device is not currently playing a program pre-set then the value returned will be 0.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Current selected program | 1..1023 / Unknown |  | response confirm |

- C.1.10.7 Source Combiner Status Attributes

The attributes in this group are may be used  with the source combiner device.

Read Configured Device Inputs

This attribute returns the list of  devices that are acting as source inputs into the source combiner device.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Device-Status |
| --- | --- | --- | --- |
| Number of entries in device input list | 0..64 |  | response confirm |
| Device input list | List of device classes and identifiers |  | response confirm |

- C.1.11 Device Event Attributes

These event attributes are general attributes for use by any device class as appropriate.

Device Lock State Changed

This event attribute signals a change in the device lock state.  The new state field indicates the new lock state of the device.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Configure-Device-Events | AVC-Device-Event-Notify |
| --- | --- | --- | --- | --- |
| New state | Locked / Unlocked |  |  | indication |

Device Availability Changed

This event attribute signals a temporary change in the availability of the device. The new state field indicates the new availability of the device. This may be because the device has been made inactive or is no longer communicating with the host node. If remains unavailable for a long period of time or a device is removed permanently, the device entry should be removed from the AVC application roster.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Configure-Device-Events | AVC-Device-Event-Notify |
| --- | --- | --- | --- | --- |
| New state | Unavailable / Available |  |  | indication |

- C.1.11.1 Camera Event Attributes

The event attributes in this group are designed for use with a camera.

Camera Panned to Limit

This event attribute signals that the camera reached the limit of the available pan range. The direction field indicates which pan limit was reached.

| Attribute Fields | Values |  | AVC-Device-Attributes |  | AVC-Configure-Device-Events |  | AVC-Device-Event-Notify |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Direction |  | Left / Right |  |  |  |  |  | indication |

Camera Tilted to Limit

This event attribute signals that the camera reached the limit of the available tilt range. The direction field indicates which tilt limit was reached.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Configure-Device-Events | AVC-Device-Event-Notify |
| --- | --- | --- | --- | --- |
| Direction | Up / Down |  |  | indication |

Camera Zoomed to Limit

This event attribute signals that the camera reached the limit of the available zoom range. The direction field indicates which zoom limit was reached.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Configure-Device-Events | AVC-Device-Event-Notify |
| --- | --- | --- | --- | --- |
| Direction | Telescopic / wide |  |  | indication |

Camera Focused to Limit

This event attribute signals that the camera reached the limit of the available focus range. The direction field indicates which focus limit was reached.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Configure-Device-Events | AVC-Device-Event-Notify |
| --- | --- | --- | --- | --- |
| Direction | Near / far |  |  | indication |

- C.1.11.2 Slide Presenter Event Attributes

The event attributes in this group may be used with a slide presenter.

Automatic Slide Show Finished

This event attribute signals that the automatic slide show has reached the end of the slides available for viewing.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Configure-Device-Events | AVC-Device-Event-Notify |
| --- | --- | --- | --- | --- |

- C.1.11.3 Stream Player Recorder Event Attributes

The event attributes in this group may be used with a stream player recorder.

Stream Player Recorder State Change

This event attribute signals that the player / recorder has changed state. For example a state change will occur when a local user starts playing a VCR. The VCR device will then inform interested parties that the VCR is now playing.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Configure-Device-Events | AVC-Device-Event-Notify |
| --- | --- | --- | --- | --- |
| New State | Playing / Recording / Paused / Paused record / Paused play / Rewinding / Fast Forwarding / Searching Forwards / Searching Backwards / Stopped / Program Unavailable |  |  | indication |

Stream Player Recorder Program Change

This event attribute signals that the player / recorder has changed the selected program. If a program is no longer selected then the New program field will indicate 0.

| Attribute Fields | Values | AVC-Device-Attributes | AVC-Configure-Device-Events | AVC-Device-Event-Notify |
| --- | --- | --- | --- | --- |
| New program | 0..1023 |  |  | indication |

Appendix I

Accessing AVC Services

(this appendix does not form an integral part of this Recommendation)

This section illustrates a typical sequence of operations that occur when accessing an AVC Service.

The following assumptions are made:

- AVC Network Elements have completed AVC Service Arbitration

- AVC Terminals have been notified which AVC Network Element is their Zone Manager

- AVC Terminals have been notified of availability of AVC Services

Figure I illustrates the generic sequence of operations required to access an AVC service.

First an AVC terminal makes an AVC Service request to its Zone Manager.

The (Node Controller at the) Zone Manager is responsible for processing the AVC Service request and determines whether it has the necessary information to decide whether it can provide the service.

If the resources required to deliver the service are exclusively under the control of the Zone Manager, it may then configure those resources using the AVC Resource Configuration commands (nodes and remote links) and NSCE commands (local links) as appropriate.

If the Zone Manager requires information on AVC infrastructure controlled by other AVC Network Elements, it retrieves this from the appropriate owner(s) by using the AVC Roster Enquire mechanism.

If some of the resources required to deliver the service are owned by other Network Elements, the Zone Manager attempts issues AVC-Resource-Reservation requests to each of the Network Elements concerned.

If the resources required to deliver the service are available (i.e. all reservations succeeded), the Zone Manager confirms the reservations, which results in the AVC infrastructure being configured to support the service.

Once all resources have been configured, the Zone Manager indicates to the requesting AVC Terminal that the AVC Service is available for use.

FIGURE I/T.132

Requesting an AVC service

The following example illustrates how the AVC Stream Routing Service operates and the interaction with AVC infrastructure management.

Figures II and III accompany the following text for a simple stream routing example in which a real time user application at Node F wishes to receive a video stream from Node A.  For the purposes of this example, the following assumptions are made:

- Node D is the Zone Manager of Zone 2 and owns the nodes E and F and links EF, DC and DE.  It jointly owns link CE with Node C

- Node C owns nodes B and A and links CB and BA.  It jointly owns the link CE with Node D.

- Node C has a complete copy of the AVC Link and Node Rosters

- Sequence of operations:

- A real time user application at Node A issues an AVC-Create-Multipoint-Channel request to its local AVC Provider via its local Node Controller, specifying the media type (e.g. video), stream source (Node A) and sink (Node F).  The request may optionally include the media format (e.g. H.261) and QoS preferences.  Figure 1 shows the logical interconnection.

- The AVC Provider at Node A forwards the request to the AVC Provider at its Zone Manager (C)

- The Node Controller at Node C consults its copy of the AVC Link Roster to evaluate possible routes between nodes A and F and identifies two routes: R1 (ABCDEF) and R2 (ABCEF).  Using the link capability and configuration information contained in the Link Roster, the Node Controller can determine the bitrate available on each route.

- The Node Controller at Node C examines the Node Roster to assess the video processing capabilities of the endpoints (A, F) and intermediate nodes on each route.  This enables it to determine whether any intermediate processing elements (e.g transcoders) are required.

- The Node Controller at Node C selects which route to use.  The method for making the selection is outside the scope of this recommendation.  Assume that the Node Controller chooses R2.

- The Node Controller at Node C instructs its local NSCE to open a logical channel with the appropriate characteristics from node C to node E.

- The Node Controller at Node C issues AVC-Reserve-Resource requests to Node D requesting resources to route video from node E to node F and providing the identifier assigned to the new multipoint channel.

- The Node Controller at Node D responds to Node C indicating whether resources to support the request are available.

- The Node Controller at Node C issues AVC-Confirm-Resource-Reservation requests to Node D to instruct it to configure the resources.

- The Node Controller at Node D issues an AVC-Open-Logical-Channel request to Node E to open a logical channel from Node E to Node F for the new multipoint channel.

- If this is successful, the Node Controller at Node D issues an AVC-Set-Node-Configuration request to Node E instructing it to bridge the new logical channels on link CE and link EF for the new multipoint channel.

- If this is successful, the Node Controller at Node D issues an AVC-Confirm-Resource-Reservation response to Node E, notifying it that the portion of the new multipoint channel between Node C and Node F has been established.

- The Node Controller at Node C issues an AVC-Open-Logical-Channel request to Node A to open a logical channel from Node A to Node B.

- If this is successful, the Node Controller at Node C issues an AVC-Set-Node-Configuration request to Node B instructing it to bridge the new logical channels on link AB and link BC for the new multipoint channel.

- The Node Controller at Node C issues an AVC-Open-Logical-Channel request to Node B to open a logical channel from Node B to Node C. The Node Controller at Node C configures its local bridging elements

- Once all resources have been configured, the Node Controller at Node C issues an AVC-Open-Multipoint-Channel response to Node F, indicating the level of service available on the  route.

- On receipt of the AVC-Open-Multipoint-Channel confirm from its local Node Controller, the user application at Node A may then start to use the multipoint channel.

FIGURE II/T.132

Stream route request from node A to node F

FIGURE III/T.132

Possible routings to realise request

FIGURE IV/T.132

Possible sequence of primitives to fulfil multipoint channel request (incremental change)

END OF T.132

Note?	-Other services such as  ITU H Series Recommendations .